SA WG2 Meeting #S2-152e(e-meeting)	S2-2206620

17 - 26 August, 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI#9, Sol#24: update to sol#24

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3

Abstract of the contribution: this pCR is proposed to update the sol#24.

1. Discussion

This pCR is proposed to update solution#24 to clarify assistance information provided by NG-RAN and AMF behavior when the assistance information is received.

2. Proposal

This paper proposes to update solution#24 to clarify assistance information provided by NG-RAN and AMF behaviour.

* First change *

6.24	Solution #24: UE Location Verification based on Obtained Information

6.24.1	Introduction

This solution addresses KI#9: Support of Positioning Requirements Related to Satellite Access.

6.24.2	Functional Description

In Rel-17, as described in clause 5.4.11.4 in TS 23.501 [2], if the AMF is not able to determine the UE's location with sufficient accuracy based on the ULI, the AMF proceeds with the Mobility Management or Session Management procedure and may initiate UE location procedure after the Mobility Management or Session Management procedure is complete, as specified in clause 6.10.1 of TS 23.273 [5], to determine the UE location. The AMF shall be prepared to deregister the UE if the information received from LMF indicates that the UE is registered to a PLMN that is not allowed to operate in the current UE location.

In this solution, it is proposed to optimize the mechanism for UE location verification, as follows:

-	If NG-RAN can verify UE location, the NG-RAN provides assistance information including the verification result to AMF.

-	When the assistance information indicates that UE location is reliable, the AMF skips the UE location verification mechanism in Rel-17.

-	When the assistance information indicates that UE location is not reliable, the following scenario may happen:

-	If the country of the UE reported location is the same as the country of the network verified UE location, the AMF triggers LCS procedure to obtain UE location and notify the UE location to NFs which have subscribed the related event.

-	If the country of the UE reported location is different from the country of the network verified UE location, the AMF initiates deregistration procedure with a suitable cause value.

NOTE:	Whether and what assistance information can be provided by NG-RAN depends on the conclusion of the network verified UE location objective in Rel-18 NR_NTN_enh WID in RAN WG.

-	Otherwise, the existing UE location verification mechanism in Rel-17 is re-used.

6.24.3	Procedures

6.24.3.1	UE Provided Location Verification based on Obtained Information

Figure 6.24.3.1-1: UE Provided Location Verification based on Obtained Information

1.	The UE sends UE location information (i.e. GNSS info) to NG-RAN over NR-Uu via satellite access.

2.	[Conditional] If the NG-RAN can verify UE location, it verifies the reported UE location.

NOTE:	Whether NG-RAN can verify UE location and how to verify the UE location depend on the conclusion of the network verified UE location objective in Rel-18 NR_NTN_enh WID in RAN WG.

3.	[Conditional] If the step 2 is performed, the NG-RAN sends N2 message including the assistance information.

4.	[Conditional] If assistance information provided by NG-RAN indicates that UE location is not-reliable, and the country of the UE reported location is the same as the country of the network verified UE location, the AMF triggers the 5GC-NI-LR procedure in clause 6.10.1 of TS 23.273 [5] to obtain UE location.

5.	[Conditional] If there is an NF has subscribed UE location event report, the AMF notifies the UE location to the NF.

6.	[Conditional] If assistance information provided by NG-RAN indicates that UE location is not-reliable and the country of the UE reported location is different from the country of the network verified UE location, the AMF deregisters the UE.

6.24.4	Impacts on services, entities, and interfaces

AMF: receive assistance information from NG-RAN and behaves accordingly.

NG-RAN: verifies the UE location reported by the UE and provides the assistance information to AMF.

NOTE:	Whether and what assistance information can be provided by NG-RAN depends on the conclusion of the network verified UE location objective in Rel-18 NR_NTN_enh WID in RAN WG.

* End of changes *