SA WG2 Meeting #152e	S2-2206743

# 17 - 26 August 2022, Electronic meeting         		            (revision of S2-220xxxx)

Source: 	Lenovo, Broadcom

Title: 	Conclusions about the types of solutions for KI#2

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution:

# Discussion & Proposal

This paper considers the two types of solutions proposed for KI#2 and recommends considering both of them in the normative phase. However, it does not recommend specific solutions for the normative phase as this will be handle in different papers.

Start of Changes

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	Conclusions for Key Issue #2

## 8.x.1	Types of Solutions

For the Key Issue #2, two types of solutions are proposed:

1) "Type-1" solutions, which enhance the TNGF/N3IWF selection procedure so that the initially selected TNGF/N3IWF is based on the Requested NSSAI; and

2) "Type-2" solutions, which enable the network to reselect a different TNGF/N3IWF, when the initially selected TNGF/N3IWF is not appropriate to serve the slices allowed for the UE.

The following table shows the list of the proposed solutions and their types.

Table 8.x.1-1: Solutions for Key Issue #2 and their types

| Solution | Type-1 | Type-2 |
| --- | --- | --- |
| Solution 10: Registration via Trusted Non-3GPP Access with TNGF Relocation |  | X |
| Solution 11: Registration via Untrusted Non-3GPP Access with N3IWF Relocation |  | X |
| Solution 12: slice related TNGF selection for WLAN access | X |  |
| Solution 13: UE getting the slices supported by N3IWF(s) | X |  |
| Solution 14: Use of AN NRF | X |  |
| Solution 15: Selecting N3IWF supporting the S-NSSAI needed by UE |  | X |
| Solution 16: N3IWF selection taking supported slices into account | X |  |
| Solution 17: Untrusted Non-3GPP Access with N3IWF/TNGF Relocation due to network slicing |  | X |
| Solution 18: ANDSP based solution for slice related TNGF selection | X |  |

If only a "Type-1" solution is defined in the normative specs, it shall not be possible for the network to relocate the initially selected TNGF/N3IWF. Hence, it shall not be possible for the network to reselect a different TNGF/N3IWF e.g., based on the Allowed NSSAI and to handle situations where the Allowed NSSAI for the UE is different from the Requested NSSAI.

If only a "Type-2" solution is defined in the normative specs, then the initially selected TNGF/N3IWF (which would be selected without considering slice information) would need to be relocated to a different TNGF/N3IWF in many situations. This will lead to a lot of signalling overhead and registration delays.

Based on the above, it is concluded that both types of solutions are necessary for efficiently selecting a TNGF/N3IWF to serve a UE over non-3GPP access. So, the TNGF/N3IWF selection may be composed of two parts:

1) 	An initial TNGF/N3IWF is selected based on a "Type-1" solution; and

2) 	If necessary, the initial TNGF/N3IWF may be relocated by the network to a different TNGF/N3IWF based on a "Type-2" solution.

End of Changes