3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206746

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2, Sol #5: Update to clarify the functionality of “any AMF”

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS Ph3 / Rel-18

Abstract: it is unclear the RAN node shown in the figure connected with “any AMF” is for what purpose, and the AMF selection logic.

1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

6.5	Solution #5: LCS architecture with "any AMF"

6.5.1	Introduction

This solution is applicable to LCS procedure including MO-LR, MT-LR, and LDR. When network based positioning method is selected by LMF, the signalling between LMF and NG-RAN, LMF and GMLC, are not transmitted to the AMF in the public network.

6.5.2	Functional Description

Architecture for this solution is illustrated as below.

Figure 6.5.2-1

When UE registers to the network, NG-RAN 1 is the serving RAN node for the UE. NG-RAN 1 shall select an AMF in the public network. It is assumed, by means of configuration, that “any AMF” cannot be selected as the serving AMF of the UE.

Multiple RAN nodes are deployed in the campus network. At a given time, only a certain RAN (e.g. RAN 1 in the figure) is the serving RAN of the UE. Other RAN nodes may be selected by the LMF to measure the UE positioning signal.

It is assumed all RAN nodes in the campus network have TNL association and NG-AP connection with the “any AMF”

Current LCS design assumes NG-RAN report UE measurement to AMF (NRPPa message) and then to LMF. Because the NG-AP message may not be cyphered, it is likely the information will be sniffed by other entity in the transport network, which leads to exposure concern.

It is proposed to update the NRPPa signalling as follows:

During the location request procedure, if network based positioning method is selected by LMF, for UE associated NRPPa signalling, it is transmitted via the serving AMF. For non UE associated NRPPa signalling, it is transmitted using "any AMF" deployed in the campus network.

The description of using "any AMF" is specified in TS 38.305 [6]:

An LMF can interact with any gNB reachable from any of the AMFs with signalling access to the LMF in order to obtain location related information to support the NR RAT-Dependent positioning methods. The information can include timing information for the TRP in relation to either absolute GNSS time or timing of other TRPs and information about the supported cells and TRPs including PRS schedule.

Signalling access between the LMF and gNB may be via any AMF with signalling access to both the LMF and gNB.

"any AMF" is an AMF defined in TS 23.501, and needs to support the following functionality in this solution.

-	handling of Namf_nonUEN2message transfer service from LMF;

-	NG-AP message capsulation and decapsulation for UL and DL non UE associated NRPPa message.

Other functionalities supported by the AMF may not be needed in "any AMF".

NOTE:	Based on configuration by the campus network, the NG-RAN(s) are configured with the "any AMF" address, and establish NG-AP association with the any AMF.

Topology of NG-RAN, "any AMF" and LMF are configured by the operator. LMF is configured with "any AMF" address. NG-RAN nodes in the campus network establishes TNL association with "any AMF" in the campus network.

6.5.3	Procedures

6.5.3.1	Network Assisted Positioning Procedure

Same as clause 6.11.2 of TS 23.273 [5].

The AMF illustrated in this call flow is the serving AMF of the UE.

Figure 6.5.3.1-1

6.5.3.2	Obtaining Non-UE Associated Network Assistance Data

Same as clause 6.11.3 of TS 23.273 [5].

The AMF illustrated in this call flow is "any AMF" deployed in the campus network. LMF is configured with "any AMF" address. NG-RAN nodes in the campus network establishes TNL association with "any AMF" in the campus network.

Figure 6.5.3.2-1

6.5.4	Impacts on services, entities, and interfaces

LMF:

-	configured with "any AMF" address.

-	if network based positioning method is selected by LMF, for UE associated NRPPa signalling, it is transmitted via the serving AMF. For non UE associated NRPPa signalling, it is transmitted using "any AMF" deployed in the campus network.

NG-RAN:

-	establishes TNL association with "any AMF".

# * * * * End of changes * * * *