3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206751

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue#2：evaluation

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: Key issue#2 “enhanced positioning architecture for NPN deployment” has two solutions in the TR and both are (mostly) stable. It is time to evaluate them for upcoming conclusion discussion.

1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

7.x	Key Issue #2: enhanced positioning architecture for NPN deployment

The clause evaluates the solutions for KI#2 as following.

Solution#4 and #5 addresses key issue#2: enhanced positioning architecture for NPN deployment. Both solutions propose the following principles:

- AMF (in the public network) for general service operation, local AMF/any AMF for LCS operation

- UE location result is transmitted from LMF to GMLC, not via AMF (in the public network)

- Non-UE NRPPa messages are transmitted between LMF and local AMF/any AMF

Open issue of this key issue is the method for UE associated NRPPa message transmission. Two options are for discussion:

- Solution#4: via local AMF, impacts are on AMF, LMF, and RAN

- Solution#5: via AMF in the public network, no new impact.

# * * * * End of changes * * * *