SA WG2 Meeting #S2-152E	S2-2206909

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	evaluation for key issue#5

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This paper proposes the evaluation for key issue #5.

*** Start of changes ***

7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

7.3 Evaluation for KI#5 Support of multi-path transmission for UE-to-Network Relay

Layer-2 UE-to-Network Relay

- The PCF based service authorization and provisioning as defined in TS 23.304 [3] are used as baseline. (Solution#25)

- NG-RAN controls the multi-path connection establishment and the specific transmission such as splitting or duplication.(solutions#26)

- Similar to Multi-Radio Dual Connectivity (MR-DC), to support multi-path transmission for a CM-CONNECTED Layer-2 Remote UE that has an AS connection via direct network communication path and indirect network communication path(solution#28)

- two redundant PDU Sessions are established by the interaction between UE and NG-RAN/5GC to transfer the data for the ProSe Services with high reliable requirements. (Solution#39)

- 5GC will provide some guidance to NG-RAN for multi-path transmission, e.g., instruct NG-RAN that which QoS flow should be duplicated or split (solution#40)

Layer-3 UE-to-Network Relay

- The PCF based service authorization and provisioning as defined in TS 23.304 [3] are used as baseline. (Solution#25)

- (without N3IWF) 5G ProSe Remote UE determines to use the multi-path transmission based on the triggers (application requirement, ProSe policy, UE implementation, etc.), the 5G ProSe Remote UE has same understanding on the PDU session parameters usage on the direct network path and indirect network path. (solution#27)

- (without W3IWF) Application layer decide when to setup the multi-path connectivity and how to aggregate/split the traffic on both paths. (with W3IWF) ATSSS feature can be used to support multi-path transmission for Layer-3 Remote UE connected to network using one 3GPP access network (i.e. direct path) and one non-3GPP access network. (soutions#28)

- Network can provision the authorization and policy parameters for multi-path transmission service to UE as existing mechanism specified in TS 23.304 [3]. E.g., Remote UE triggers to initiate additional connection to the data network via 3GPP access while Remote UE connecting to the same data network via L3 Relay UE. Vise versa.(solution#29)

- two redundant User Plane paths are established go through two different NG-RANs or same NG-RAN, depending on the scenarios.(solutions#41)

*** End of changes ***