3GPP TSG-SA WG2#152E e-meeting	S2-2206923

Elbonia, 17 - 26 August 2022	(was S2-220xxxx)

Source:	Samsung

Title:	KI#3, Solution #20 Update: URSP delivery for the UE in EPS

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This contribution proposes to update Sol#20 for KI#3.

- Introduction

To address the ENs:

Editor's note:	It is FFS, how the SMF+PGW-C can know the UE-PCF ID.

The above EN can be resolved by reusing the existing PCF discover and selection defined in in clause 6.3.7.1 of TS 23.501, for UE policy delivery procedure initiated by UE in EPS.

Editor's note:	It is FFS, whether and how to handle the UE Policy Association established in 5GS when the UE moves to EPS. When the established PDU Session is moved to EPS, and if the same UE-PCF needs to be selected (e.g. when the UE-PCF was notified about UE policy information delivery failure in 5GS described in TS 23.503 [4]), whether not to terminate the UE Policy Association and whether/how to receive PCF Selection Assistance Info and UE-PCF ID from the UDM (as described in clause 6.3.7 of TS 23.501 [2]) can be further studied.

The above EN can be resolved by introducing procedure of relocation of an UE Policy Association established in 5GS to EPS.

- Proposal

It is proposed to capture the following changes into TR 23.700-85.

# * * * * First change * * * *

## 6.20	Solution #20: URSP delivery for the UE in EPS

### 6.20.1	Description

According to clause 5.2.1 of TS 23.503 [4], the SM-PCF does not support N15 reference point while the UE-PCF does not support N7 reference point. In this solution, for URSP delivery from UE-PCF in 5GC to the UE in EPS, the UE Policy Association establishment between SM-PCF and UE-PCF is proposed. The URSP can be transparently delivered to the UE via SM-PCF, SMF+PGW-C, SGW, and MME. The architecture is shown in Figure 6.20.1-1.

Figure 6.20.1-1: Non-roaming architecture for URSP delivery to the UE in EPS

As depicted in Figure 6.20.1-1, an interface (Nx) needs to be specified between SM-PCF and UE-PCF for establish an UE Policy Association. N15 interface can be reused as Nx.

.

When the UE sends Attach Request in EPS (e.g. Initial Attach procedure, 5GS to EPS Mobility procedure without N26), the UE sends UE Policy Container including list of PSIs and URSP delivery over EPS Indication in PCO to UE-PCF. The detail procedure is introduced in clause 6.20.2.1.

When the UE policy has been updated in PCF while UE is in EPS, the UE-PCF sends UE Policy Container to SMF+PGW-C. The SMF+PGW-C sends PCO including UE Policy Container to the UE. The detail procedure is introduced in clause 6.20.2.2.

When the UE sends Registration Request in 5GS, UE Policy Container including list of PSIs and URSP delivery over EPS Indication to UE-PCF. The detail procedure is introduced in clause 6.20.2.3.

In 5GS to EPS Handover/Idle mode mobility using N26, the UE Policy Association established in 5GS can be maintained in EPS. The detail procedures are introduced in clause 6.20.2.4, 6.20.2.5, 6.20.2.6.

### 6.20.2	Procedures

#### 6.20.2.1	UE Policy delivery procedure initiated by the UE in EPS

Figure 6.20.2-1: UE Policy delivery procedure initiated by the UE in EPS

1.	UE sends UE Policy Container including the list of PSIs and URSP delivery over EPS Indication in PCO to SMF+PGW-C. The PCO can be included in Attach Request and Create Session Request as described in Figure 5.3.2.1-1 of TS 23.401 [8].

2.	The SMF+PGW-C invokes Npcf_SMPolicyControl_Update Request to the SM-PCF to forward the UE Policy Container to the SM-PCF. The SMF+PGW-C provides SUPI, UE Policy Container, , and UE-PCF ID to the SM-PCF. The SM-PCF can notice that the UE-PCF Association Establishment with UE-PCF is allowed in this network by the presence of UE Policy Container in PCO and UE-PCF ID.

Note: The SMF+PGW-C discovers and selects the UE-PCF as described in clause 6.3.7.1 of TS 23.501 [2].

3.	The SM-PCF establishes the UE-PCF Association with the UE-PCF which is identified by UE-PCF ID received from the SMF+PGW-C, and forwards the UE Policy Container to the UE-PCF.

4.	The UE-PCF sends to SM-PCF the UE-PCF Association Establishment Response with result of UE-PCF Association establishment result.

5.	The UE-PCF gets policy subscription related information and the latest list of PSIs from the UDR using Nudr_DM_Query service operation. The UE-PCF creates the UE policy container including UE policy information as defined in clause 6.6 of TS 23.503 [4] (step 6 of Figure 4.16.11-1 of TS 23.502 [3] can be reused).

6.	When the UE-PCF decides to provide the UE policy to the UE, the UE-PCF sends to SM-PCF the UE Policy Container.

7.	The SM-PCF forwards the UE Policy Container to the SMF+PGW-C by Npcf_SMPolicyControl_Update Response.

8.	The Initial Attach procedure is continued as described in Figure 5.3.2.1-1 of TS 23.401 [8]. The SMF+PGW-C sends the PCO which is including the UE Policy Container to the UE. The PCO can be included in Attach Accept. The UE sends the acknowledgement on successful reception of the UE Policy to the SMF+PGW-C, this acknowledgement can be included in the Attach Complete and Modify Bearer Request.

9.	The SMF+PGW-C forwards the UE's acknowledgement by invoking Npcf_SMPolicyControl_Update Request to the SM-PCF.

10.	The SM-PCF forwards the UE's acknowledgement to the UE-PCF.

#### 6.20.2.2	UE Policy delivery procedure initiated by the PCF

Figure 6.20.2-2: UE Policy delivery procedure initiated by the PCF

1.	The UE-PCF is notified from the UDR that the URSP is updated.

2.	The UE-PCF sends to SM-PCF the UE Policy Container which includes the updated URSP.

3.	The SM-PCF forwards the UE Policy Container to the SMF+PGW-C by Npcf_SMPolicyControl_UpdateNotify Request.

4.	The SMF+PGW-C sends the PCO which is including the UE Policy Container to the UE. The step 2 to 5 described in Figure 5.4.3-1 of TS 23.401 [8] is used. The PCO can be included in Update Bearer Request, Downlink NAS Transport, and Direct Transfer. The UE sends the acknowledgement on successful reception of the UE Policy to the SMF+PGW-C, this acknowledgement can be included in Direct Transfer, Uplink NAS Transport, and Update Bearer Response.

5.	The SMF+PGW-C forwards the UE's acknowledgement by invoking Npcf_SMPolicyControl_Update Request to the SM-PCF.

6.	The SM-PCF forwards the UE's acknowledgement to the UE-PCF.

#### 6.20.2.3	URSP delivery over EPS Indication in 5GS

During Registration procedure in 5GS, the UE sends UE Policy Container including the list of PSIs and URSP delivery over EPS Indication to AMF in Registration request as described in clause 4.2.2 of TS 23.502 [3].

AMF transparently forwards UE Policy Container to UE-PCF during UE Policy Association Establishment as described in clause 4.16.11. UE-PCF stores URSP delivery over EPS Indication. The UE-PCF determines whether the UE Policy Association established in 5GS can be maintained in EPS based on this indication as described in clause 6.20.2.4 and 6.20.2.5.

#### 6.20.2.4	5GS to EPS Handover using N26 interface

Figure 6.20.2-3: 5GS to EPS handover for single-registration mode with N26 interface (Figure 4.11.1.2.1-1 in TS 23.502)

Step 18 includes the deregistration of the old AMF for 3GPP access from the HSS+UDM as specified in clause 4.11.1.5.3 in TS 23.502 [3]. The registered AMF for 3GPP access initiates UE Policy Association Termination procedure as defined in clause 4.16.13.1 in TS 23.502 [3]. The UE-PCF determines whether to terminate the UE Policy Association based on URSP delivery over EPS Indication received from UE (as described in clause 6.20.2.1 and 6.20.2.3). The UE-PCF rejects the Npcf_UEPolicyControl_Delete request from the AMF and informs to the AMF that the UE Policy Association can be maintained in EPS. In this case, the AMF initiates the procedure to move the UE Policy Association established between the AMF and UE-PCF to between SM-PCF and UE-PCF as described in clause 6.20.2.6.

#### 6.20.2.5	5GS to EPS Idle mode mobility using N26 interface

Figure 6.20.2-4: 5GS to EPS Idle mode mobility using N26 interface (Figure 4.11.1.3.2-1 in TS 23.502)

In step 15, The UE-PCF determines whether to terminate the UE Policy Association based on URSP delivery over EPS Indication received from UE (as described in clause 6.20.2.1 and 6.20.2.3). The UE-PCF rejects the Npcf_UEPolicyControl_Delete request from the AMF and informs to the AMF that the UE Policy Association can be maintained in EPS. In this case, the AMF initiates the procedure to move the UE Policy Association established between the AMF and UE-PCF to between SM-PCF and UE-PCF as described in clause 6.20.2.6.

#### 6.20.2.6	Relocate an established UE Policy Association to SM-PCF

Figure 6.20.2-5: Relocation of an established UE Policy Association to SM-PCF

- As described in clause 6.20.2.4 and 6.20.2.5, the AMF requests to UE-PCF to terminate the UE Policy Association, when the UE moves from 5GS to EPS.

- The UE-PCF determines whether to terminate the UE Policy Association based on URSP delivery over EPS Indication received from UE (as described in clause 6.20.2.1 and 6.20.2.3). The UE-PCF rejects the Npcf_UEPolicyControl_Delete request from the AMF and informs to the AMF that the UE Policy Association can be maintained in EPS.

- When the AMF is informed that the UE Policy Association can be reused in EPS, the AMF determines which Policy Control Request Triggers can be supported in SMF (e.g. Location change, PLMN change) among the PCRTs of the UE Policy Association.

- The AMF sends UE-PCF ID, UE Policy Association ID, Policy Control Request Triggers for UE Policy Association (determined in step 3), and Information on the UE policy related Policy Control Request Trigger condition that can be met in Nsmf_PDUSession_UpdateSMContext Request to SMF+PGW-C.

- The SMF+PGW-C transfers UE Policy Association information received from the AMF in step 4 to SM-PCF by Npcf_SMPolicyControl_Update Request.

- The SM-PCF notices the UE Policy Association established in 5GS is moved from the AMF to the SM-PCF when SM-PCF receives UE-PCF ID and UE Policy Association ID from SMF+PGW-C. The SM-PCF sends UE Policy Association ID and Information on the UE policy related Policy Control Request Trigger condition that can be met in Npcf_UEPolicyControl_Update Request to UE-PCF. The UE-PCF updates the stored information provided by the AMF with the information provided by the SM-PCF.

- The UE-PCF updates the policy decision based on the information provided by SM-PCF. Step 4 of clause 4.16.2.1.2 in TS 23.502 [3] can be reused. The UE-PCF updates Policy Control Request Trigger parameters for UE Policy Association to be supported in SMF+PGW-C (e.g. remove PCRTs that cannot be supported in SMF+PGW-C).

- The UE-PCF sends UE Policy Container, Policy Control Request Triggers of UE Policy Association in Npcf_UEPolicyControl_Update Response to SM-PCF. SM-PCF stores the updated Policy Control Request Triggers of UE Policy Association. The UE Policy Container is delivered to the UE (as described in clause 6.20.2) after TAU procedure.

- The SMF+PGW-C updates the PCRTs of UE Policy Association received from the AMF (in step 4) with the PCRTs of UE Policy Association received from the SM-PCF (in step 8). After this step, when a PCRT of UE Policy Association condition is met, the SMF+PGW-C sends Information on the UE policy related Policy Control Request Trigger condition that can be met to the SM-PCF and the SM-PCF reports it to the UE-PCF.

- The SMF+PGW-C sends Nsmf_PDUSession_UpdateSMContext Response to AMF. The AMF notices the UE Policy Association is successfully moved to SM-PCF.

### 6.20.3	Impacts on services, entities and interfaces

SMF+PGW-C:

-	Shall be able to include the UE Policy Container received from the PCF into the PCO, and forward to the UE via SGW and MME transparently.

-	Shall be able to forward the UE Policy Container from the UE to the UE-PCF.

-	Shall be able to provide the UE-PCF ID to SM-PCF.

-	Shall be able to forward the acknowledgement of UE reception of the UE Policy to the PCF.

-	Shall be able to provide Information on the UE policy related Policy Control Request Trigger condition that can be met to SM-PCF when the condition is met.

SM-PCF:

-	Shall be able to handle UE Policy Association with UE-PCF.

-	Shall be able to provide to SMF+PGW-C, the UE Policy Container which includes the UE Policy for the UE in EPS.

-	Shall be able to relocate the UE Policy Association established in 5GS from AMF to SM-PCF.

UE-PCF:

-	Shall be able to handle UE Policy Association with SM-PCF.

-	Shall be able to handle UE Policy Association for the UE in EPS based on the URSP delivery over EPS Indication provided by the UE.

AMF:

-	Shall be able to relocate the UE Policy Association established in 5GS from AMF to SM-PCF.

MME:

-	No impacts.

UE:

-	Shall be able to store the URSP received over EPS entities.

-	Shall be able to send the acknowledgement of UE reception of the UE Policy to the PCF over EPS entities.

-	Shall be able to send the list of PSIs stored in the UE and URSP delivery over EPS Indication to the PCF by encapsulating in UE Policy Container, and by including the UE Policy Container into PCO over EPS entities.

# * * * * End of changes * * * *