SA WG2 Meeting #152e	S2-2207155

# August 17th – August 26th ; Elbonia               								    (revision of S2-2206829r06)

Source: 	Intel, vivo

Title: 	KI#5 Interim Conclusion

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution:

# 1 Discussion

This pCR provides the interim conclusion proposal for key issue #5. Based on the evaluation of solutions provided in S2-2206828, it is proposed to agree the interim conclusion for KI#5 as provided below.

# 2 Proposal

It is proposed to update TR 23.700-81 as follows.

Start of change

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X   KI#5: Enhance trained ML model sharing

Interim Conclusion:

It is recommended to use the following principles and procedures as the basis for the normative work.

-	The following are the trained ML model sharing parameters to enable ML model sharing between different vendors - Interoperability indicator as specified in Sol#15.

-  The Interoperability indicator indicates a list of AnLF providers (vendors) that are allowed to retrieve ML models from the MTLF;

-	During registration of NWDAF containing MTLF with NRF, Interoperability indicator to trained ML model sharing are provided as part of NF profile to the NRF.

-	During NWDAF containing MTLF discovery procedure, the consumer (NWDAF containing AnLF) may include Interoperability indicator per Analytics ID(s) in the discovery request.

NOTE: When ML model containing MTLF and NWDAF containing AnLF belong to different vendors, how the ML model privacy is supported is in scope of SA3.

Editor's note:	Whether others parameters (e.g. Interoperability token, ML model file format and/or AI Framework information) to enable ML model sharing between different vendors is FFS.

End change