SA WG2 Meeting #S2-152E	S2-2207462

17 – 26 August , 2022, E-meeting                            rev of S2-2206906r16

Source:	Xiaomi

Title:	conclusion for Key Issue#3

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes the conclusion for key Issue #3.  This paper merged S2-2206015, S2-2206483, S2-2206906 and S2-2205743

*** Start of changes ***

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.3	Conclusion on Key Issue #3

The following interim principles are agreed for Key Issue #3 "Management of PIN and PIN Elements":

1)	PEMC is responsible for PIN Element management (e.g., add/remove the PINE/PEGCs, etc.) and PIN management (e.g. create/modify/delete/activate/deactivate a PIN, etc.).

Editor’s note: If there is other possible  PIN related information needed will be decided on the final conclusion..

Editor’s note: whether UE’s subscription data should contain authorization information, and how this information is used in the 5GC is FFS .

9)	When communicating between a PEMC and a PINE behind a PEGC, via 5GC and the communication requires the PEGC, or when communication between PINEs requires multiple PEGCs and 5GC , the existing functionalities in 5GS for routing the traffic of the communications via UPF(s) can be applied if available.

Editor’s note: whether AF information needs to be stored in UDR for the case, e.g., when UE change PEMC UE, or user’s PEMC UE is recovered, etc., is FFS.

Editor’s note: whether AF or 5GC NF will be involved in PIN/PINE management depends on the conclusion of KI#1 (whether the architecture of PIN contains the PIN AF or existing 5GC NF)

*** End of changes ***