3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206883r03

Elbonia, 17-26 August 2022

Title:	[Draft] LS on N6 PDU Set identification/marking and Traffic classification.

Response to:

Release:	Release 18

Work Item:	FS_XRM

Source:	SA2

To:	SA4

Cc:

Contact Person:

Name:	Dario S. Tonesi

Tel. Number:

E-mail Address:	dtonesi AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

SA2 have further discussed the different examples of PDU Set indicated by SA4 in their LS S2-2203658/S4-220505, and would like to ask SA4 to kindly provide further feedbacks on the following questions:

Q1: Packet ratio for FEC

In S2-2203658/S4-220505, the following examples were included:

In yet another example, a PDU Set may be mapped to all source and repair packets of an Application Layer FEC source block.

….

Typically, for an applicational layer, source block packets from 0 to K-1 identify the source symbols of a source block in sequential order, where K is the number of source symbols in the source block.  Encoding Symbol IDs K onwards identify repair symbols generated from the source symbols using an FEC encoder, e,g, Raptor. Typically, N >= K packets are sent, carrying an FEC source or repair symbols. Typically, the decoder requires only any K or only a small amount more than K packet of the N packets to recover the source packets.

SA2 discussed some candidate solutions proposing packet transmission based on the ratio of source symbol packets, i.e. K/N in above example. SA2 would like to ask SA4 whether the above ratio is static for a specific XRM service, and whether application layer can provide such a ratio to 5GS.

Q2: UPF identification on “number of PDU in the PDU Set”

SA2 discussed some candidate solutions that request UPF to identify “number of PDU in the PDU Set” from the downlink traffic. SA2 would like to ask SA4 whether it’s feasible for application/media layer to obtain such information before the first packet of the PDU Set is sent out, and provide the information to UPF together with the first packet of the PDU Set.

Q3: SA2 would like to ask SA4 whether the following scenario exists for some XR service flow: The non-I frames (e.g. P frame or B frame) transmitted/decoded between two successive I frames directly or indirectly refers to the 1st I frame of the two successive I frames?

Q4: SA2 would like to ask SA4 whether RTP/SRTP could be transferred over TLS/DTLS?

.

In yet another example, a PDU Set may be mapped to all source and repair packets of an Application Layer FEC source block.

….

Typically, for an applicational layer, source block packets from 0 to K-1 identify the source symbols of a source block in sequential order, where K is the number of source symbols in the source block.  Encoding Symbol IDs K onwards identify repair symbols generated from the source symbols using an FEC encoder, e,g, Raptor. Typically, N >= K packets are sent, carrying an FEC source or repair symbols. Typically, the decoder requires only any K or only a small amount more than K packet of the N packets to recover the source packets.

2. Actions:

To SA4:

ACTION: 	SA2 kindly ask SA4 to take the above requests into account and provide feedback as deemed necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #153E	10-14 October 2022	Electronic meeting

TSG-SA2 Meeting #154	14-20 November 2022	Canada