SA WG2 Meeting #152e				   S2-2206734r01

# 17 - 26 August, 2022, Electronic meeting		       (revision of S2-220xxxx)

Source: 	Charter Communications, Lenovo

Title:	Conclusion for KI#5

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution: This paper provides a conclusion for KI#5

# Introduction/Discussion

Key issue #5 aims to study how the data traffic of an MA PDU Session can be switched between one non-3GPP access path and another non-3GPP access path in the same PLMN.

Solutions 5.2 through 5.6 all provide procedures of switching traffic of a MA PDU Session from untrusted non-3GPP access path to trusted non-3GPP access path, while Solution 5.1 provide procedures of switching traffic of a MA PDU Session from trusted non-3GPP access path to untrusted non-3GPP access path.

For the most part, the current set of solutions are aligned in the procedures required to switch between any two (same or different types) non-3GPP access paths.  This contribution attempts to identify the differences proposed in the current set of solutions, in order to aid in evaluating the benefits in the differences.

According to the evaluation paper S2-2206729, solutions #5.5 and solution #5.6 are very similar and incorporates aspects from solutions #5.2 through #5.4. As a result, this paper recommends using solution #5.5 as the base since it has less editor’s notes to address.

# 2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-53 v0.3.0.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause enlists the conclusions of the study, including agreements for work to be done in the normative phase.

## 8.x	Conclusion for KI #5: Switching traffic of an MA PDU Session between two non-3GPP access paths

The solution to be specified in the normative phase shall support the following principles:

1)	The path switching between non-3GPP accesses is performed during the Registration procedure. The UE sends a Registration Request via the target non-3GPP access network and requests user-plane establishment using the "PDU Sessions to be activated" parameter.

Editor’s note: It is FFS whether a new registration type (e.g., Registration type = non-3GPP path switching) or a new indication shall be used.

2)	The AMF updates the registration for non-3GPP access in UDM after the data traffic is switched to the new access.

3) 	Path switching between any two non-3GPP accesses shall be supported.

4) The network capability indication of supporting non-3GPP path switching is sent to the UE, so that the UE can request path switching only if it is supported by the network.

  - Editor’s note:  It is FFS whether this capability exchange is performed at 5G Mobility Management (5GMM) signaling or 5G Session Management (5GSM) signaling (i.e., Registration procedure or MA PDU Session establishment procedure).

# * * * * End of changes * * * *