3GPP TSG-SA WG2 Meeting #152-e	S2-2206762

# E-Meeting, 17th – 26th Aug 2022

Title:	[draft]LS Out on NSSRG restriction on pending NSSAI

Response to:	S2-2205406/ C1-224073

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-xxxxxx, S2-xxxxxx

1. Overall Description:

SA2 thanks for CT1 LS on the NSSRG restriction on pending NSSAI. SA2 answers the question as follows:

Question: Should the UE take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI?

SA2 Answer: No. The NSSRG information is provided together with the Configured NSSAI. The UE only includes in the Requested NSSAI S-NSSAIs that share a common NSSRG. The UE shall not include in the Requested NSSAI any of the S-NSSAIs from the Pending NSSAI the UE stores (as the role of sending Pending NSSAI is to not include its S-NSSAIs in the Requested NSSAI). It is possible that a new Requested NSSAI is incompatible with any of the S-NSSAIs in the Pending NSSAI.

SA2 has agreed that when NSSAA completes for a S-NSSAI, the AMF checks if this S-NSSAI is compatible with the Allowed NSSAI and, if not, they are explicitly removed from the Pending NSSAI, but they are not added to the allowed NSSAI as in the attached CRs SA2 has agreed.

2. Actions:

To CT1 working group.

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#153e		10-14,Oct, 2022	E-Meeting

3GPP TSG SA2#154e		14-18, Nov,2022	E-Meeting