SA WG2 Meeting #152e	S2-2206829r01

# August 17th – August 26th ; Elbonia               			(revision of S2-220)

Source: 	Intel

Title: 	KI#5 Interim Conclusion

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution:

# 1 Discussion

This pCR provides the interim conclusion proposal for key issue #5. Based on the evaluation of solutions provided in S2-2206828, it is proposed to agree the interim conclusion for KI#5 as provided below.

# 2 Proposal

It is proposed to update TR 23.700-81 as follows.

Start of change

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X   KI#5: Enhance trained ML model sharing

It is recommended to use the following principles and procedures as the basis for the normative work.

- The following are the trained ML model sharing parameters to enable ML model sharing between different vendors - Interoperability indicator, ML model filter information i.e., ML model file format supported including serialization file formats, Supported AI Framework Information per Analytics ID(s).

- During registration of NWDAF containing MTLF with NRF, ML model filter information specific to trained ML model sharing are provided as part of NF profile to the NRF.

- During NWDAF containing MTLF discovery procedure, the consumer (NWDAF containing AnLF) may include ML model filter information (ML model file format and AI Framework information) per Analytics ID(s) in the discovery request.

NOTE: When ML model containing MTLF and NWDAF containing AnLF belong to different vendors, how the ML model privacy is supported is in scope of SA3.

End change