SA WG2 Meeting #S2-152e(e-meeting)	S2-2205658

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell

Title:	KI#6, Conclusion

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN/Rel-18

Abstract of the contribution: Introduction interim conclusion for KI#6.

# 1. Introduction

This paper introduces interim conclusion for KI#6

# 2. Discussion

Based on the current proposed solutions and initial evaluations in TR 23.700-88 for KI#6 and considering the architecture issue addressed in KI#1, it’s rather clear that there is no clear benefit and reason for 5GC to be involved in the PIN service (e.g., no correlation between the PIN services and functionalities offered by 5GC). Thus, it’s recommended that the main PIN functionality shall be maintain in the PIN network (e.g. PEMC, PEGC, PINE and AF manages PIN related information). The PIN application-level communication between the PIN application server and the various PIN elements (e.g. PEGC, PEMC, PINE) may happen over the UP as proposed in the architecture option proposed in Solution #0D.

The 5GS may be provided with parameters for policy and QoS control for PIN traffic based on existing logics where AF (including PIN AF) can push necessary information to 5GC.

Based on above, it’s proposed to support the conclusions below for KI#6.

# 3. Proposal

This paper proposes the following conclusion points to TR 23.700-88 clause 8.6.

* Start of change *

## 8.6	Conclusion on Key Issue #6

The following interim conclusions are agreed for policy and parameters provisioning for PIN:

-	The AF for PIN service may provide the necessary PIN related parameters to 5GC for differentiation and policy/QoS control of specific PINE traffic via 5GC.

- 	The AF for PIN service may provide necessary PIN related parameter to PEGC (e.g. URSP like information) via 5GC.

-	PIN related policy and parameters are exchanged between AF and PEMC/PEGC/PINEs, or between PEMC and PINEs.

-	Information details exchanged in the PIN (i.e. between AF and PEMC/PEGC/PINEs, or between PEMC/PEGC and PINEs) is based on application layer implementation and it is out of scope of SA2.

* End of changes *