3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2205700

# Elbonia, August 17 – 26, 2022	(revision of S2-2202047)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3583 | rev | 2 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Mapped NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI 17 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This is a resubmission of S2-2202047(CR 3583) which is agreed in SA2 #150e but noted in SA#96e. This CR proposed to clarify the mapped NSSAI, in order to eliminate the confusion pointed out by the LS (C1-222095) from CT1 and align with the SA2 logic as described in the LS reply in the agreed S2-2203151 in SA2#150e. It is proposed to clarify the clause 5.15.4.1.2 for both non-roaming and roaming scenarios as the following: - The network shall provide mapping of Allowed NSSAI if the HPLMN S-NSSAI is different from the Serving PLMN S-NSSAI. - If all the S-NSSAIs in an Allowed NSSAI have the same values as the S-NSSAIs in the UE's current Network Slice configuration information for the Serving PLMN, the network need not provide the mapping information to the UE. - The UE provides the mapping of Requested NSSAI for registration, if the mapping information is provided to the UE, to ensure that the network is able to verify whether the S-NSSAI(s) in the Requested NSSAI are permitted based on the Subscribed S-NSSAIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on Mapped NSSAI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of providing mapped NSSAI to UE is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.4.1.2	Mapping of S-NSSAIs values in the Allowed NSSAI and in the Requested NSSAI to the S-NSSAIs values used in the HPLMN

For both non-roaming and roaming scenarios, one or more S-NSSAIs in an Allowed NSSAI provided to the UE can have values which are not part of the UE's current Network Slice configuration information for the Serving PLMN. In this case, the network shall provide the Allowed NSSAI together with the mapping of each S-NSSAI of the Allowed NSSAI to the corresponding S-NSSAI of the HPLMN. If all the S-NSSAIs in an Allowed NSSAI have the same values as the S-NSSAIs in the UE's current Network Slice configuration information for the Serving PLMN, the network need not provide the mapping of Allowed NSSAI to the UE. This mapping information allows the UE to associate Applications to S-NSSAIs of the HPLMN as per NSSP of the URSP rules or as per the UE Local Configuration (if available), as defined in clause 6.1.2.2.1 of TS 23.503 [45] and to the corresponding S-NSSAI from the Allowed NSSAI.

For both non-roaming and roaming cases, during registration procedure the UE provides the mapping of S-NSSAIs values in the Requested NSSAI to the corresponding S-NSSAI values used in the HPLMN, if the mapping information is provided to the UE, to ensure that the network is able to verify whether the S-NSSAI(s) in the Requested NSSAI are permitted based on the Subscribed S-NSSAIs, as defined in clause 4.2.2.2.2 of TS 23.502[3]. These values are found in the mapping previously received from the Serving PLMN of the S-NSSAIs of the Configured NSSAI for the Serving PLMN or of the S-NSSAIs of the Allowed NSSAI for the Serving PLMN and Access Type to the corresponding S-NSSAIs values used in the HPLMN.

# * * * * End of changes * * * *