SA WG2 Meeting #152E (e-meeting)	S2-2206003

17 - 26 Aug, 2022, Electronic meeting

Source:	Samsung, Nokia, Nokia Shanghai Bell, Interdigital, LG Uplus

Title:	New WID: NSSRG in EPS

Document for:	Agreement

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: NSSRG in EPS

Acronym: TEI18_NSSRG_EPS

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

To meet the GSMA requirement for constraints on simultaneous use of network slices, NSSRG (Network Slice Simultaneous Registration Group) was introduced. With the help of NSSRG, AMF in 5GS ensures that UE is not able to use slices simultaneously which does not share the common NSSRG information. But this GSMA requirement is not fully implemented as the same UE may be able to get services of the slices which does not share a common NSSRG information if it is attached to EPS and triggers multiple PDN connections. There are two scenarios/use cases which will occur when UE trigger multiple PDN connections in the EPS and this NSSRG feature does not work.

1st use case (when PDN connections are handled by the same SMF+PGW-C)

At the moment when SMF+PGW-C supports more than one S-NSSAI and APN is valid for more than one S-NSSAI then during PDN connection establishment SMF+PGW-C provides one S-NSSAI which is part of the UE’s subscribed S-NSSAI and the S-NSSAI is not subjected to NSSAA. For the subsequent PDN connections from the same UE, if SMF+PGW-C does not consider the NSSRG information then there is probability that SMF+PGW-C will provide one slice which may not share the common NSSRG information with the S-NSSAI of the already established PDN connections and as a consequence it will lead to failure of the fulfilment of GSMA requirement.

2nd use case (when PDN connections are handled by different SMF+PGW-Cs)

If the subsequent PDN connection land at a different SMF+PGW-C, then most probably it will provide one S-NSSAI which may not share a common NSSRG information with the S-NSSAI of the already established PDN connections as it is not aware of any established PDN connection that is handled by a different SMF+PGW-C and as a consequence it will again lead to the failure of the fulfilment of GSMA requirement.

# 4	Objective

The objective is to address the NSSRG in EPS by:

-	Assuming that SMF+PGW-C consider the received NSSRG information from UDM while providing the S-NSSAI for subsequent PDN connections.

-	Also SMF+PGW-C queries the UDM for any PDU Session ID(s) and S-NSSAI(s) that are already registered in the UDM for the UE.

Note: This work will require 0.5 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.502 | Add description that SMF+PGW-C queries the UDM to know about any existing PDU sessions and corresponding S-NSSAIs and then considering NSSRG information while providing S-NSSAI to UE in PCO. | SA#99 (March 2023)? |  |

# 6	Work item Rapporteur(s)

Ashok Kumar Nayak, Samsung (ashok.nayak@samsung.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Interdigital

KT Corp.

LG Uplus

Nokia

Nokia Shanghai Bell

Samsung

Xiaomi