3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206636

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5: Evaluation and Conclusion for Layer-2 UE-to-Network Relay

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This paper proposes the evaluation and conclusion for KI#5 on multi-path transmission for Layer-2 UE-to-Network Relay.

# 1. Introduction/Discussion

This KI covers support of multi-path transmission for UE-to-Network Relay, including both Layer-2 and Layer-3 UE-to-Network Relay cases.

This contribution provides an overall evaluation for the Layer-2 UE-to-Network Relay case and corresponding conclusion.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change (all new) * * * *

## 8.X	Key Issue #5: Support of multi-path transmission for UE-to-Network Relay

This KI covers support of multi-path transmission for UE-to-Network Relay, including both Layer-2 and Layer-3 UE-to-Network Relay cases.

For Layer-2 UE-to-Network Relay:

-	Sol#26 proposes a high-level procedure for multi-path transmission via Layer-2 UE-to-Network Relay. NG-RAN can decide that multi-path transmission for a QoS flow can be performed during or after a QoS Flow establishment, considering the authorization information from AMF. Coordination with RAN WG2 is needed for solution alignment and update based on RAN WG2 progress.

-	Sol#28 proposes that similar approach with Multi-Radio Dual Connectivity (MR-DC) can be introduced to support multi-path transmission for a CM-CONNECTED Layer-2 Remote UE, and the operation details of the MR-DC is within RAN WG scope. The aligned with Sol#26, however lacks the procedures.

-	Sol#39 proposes that the multi-path transmission for Layer-2 UE-to-Network Relay is achieved by using Redundant PDU Sessions. However, this solution only improves reliability, and is not used for improved data rates for the Remote UE.

-	Sol#40 proposes the 5GC influenced multi-path transmission via Layer-2 UE-to-Network Relay. SMF determines the multi-path transmission guidance and provides the guidance to NG-RAN, to instruct NG-RAN that which QoS flow should be duplicated or split. The principles of multi-path transmission rely on NG-RAN determination with also 5GC guidance. However, in MR-DC, it is the RAN itself that makes the determination and does not depend on 5GC guidance. So a similar mechanism should be used for alignment with existing RAN procedures.

For Layer-3 UE-to-Network Relay:

Editor’s Note: The evaluation for Layer-3 UE-to-Network is FFS.

# * * * * Second change (all new) * * * *

## 8.X	Key Issue #5: Support of multi-path transmission for UE-to-Network Relay

For Key Issue #5 (Support of multi-path transmission for UE-to-Network Relay), the followings are taken as conclusion:

-	Sol#26 can be used as baseline for multi-path transmission via Layer-2 UE-to-Network Relay.

NOTE:	Coordination and update are needed based on RAN WG2 progress during the normative phase.

Editor’s Note: The conclusion for Layer-3 UE-to-Network is FFS.

# * * * * End of changes * * * *