3GPP SA WG2 Meeting #153e	S2-2208431

# e-meeting, Oct 10-17, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI#2 Conclusion Updates to reduce network load

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution:  This paper proposes updates to interim conclusions for key issue 2.

# Discussion

The Key Issue 2 definition in Clause 5.2 contains the following

Solutions for this key issue shall not cause degradation of UPF performance.

NOTE:	Coordination with FS_ UPEAS / Study on UPF enhancement for Exposure and SBA may be needed.

Interim conclusions for key issue 2 reference solution 9, which contains the following:

Table 6.9.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF / UPF | UE ID for the UE that uses the application. |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Start/end time | UPF | Start and end time of traffic detection. |
| Data volume | UPF | Measured data traffic volume (per UL/DL). |
| Data duration | UPF | Measured average data traffic (packets) duration (per UL/DL). |
| QoS flow Bit Rate | UPF | The observed bit rate (per UL/DL). |
| Packet transmission | UPF | The observed average number of packet transmission (per UL/DL). |
| UL/DL Packet Delay | UPF, AF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| IP 3-tuple | UPF | to Identify a service flow of the UE that uses the application. |
| Packet's URL | UPF | URL is extracted from the inspected user plane packets. |
| Packet's Domain Name | SMF/UPF | The domain name is extracted from the inspected user plane packets in a QoS flow. |
| Size of packets | SMF/UPF | Average size of packets. |
| PFD Information | NEF(PFDF) | PFD Information stored in the NRF and retrieved by NEF, as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name | NEF(PFDF) | A Domain name matching criteria and information about applicable protocol(s). |

NOTE 1:	Extensive reporting of all traffic flows may conflict with requirement to avoid extra UPF load in the key issue. The issue regarding the performance of UPF user plane traffic handling will depend on the outcome of FS-UPEAS SID.

So the issue of extensive load is anticipated to be resolved by FS_ UPEAS. However, this study merely provides a framework for event subscriptions at the UPF. It is rather the subscribed events that determine the load. Those events are defined for instance by solution 9.

Extensive flow related reporting at the UPF for the events listed in solution 9 may lead to high signalling load and high processing load e.g. at UPF

It is thus suggested to add the following Note:

NOTE 1:	Extensive reporting related to all traffic flows at the UPF should be avoided due to the high UPF load. An NWDAF preferably subscribes only for reporting for some UEs to limit the load.

# Proposal

It is proposed to update TR 23.700-81 as follows

*** Start of changes ***

## 8.2	Key Issue #2: NWDAF-assisted application detection

Interim conclusions:

It is concluded to focus on the use case of PFD updates for known applications in Solution#9.

NOTE 1:	Extensive reporting related to all traffic flows at the UPF should be avoided due to the high UPF load. An NWDAF preferably subscribes only for reporting for some UEs to limit the load.

NOTE 2:	Coordination with the FS_UPEAS conclusions is required for final conclusions.

## *** End of changes ***