SA WG2 Meeting #153e	S2-2208533

10 – 17 October 2022, e-Meeting	(revision of S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	KI#1: Conclusions

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: Proposes conclusions for KI #1.

# 1. Introduction

Conclusions for KI #1 in TR 23.700-71 for KI #1 are provided here.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-71.

*** First Change (all new) ***

# 8.1	Key Issue #1: Architectural Enhancement to support User Plane positioning

The following aspects are concluded for normative support.

-	PDU session establishment between a UE and LMF is supported as in Solution #1.

-	User plane connection establishment between a UE and AF or LCS Client is supported as in Solution #19 for a periodic or triggered MT-LR for the case where UE based location is used and where the UE determines location estimates.

-	The user plane connections for Solutions #1 and #19 are supported using TCP/IP for transport and common security procedures and protocol to be determined by SA3.

-	Upper layer protocols for Solutions #1 and #19 can differ.

*** End of Changes ***