3GPP SA WG2 Meeting #153E	S2-2208181r03

Elbonia, October 10 - 17, 2022	(was S2-22XXXX)

Source:	Ericsson

Title:	KI#1, 4: Update to solution 20

Document for:	Discussion/Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC

Abstract of the contribution: This contribution proposes an update to solution 20.

# 1. Proposal

It is proposed to accept the following changes to TR 23.700-78.

FIRST CHANGE

## 6.20	Solution #20: Merged solution to support Data Channel usage in IMS network

### 6.20.1	Description

#### 6.20.1.1	General

This solution addresses Key Issue #1 "Enhancement to support Data Channel usage in IMS network" and focuses on how IMS network architecture and procedures are enhanced to support Data Channel usage. This contribution is trying to merge existing solutions of DC architecture and procedure in TR 23.700-87 to establish a joint proposal for evaluation and conclusion for KI#1.

Generally, this contribution includes principles and contents from solution #1, solution #3, solution #6, solution #7 and solution #16.

The interim conclusion in clause 9.1 is also considered when shaping the merged solution.

This solution supports separation of signalling function and media function supporting Data Channel. A new service based function, Data Channel Signalling Function (DCSF), is introduced. There are two ways to provide the DC media function to fulfil different requirements of IMS migration to support DC:

-	by enhancing the existing IMS MRF (called enhanced MRF) to perform media functions related to DC, which interacts with IMS AS via legacy MRF interfaces, i.e. Mr'/Cr; and

-	by introducing a new service based function called Data Channel Media Function (DCMF), which interacts with IMS AS via the new SBA interface DC2.

For both options, the DC media function is specified in clause 6.20.1.2.

NOTE 1:	The above MRF can be a standalone enhanced MRF that performs only DC functions, or a combined MRF that performs the DC functions as well as other media functions (e.g. voice/video, etc).

NOTE 2:	DCMF can also be collocated with other functional entities.

The discovery and selection of DCSF, DCMF,/enhanced MRF and IMS AS are supported by NRF based mechanisms, referring to solutions #14 and #17.

#### 6.20.1.2	Architecture

The two architecture options of this solution are shown in Figure 6.20.1.2-1 and Figure 6.20.1.2-2.

Figure 6.20.1.2-1: Architecture option of IMS supporting DC usage with DCMF

Figure 6.20.1.2-2: Architecture option of IMS supporting DC usage with enhanced MRF

The following functional entities are introduced or enhanced to support DC usage in IMS network:

1)	Data Channel Signalling Function (DCSF):

-	Implements data channel control logic through DC1 control API;

-	Receives event reports from the IMS AS and decides the usage of data channel;

-	Manages bootstrap data channels and (if applicable) application data channels by interacting with IMS AS;

-	Enables uploading of data channel applications as specified in TS 26.114 [6], however this is out of scope;

-	Controls download of IMS data channel applications to the UE;

-	Supports HTTP web server to terminate HTTP traffic associated with downloading of data channel applications via DCMF and/or enhanced MRF. Additionally, may act as a proxy for HTTP for resources hosted by external third-party applications;

-	Interacts with NEF for data channel capability exposure via N33 and MDC3;

-	Interacts with DC application Server via DC3 for DC resource control and traffic forwarding via MDC3.

2)	Data Channel Application Repository (DCAR):

-	Supports data channel applications storage which can be internally integrated inside the DCSF or standalone.

NOTE 1:	When the DCAR is separate from the DCSF, the interface DC5 between DCAR and DCSF is out of scope of 3GPP.

3)	Data Channel Media Function (DCMF) / enhanced MRF:

-	DCMF and enhanced MRF support the same DC media functionalities as below;

-	Manage the media resources for data channel media (bootstrap and application data channel if applicable) under the control of IMS AS;

-	Terminates bootstrap data channel to/from the UE and forwards HTTP data channel application traffic between the UE and DSCF via MDC1;

-	Anchors application data channel in P2P scenarios, if required, and accordingly supports e.g. data traffic forwarding and possible media interworking between DCMTSI client and MTSI client;

-	Terminates application data channels from the UE and forward data channel traffic to DC Application Server via MDC2 in A2P/P2A scenarios.

NOTE 2:	The definition of DC Application Server is out of scope of 3GPP.

4)	IMS AS:

-	Interworks with DCSF via DC1 for event notification for data channel and data channel control procedures;

-	Interacts with DCMF via DC2 and with enhanced MRF via Mr'/Cr for data channel media resource management.

NOTE 3:	The IMS AS may be integrated with the DCSF in which case the DC1 interface is internal.

The following service based reference points are introduced or enhanced to support DC usage in IMS network:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and DCMF.

-	DC3:	Reference point between the DCSF and NEF.

-

-	DC5:	Reference point between the DCSF and DCAR.

NOTE 4:	DC5 are not specified in 3GPP.

Editor's note:	Whether DC3 is specified in this release is FFS.

The following reference points are introduced or enhanced to support DC media handling:

-	Mb:	Existing reference point used for transport of data channel media between DCMF and IMS-AGW.

-	Mr'/Cr:	SIP based reference point between IMS AS and enhanced MRF

-	MDC1:	Reference point for transport of data channel media between DC media function (either DCMF or enhanced MRF) and DCSF.

-	MDC2:	Reference point for transport of data channel media between DC media function (either DCMF or enhanced MRF) and DC Application Server.

-	MDC3:	Reference point of data channel media between DCSF and DC Application Server.

NOTE 5:	MDC1, MDC2 and MDC3 are not specified in 3GPP.

DC1, DC2 and DC3 are service based interfaces so that the following services are to be specified:

-	Ndcsf:

-	Exposure of data channel control. (Possible service consumer: NEF).

-	Ndcmf:

-	Management of data channel media resources. (Possible service consumer: IMS AS).

-	Nimsas:

-	Notification of data channel media events, e.g. media resource status or media information, to IMS AS during an IMS session. (Possible service consumer: DCSF).

-	Management of data channel media resources. (Possible service consumer: DCSF).

-	Nnef:

-	Enhancements on exposure for data channel capabilities (Possible service consumer: DC Application server).

Editor's note:	Whether enhancements to Nnef is specified in this release is FFS.

### 6.20.2	Procedures

#### 6.20.2.1	NF service registration and discovery

The DCSF, DCMF/ MRF and IMS AS shall register their services at the NRF before providing services to consumers.

The registration and discovery of DCSF and IMS AS refers to solution #17.

The registration and discovery of DCMF/ MRF refers to solution #14.

#### 6.20.2.2	UE initiated bootstrap data channel establishment

Figure 6.20.2.2-1: UE Initiated Bootstrap Data Channel Establishment 1(2)

1.	UE#1 sends the SIP INVITE request with an initial SDP to AS, through originating network P-CSCF and S-CSCF. The initial SDP contains offers for the bootstrap data channel establishment requirement. In this example procedure, SDP contains both bootstrap data channel offers for originating side and terminating side.

NOTE:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup.

NOTE:	The trigger for a DC subscribed UE to initiate or not initiate a DC request is to be determined during the normative phase.

2.	IMS AS validates the Data Channel media description information and/or user subscription data to determine whether the DC Call request should be triggered to DCSF.

If the DC call request needs to be sent to DCSF, the IMS AS selects a DCSF for this session based on local configuration or discovery and selection of a DCSF instance via NRF.

3.	IMS AS notifies the DCSF the call event request.

4.	After receiving the DC control request, the DCSF determines the policy about how to process the bootstrap data channel establishment requirement based on the related parameters in the DC control request and/or operator strategy.

5.	Since the SessionEstablishmentEvent indicates that served user offered a local BS stream, DCSF prepares originating side MDC1 media information required by DCMF.

6. DCSF requests IMSAS to connect the served user offered local BS stream to the DCMF.

7.	The IMS AS selects a DCMF based on local configuration or discovers and selects a DCMF instance or enhanced MRF supporting DC media function via NRF.

8.	IMS AS invokes DCMF service to instruct DCMF to allocate an Mb resource to terminate the media stream specified by the DCSF. For IMS AS to stand alone DCMF, DC2 is used to reserve data channel media resources. If enhanced MRF is used, IMS AS uses Mr'/Cr to enhanced MRF to reserve data channel media resources.

The media resource for interface MDC1 to DCSF is also reserved based on DCSF media information created in step 5.

9. The DCMF/MRF responds with an SDP answer.

10. IMS AS acknowledges the MRF created endpoint.

11.	IMS AS responds to  the DCMF connect control request to DCSF. The response may include the success of the operation.

12.	The DCSF stores the media resource information and prepares for bootstrap DC traffic to originating UE#1. 
The DCSF prepares terminating side media information required by DCMF to offer a remote bootstrap stream to terminating UE#2.

13. DCSF requests IMSAS to offer a DCMF connected stream.

14. IMS AS invokes DCMF service to instruct DCMF to offer a Mb media stream.

15. DCMF replies with an SDP offer as per DCSF provided MDC1 media specification.

16. IMS AS responds to the DCMF connect control request to DCSF including media stream identity created.

17. DCSF prepares for remote bootstrap DC traffic to terminating UE#2. DCSF responds to the Notify operation.

NOTE:	Steps 6~16 may be collapsed to operations including instructions for multiple media streams in a single request.

18-20.	IMS AS sends the INVITE which includes the updated SDP offer adding media information of DCMF or enhanced MRF to originating S-CSCF and then to remote network side and UE#2. The SDP offer for bootstrap data channel to UE#2 will in this case be included.

Figure 6.20.2.2-2: UE Initiated Bootstrap Data Channel Establishment 2(2)

21-23.	UE#2 and terminating network returns an 18X response with SDP answer to bootstrap DC to originating network. If the terminating network decides to establish another bootstrap DC to UE#1 and UE#2, the corresponding data channel media description is also included in SDP answer.

24.	IMS AS invokes DCMF service to instruct DCMF on data channel media resource modification based on the SDP answer received from terminating network. For IMS AS to stand alone DCMF, DC2 is used to reserve data channel media resources. If enhanced MRF is used, IMS AS uses Mr'/Cr to enhanced MRF to modify data channel media resource.

25. DCMF acknowledges the update.

26.	IMS AS notifies the provisional response event  to DCSF.

27.	The DCSF responds to the notification.

28-29.	The IMS AS modifies SDP answer for bootstrap data channels to UE#1 in 18X response which may include local bootstrap data channel to UE#1 and/or remote bootstrap data channel to UE#1 and sends 18X response to S-CSCF and P-CSCF.

30.	The originating network P-CSCF executes QoS procedure for bootstrap data channel media based on the SDP answer information from the 18X response.

31.	P-CSCF returns the 18X response to UE#1.

32.	The PRACK and UPDATE negotiation between originating and terminating network.

33-34. UE#2 and terminating network returns a 200 OK response.

35. IMS AS notifies the successful session establishment event to DCSF.

36. The DCSF responds to the notification.

37-39. 200 OK forwarded to UE#1.

40~41.	The bootstrap data channels have been established between originating DCMF or enhanced MRF and UE#1/UE#2. The UEs send application request messages to DCMF or enhanced MRF via bootstrap data channel with its data channel capabilities. The DCMF or enhanced MRF forwards the message to received media point of DCSF. The DCSF provides the proper data channel applications to UE#1 and UE#2 based on their data channel capabilities through DCMF or enhanced MRF.

42.	The bootstrap data channels have been established between terminating DCMF or enhanced MRF and UE#1/UE#2. The data channel application is requested and downloaded to UE#1 and UE#2 from terminating DCSF.

43.	The subsequent procedures continue.

#### 6.20.2.3	UE initiated P2P application data channel establishment

Figure 6.20.2.3-1: UE Initiated P2P Application Data Channel Establishment

0.	IMS session and bootstrap data channel has been established. Data channel applications have been downloaded to UE#1 and UE#2.

1.	UE#1 sends the SIP reINVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the established bootstrap data channel information, as well as the application data channel establishment requirement which is indicated by stream ID (dcmap value: greater than or equal to 1000), stream Label, etc.

2.	IMS AS validates the Data Channel media description information and/or user subscription data to determine whether the media change request should be triggered to DCSF.

3.	IMS AS notifies the DCSF the media change request event.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment requirement based on the related parameters in the notification and/or operator strategy.

5.	DCSF determines (in this example) that the added application data channel stream of the offer takes UE#2 as target endpoint not requiring anchoring in the local DCMF or enhanced MRF. If DCMF or enhanced MRF needs to anchor application data channel, DCMF would have used the Nimsas_MediaStreamControl service at this point.

6.	DCSF responds to the notification.

7-8.	IMS AS sends the reINVITE to originating S-CSCF and then to remote network side and UE#2.

9-11.	UE#2 and terminating network returns a 200 OK response with SDP answer for application DC to originating network.

12.	IMS AS notifies the successful data channel modification to DCSF.

13.	DCSF responds to the notification.

14-15.	The IMS AS sends 200 OK response to S-CSCF and P-CSCF.

16.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

17.	P-CSCF returns the 200 OK response to UE#1.

18.	UE#1 send ACK to the terminating network.

19.	The application data channel between UE#1 and UE#2 is established, in this example, not anchored on DCMF/enhanced MRF.

#### 6.20.2.4	UE initiated P2A/A2P application data channel establishment

Figure 6.20.2.4-1: UE Initiated P2A/A2P Application Data Channel Establishment

0-3.	Steps 0-3 of clause 6.20.2.3 applies.

4.	After receiving the media change request notification, the DCSF determines the policy about how to process the application data channel establishment requirement based on the related parameters in the notification and/or operator strategy.

5.	DCSF determines (in this example) that the added application data channel stream of the offer takes the Data Channel Application as target endpoint, hereby requiring anchoring in the local DCMF or enhanced MRF. DCSF prepares originating side MDC2 media information required by DCMF.

6. DCSF requests IMSAS to connect the offered application data channel stream to the DCMF.

7.	IMS AS invokes DCMF service to instruct DCMF to allocate an Mb resource to terminate the media stream specified by the DCSF.For IMS AS to stand alone DCMF, DC2 is used to reserve data channel media resources. If enhanced MRF is used, IMS AS uses Mr'/Cr to enhanced MRF to reserve data channel media resources.

The media resource for interface MDC2 to DC Application Server is also reserved based on DCSF media information created in step 5.

8. The DCMF/MRF responds with an SDP answer.

9. IMS AS acknowledges the MRF created endpoint.

10.	IMS AS responds to the DCMF connect control request to DCSF.

11.	The DCSF uses the media resource information and establishes MDC2 WebRTC PeerConnection with DC Application Server, for application DC traffic to UE#1 relayed via DCMF or enhanced MRF.

Note: Details on how MDC2 media point information is exchanged between DCMF/MRF and the DC Application Server is FFS.

12. DCSF responds to the Notify operation.

13-14.	IMS AS sends the reINVITE which does not include the SDP offer for application data channel to originating S-CSCF and then to remote network side and UE#2.

15-17.	UE#2 and terminating network returns a 200 OK response with SDP answer for audio/video.

18.	IMS AS notifies the successful media change request to DCSF.

19.	DCSF responds to the notification.

NOTE:	Steps 13~19 may not be necessary if UE#2 and terminating network are not involved in this A2P/P2A session.

20-21.	The IMS AS includes SDP answer for application data channels to UE#1 in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF.

22.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

23.	P-CSCF returns the 200 OK response to UE#1.

24.	UE#1 send ACK to the terminating network.

25.	The application data channel between UE#1 and DC application is established via DCMF or enhanced MRF. DCMF or enhanced MRF forwards data channel traffic between UE#1 and DC application server based on media point information received in step 5 and 11.

#### 6.20.2.5 	IMS AS – DSCF Discovery

DSCF and IMS AS assume there is an implicit subscription to all IMS DC session relevant events in IMS AS for every IMS UE engaged in an IMS DC session. IMS AS notifies the DSCF all relevant IMS DC events requiring action from the DCSF. The IMS AS discovers the DSCF instance to communicate with in one of 2 ways:

_- 	If an NRF is used to register the DSCF NF profile, the IMS AS discovers the DSCF instance to be used for a specific UE engaged in an IMS session using existing procedures based on TS 23 501 [XX].

- If an NRF is not used, the IMS AS is configured with the contact information for a DSCF. This may be the target DSCF instance, or the request could be directed to the target DSCF instance to be used by the IMS AS for subsequent interactions for the subject UE.

#### 6.20.2.6  	IMSAS Services

##### 6.2.20.2.6.1	General

Editors Note: Figures 6.20.2.6.1-1 and 6.20.2.6.1-2 are still under discussion ans is an attempt to probide a solution to handle the first HTTP GET wtih only a root and no context for DCSF.

Figure 6.2.20.2.6.1-1 below depicts the protocol stack per the Mb and MDC1 interfaces of the DCMF, used as a basis for deriving DCMF attributes for operations requesting bootstrap data channel payload relay between a DCMTSI UE and the DCSF, hereby providing appropriate context to DCSF for HTTP requests initiated over bootstrap data channels e.g. DC 0.

Figure 6.20.2.6.1-1: DCMF Mb – MDC1 Protocol View for Service/Operation/Attribute derivation

The DCMF has to provide the DCSF with bootstrap data channel context (served IMS subscriber, IMS call session Identity and data channel) as part of the initial HTTP GET root request on each bootstrap data channel. Once DCSF has received the context of the initial request, it can use standard WEB mechanisms (e.g. web cookies) to have the UE itself to include the context in subsequent HTTP requests within that specific bootstrap data channel. Details on how to represent the context of the initial HTTP request of each bootstrap channel opened, is to be determined during the normative phase.

Figure 6.2.20.2.6.1-2 below depicts the protocol stack per the Mb and MDC2 interfaces of the DCMF, used as a basis for deriving DCMF attributes for operations requesting data channel relay between a DCMTSI UE and the Data Channel Application Server providing the P2A/A2P application.

Figure 6.20.2.6.1-2: DCMF Mb – MDC2 Protocol View for Service/Operation/Attribute derivation

Figure 6.20.2.6.1-3 depicts a functional view DCMF-DCSF view depicting the above protocol stack.

Figure 6.20.2.6.1-3: DCMF – DCSF Functional View

The following table shows the IMSAS Services and IMSAS Service Operations.

Table 5.2.8.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimas_SessionEventControl | Subscribe (currently not used) | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
|  | Decline | Request/Response | DCSF |
|  | AcceptAndReply |  |  |
| Nimsas_MediaStreamControl | ConnectDCMF | Request/Response | DCSF |
|  | DisconnectDCMF | Request/Response | DCSF |
|  | Remove | Request/Response | DCSF |

##### 6.20.2.6.2	Nimsas_SessionEventControl Service

###### 6.20.2.6.2.1	General

Service description: This service enables the consumer to be notified and take part in session events when served IMS subscribers takes part in IMS sessions.

The following operations are available for this service:

-	Notifying IMS session control events of a specific IMS subscriber to NFs.
This service operation reports the event to the consumer that has subscribed implicitly via configuration in the IMS AS.
Note: Explicit subscription management is FFS.

-	NFs consumers can request IMS AS to decline the received session event.

6.20.2.6.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMSAS to notify consumers of session events related to a specific served IMS subscriber. The notification specifies if IMS AS allows the consumer to take part in session event (using operations ‘Decline’ and ‘AcceptAndReply’) and/or media stream control (using the Nimsas_MediaStreamControl service) before replying to the notification.
Note: It must be possible for the consumer to specify event triggering condition on an event identity level. Details on how the consumer announces wanted condition to IMSAS is FFS.

Inputs, Required: SessionID, EventID,

Inputs, Optional: AssertedServedID, CallingID, CalledID, SessionCase, EventDirection, UeMediaCapabilities, MediaStreamInfoList, EventControlOperationAvailableIndication, MediaStreamControlAvailableIndication

Outputs, Required: None.

Outputs, Optional: None

The following events are related information are specified.

Table 6.20.2.6.2.2-1 : List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | AssertedServedID, CallingID, CalledID, SessionCase, EventDirection, UeMediaCapabilities, MediaStreamInfoList, MediaStreamControlAvailableIndication |
| SessionEstablishmenProvisionalEvent | AssertedServedID, CallingID, CalledID, SessionCase, UeMediaCapabilities, MediaStreamInfoList, MediaStreamControlAvailableIndication |
| SessionEstablishmentSuccessEvent | AssertedServedID, CallingID, CalledID, SessionCase, UeMediaCapabilities, MediaStreamInfoList, MediaStreamControlAvailableIndication |
| SessionEstablishmentFailureEvent | MediaStreamControlAvailableIndication |
| mediaChangeRequestEvent | AssertedServedID, CallingID, CalledID, SessionCase, EventDirection, UeMediaCapabilities, MediaStreamInfoList, EventControlOperationAvailableIndication, MediaStreamControlAvailableIndication |
| mediaChangeRequestSuccessEvent | AssertedServedID, CallingID, CalledID, SessionCase, EventDirection, UeMediaCapabilities, MediaStreamInfoList, MediaStreamControlAvailableIndication |
| mediaChangeRequestFailureEvent | MediaStreamControlAvailableIndication |
| SessionTerminationEvent | Status Code |

###### 6.20.2.6.2.3	Nimsas_SessionEventControl_Decline service operation

Service operation name: Nimsas_SessionEventControl_Decline

Description: The consumer requests the IMS AS to decline session event execution with an optional reason phrase. 
The service can be used before responding to a Nimsas_SessionEventControl_Notity request, indicating that the EventControlOperations option is available.

Inputs, Required: SessionID.

Inputs, Conditional: ReasonPhrase.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

###### 6.20.2.6.2.3	Nimsas_SessionEventControl_AcceptAndReply service operation

Service operation name: Nimsas_SessionEventControl_AcceptAndReply

Description: The consumer requests the IMS AS to accept and reply to the session event request without involving the remote party of the session.
The operation can be used before responding to a Nimsas_SessionEventControl_Notity request, indicating that the EventControlOperations option is available.

Inputs, Required: SessionID.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 6.20.2.6.3	Nimsas_MediaStreamControl Service

###### 6.20.2.6.3.1	General

Service description: This service enables the consumer to control IMS AS handling of media streams within an IMS session. The service can be used before responding to a Nimsas_SessionEventControl_Notity request, indicating that the consumer can take part in media stream control.

The following operations are available for this service:

-	NFs consumers can request IMS AS to connect a UE offered media stream to a DCMF resource.
IMS AS will remove the connected DCMF stream from the UE offer before sending the request to the other party of the session.

-	NFs consumers can request IMS AS to allocate a new media stream connected to a DCMF resource. 
IMS AS will add the connected DCMF stream to the offer before sending the request to the other party of the session.

-	NFs consumers can request IMS AS to disconnect a previously connected DCMF stream.

-	NFs consumers can request IMS AS to remove an offered media stream before sending the request to the other party of the session.

###### 6.20.2.6.3.2	Nimsas_MediaStreamControl_ConnectDCMF service operation

Service operation name: Nimsas_MediaStreamControl_ConnectDCMF.

Description: This service enables the consumer to request the IMS AS to either connect a UE offered media stream to a Data Channel Media Function (DCMF), or add a connected DCMF stream to the offer before sending to the other party of the session.
The indication specifying if a DCMF connected stream shall be offered is the absence of the MediaStreamID parameter in the request.
The DCMF_MediaSpecification parameter includes detailed information required by DCMF to provide interworking between Mb and MDC1/MDC2 interfaces.

Inputs, Required: SessionID, DCMF_MediaSpecification.

Inputs, Conditional: None.

Inputs, Optional: MediaStreamID

Outputs, Required: Operation execution result indication, MediaStreamID.

Outputs, Optional: None

###### 6.20.2.6.3.3	Nimsas_MediaStreamControl_DisconnectDCMF service operation

Service operation name: Nimsas_MediaStreamControl_DisconnectDCMF.

Description: This service enables the consumer to request the IMS AS to disconnect a previously connected DCMF stream. IMSAS will remove the connected DCMF media stream from the session.

Inputs, Required: SessionID, MediaStreamID.

Inputs, Conditional: None.

Inputs, Optional: None

Outputs, Required: Operation execution result indication,.

Outputs, Optional: None

###### 6.20.2.6.3.4	Nimsas_MediaStreamControl Remove service operation

Service operation name: Nimsas_MediaStreamControl_Remove

Description: This service enables the consumer to request the IMS AS to remove a specific media stream from a media offer request.

Inputs, Required: SessionID, MediaStreamID.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

#### 6.20.2.6  	DCMF Services

FFS

### 6.20.3	Impacts on existing nodes and functionality

UE impacts:

-	The UE supports indicating its DC capability and discovery of network DC capability;

-	The UE supports SDP negotiation enhancements for bootstrap data channel and application data channel;

-	The UE supports data channel application request, downloading and DC media handling.

P-CSCF impacts:

-	P-CSCF supports media resource reservation and modification for bootstrap data channel and application data channel.

IMS AS impacts:

-	IMS AS supports service based discovery and selection of DCSF, DCMF and enhanced MRF;

-	IMS AS supports call event notification to DCSF via DC1 interface;

-	IMS AS supports DC media management with DCMF via service based DC2 interface and to enhanced MRF via Mr'/Cr interface.

NEF impacts:

-	Exposure for data channel capabilities.

MRF impacts:

-	MRF supports data channel media handling when enhanced with DC media functionality.

NRF impacts:

-	NRF supports service based discovery and selection of DCSF, DCMF and enhanced MRF.

END OF CHANGES