3GPP TSG-SA WG2 Meeting #153e	S2-2208209r01

# Oct 10 – 14, 2022

# Title:	[draft]LS on XR and Media Services

# Response to:

# Release:	Rel-18

# Work Item:	FS_XRM, FS_NR_XR_enh

Source:	3GPP TSG SA WG2

To:	RAN WG2, RAN WG3

Cc:	RAN WG1

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank RAN1’s LS reply (R1-2205531/S2-2205421) on UE power saving for XR and media services. So far, SA2 has started to evaluate and conclude solutions for key issues for the study of XR and Media Services and has reached conclusions in chapter 8 of TR 23.700-60. SA2 would like to coordinate with RAN WGs about the following aspects:

- In KI#3, SA2 has been studying what information is useful and can be exposed by 5GS to the server.

In the above mentioned KI, SA2 concluded to introduce support for L4S using ECN bits marking in the inner user plane IP header for both, UL and DL on a given QoS Flow. The purpose is to signal to the application layer (data source) that, to maintain certain latency, application layer rate adaptation is to be applied. The criteria for NG-RAN to determine the event when to perform the L4S marking is up to NG-RAN implementation. Two variants of L4S marking are concluded to be introduced, (1) L4S marking in the NG-RAN node and (2) L4S marking by the PSA UPF based on information provided by NG-RAN. SA2 would like to ask RAN2 and RAN3 following questions:

- Q1: For the case of variant (1), is it feasible for NG-RAN to perform L4S marking on a given QoS Flow?

- Q2: For the case of variant (2) where the PSA UPF performs L4S marking, is it feasible for NG-RAN to indicate in GTP-U to UPF the event for L4S marking on a given QoS Flow?

- SA2 is also considering RAN node congestion and per QoS flow congestion informationexposure on N2 as well as on N3.

Hence SA2 would also like RAN WGs´ feedback regarding RAN support for reporting via N2 and N3 (i.e. GTP-U to UPF) of following information:

-	Congestion event and congestion level information for a given QoS flow;

-	Round-trip delay for a given QoS Flow and combination of QoS Flows;

-	Mean Data rate for a given QoS flow.

- In KI#4&5, SA2 has been discussing the extension of the 5GS QoS framework to support the efficient handling of PDU Set, mainly including PDU Set identification and PDU Set level QoS. SA2 understands the uplink PDU Set handling is closely related to the interaction between the UE and RAN. Hence, SA2 decides to focus on the downlink PDU Set handling and to align with RAN’s progress and decision for uplink, if any.

- In KI#8, SA2 has discussed possible solutions to enhance power saving. A proposal was made in SA2 whereby the core network would provide to RAN, besides the overall jitter range, also jitter statistics (i.e. an average jitter worked out using a moving window of several periods and the range around that average).  SA2 would like to request RAN WGs to feedback whether they believe that the jitter statistics would also be useful to configure CDRX.

Editor's note:	If the other KIs also need to send LS to RAN to coordinate open questions during the meeting, they can be added in the LS based on consensus.

2. Actions:

To RAN WG1, RAN WG2, RAN WG3:

ACTION: 	SA2 kindly asks RAN WG1, RAN2 and RAN3 to take the above information into account and reply.

3. Date of Next TSG SA WG2 Meetings:

SA WG2 Meeting #154      	14 – 18 November 2022			Toulouse, France

SA WG2 Meeting #155     	20 – 24 February 2023			   	EU, EU