3GPP TSG-SA WG2 Meeting #153e	S2-2208209r12

# Oct 10 – 14, 2022

# Title:	[draft]LS on XR and Media Services

# Response to:	LS on UE Power Saving for XR and Media Services (R1-2205531/S2-2208105)

# Release:	Rel-18

# Work Item:	FS_XRM, FS_NR_XR_enh

Source:	3GPP TSG SA WG2

To:	RAN WG1, RAN WG2, RAN WG3

Cc:

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank RAN1’s LS reply (R1-2205531/S2-2208105) on UE power saving for XR and media services. So far, SA2 has started to evaluate and conclude solutions for key issues for the study of XR and Media Services and has reached conclusions in chapter 8 of TR 23.700-60. SA2 would like to coordinate with RAN WGs about the following aspects:

- In KI#3 (Network exposure), SA2 has been studying what information is useful and can be exposed by 5GS to the server. In the mentioned KI, SA2 clarifies supports for L4S and for exposure of congestion level are pending RAN WG's feedback on the feasibility of RAN judgment and/or exposure of the corresponding info (e.g. per QoS flow congestion level).

  - SA2 is also considering RAN node congestion and per QoS flow congestion information exposure on N2 as well as on N3.

- In KI#4&5 (PDU Set based QoS framework), SA2 has been discussing the extension of the 5GS QoS framework to support the efficient handling of PDU Set, mainly including PDU Set identification and PDU Set level QoS. SA2 understands the uplink PDU Set handling is closely related to the interaction between the UE and RAN. Hence, SA2 decides to focus on the downlink PDU Set handling and to align with RAN’s progress and decision for uplink, if any.

SA2 has agreed to send to the gNB the information captured in TR 23.700-60 clause 8.xx (see agreed pCR S2-2228568rXX).

- In KI#8, SA2 has agreed to send information to the RAN to support CDRX optimizations. In RAN1’s LS reply (R1-2205531/S2-2208105), SA2 notes that RAN1 indicates that it may be helpful for the core network to provide RAN with the following pieces of information: PDU set periodicity and start time, PDU set end indication, PDU set level QoS parameters, PDU set size (number of bits) or number of PDUs in a PDU set, PDU set identity and relationship information among PDUs within the same PDU set, and Jitter information. SA2 would like to inform RAN1 that relying on above listed PDU Set information alone may not result in an optimal CDRX configuration, since a data burst may include one or multiple PDU Sets. As a result, e.g. a PDU set end indication may or may not mark the end of a Data Burst. (See agreed pCR S2-2209159rYY) Furthermore, SA2 would like to point out that a QoS flow may be characterized by multiple PDU set periodicities (i.e., one for each traffic pattern).

2. Actions:

To RAN WG1, RAN WG2, RAN WG3:

ACTION: 	SA2 kindly asks RAN WG1, RAN2 and RAN3 to take the above information into account and reply.

3. Date of Next TSG SA WG2 Meetings:

SA WG2 Meeting #154      	14 – 18 November 2022			Toulouse, France

SA WG2 Meeting #155     	20 – 24 February 2023			   	EU, EU