SA WG2 Meeting #153e	S2-2208222r01

# October 10th – 14th, 2022; Elbonia              		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Update to the solution 20 (UE IP address mapping information exposure by UPF)

Document for: 	Approval

Agenda Item: 	9.25.1

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: Update to the solution 20 (UE IP address mapping information exposure by UPF)

# 1	Discussion

The EN on exposing DNN and/or S-NSSAI OR IP domain is removed as this information is not exposed more than what is defined in R17 specifications, taking e.g. 23.502 Figure 4.15.10-1: AF specific UE ID retrieval which is by the way the baseline of this solution

1.	AF requests to retrieve UE ID via the Nnef_UEId_Get service operation. The request message shall include UE address (IP address or MAC address) and AF Identifier, it may include, MTC Provider Information, Application Port ID, IP domain. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application. If available the AF may also provide the corresponding DNN and/or S-NSSAI.

Following EN is removed as the AF is not meant to keep IP address binding information

Editor's note:	It is FFS how long the binding information can be kept at the AF.

##### Please Note that 29.510 Table 6.1.6.2.21-1: “Definition of type BsfInfo” defines the following

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4AddressRanges | array(Ipv4AddressRange) | O | 1..N | List of ranges of IPv4 addresses handled by BSF. If not provided, the BSF can serve any IPv4 address. |
| dnnList | array(Dnn) | O | 1..N | List of DNNs handled by the BSF. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. If not provided, the BSF can serve any DNN. |
| ipDomainList | array(string) | O | 1..N | List of IPv4 address domains, as described in clause 6.2 of 3GPP TS 29.513 [28], handled by the BSF. If not provided, the BSF can serve any IP domain. |

# 2 Proposal

It is proposed to update TR 23700-62 as follows

* * * Start of changes * * *

Second change

## 8.2	Conclusions for KI#2

editor's note: it is FFS whether The solution described in solution 20 (UE IP address mapping information exposure by UPF) will be endorsed.