3GPP TSG-WG SA2 Meeting #153E	S2-2208242r01

October 10th – 17th, 2022, Elbonia	(revision of S2-220xxxx)

Source:	Ericsson, Vivo

Title:	KI#8: Update: Evaluation and Conclusions

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract: 	An update of the evaluation and conclusions for KI#8 are proposed.

# 1. Discussion

This contribution updates the evaluation for KI#8: Supporting Federated Learning in 5GC.

# 2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-81 v1.0.0.

* * * First change* * * *

## 7.8	Key Issue #8: Supporting Federated Learning in 5GC

Table7.8-1: Solution evaluation for KI #8

| Solution | Common Idea | Main Impacts | Differences |
| --- | --- | --- | --- |
| #21 Federated Learning procedure between different NWDAFs #22 Federated learning group creation #23 Support of federated learning for model training #24 Horizontal Federated Learning among Multiple NWDAFs #53 Horizontal Federated Learning with Multiple NWDAF | - FL Consumer selects Server NWDAF from NRF when the FL operation is triggered by request from FL Consumer - Server NWDAF selects Client NWDAFs from NRF when it determines FL operation is required - Client NWDAFs perform local training and Server NWDAF performs model aggregation based on the interim models trained from Client NWDAFs | - Support to register FL related information of Server NWDAFs and Client NWDAFs to NRF - Support FL aggregation capability in Server NWDAF, and FL participation capability in Client NWDAFs - Support to the interaction between Server NWDAF and Client NWDAFs | - In Sol #21, Client NWDAFs get user consent from UDM before collecting UE related parameters - Sol #22 only defines the FL participants (Server NWDAF and Client NWDAF) selection procedure, but not the FL training procedure - In Sol #23, Server NWDAF selects Client NWDAFs based on the training data, i.e. Event ID, required to train the ML model - n Sol #24, Server NWDAF updates the training status to FL Consumer, and FL Consumer determines to continue or terminate the training process - Sol #53 proposes a solution in the scenario where FL Consumer first selects a Server NWDAF without FL aggregation capability. In this case, the first Server NWDAF selects another Server NWDAF with FL aggregation capability from NRF and is kicked off from FL procedure |
| #51 Selection, Monitoring, and Maintenance of NWDAF(s) for Federated Learning in 5GC | - Server NWDAF selects Client NWDAFs from NRF and Client NWDAFs decide to join or not - Server NWDAF performs Client NWDAF Re-selection when needed - New Client NWDAFs join FL operation in execution phase, triggered by new Client NWDAFs or Server NWDAF | - Support to register FL related information of Server NWDAFs and Client NWDAFs to NRF - Support Server NWDAF to monitor status of Client NWDAFs - Support Client NWDAFs to inform to Server NWDAF | N/A |
| #52 FL training update to NWDAF containing AnLF from NWDAF containing MTLF | - Server NWDAF notifies FL Consumer of the Training Accuracy Metric - Server NWDAF provided re-trained ML model based on the Base Accuracy Metric evaluated by FL Consumer in real application. | - Support Server NWDAF to get the Training Accuracy Metric - Support FL Consumer to get the Base Accuracy Metric | N/A |

Based on the above analysis, we get the following observations:

Observation 1: Sever NWDAF and Client NWDAF registers to NRF with their FL related information, e.g. Analytics ID, ML Filter Information, FL Capability.

Observation 2: FL Consumer or local configuration in Server NWDAF triggers FL operation, and Server NWDAF selects Client NWDAFs from NRF.

Observation 3: Server NWDAF performs model aggregation and Client NWDAFs perform local training. Server NWDAF exchanges FL training related information with Client NWDAFs, e.g. interim model, initial model configuration.

Observation 4: For most solutions, model sharing between NWDAF instances is a prerequisite for FL operation. And it is a general issue studied in KI #5.

Observation 5: There are solutions that propose to improve the FL model by providing FL Consumer's feedback (e.g. model accuracy in real application) to NWDAF Server. And this issue is related to KI #1.

Observation 6: There are solutions that propose the maintenance of FL process, include the procedure which enables NWDAF Server to monitor the status update of the NWDAF Clients and the dynamic procedure which enables NWDAF Clients to join or quit the FL operation in execution phase.

Observation 7: 6 out of 7 solutions focus on horizontal FL.

* * * End of changes * * * *