3GPP TSG-SA WG2 Meeting #153E (e-meeting)	S2-2208249r01

# Oct 10 – 17, 2022, Elbonia																			 (revison of )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0233 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple location report for MT-LR Immediate Location Request for the regulatory service |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, T-Mobile USA |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MLR |  |  |  |  |  |  | Date: |  | 2022-09-30 |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |
| Reason for change: |  | - When SBA functionality for LCS was defined it followed the model established for the non-service based architecture requiring the use of the complex periodic deferred location request if more than one location may be calculated and supplied – however the introduction of SBA interfaces for LCS can enable simpler methods for supplying multiple location for some location scenarios – specifically regulatory services. - The Proposed solution introduces a new procedure for MT-LR Immediate Location Request for the regulatory service to provide multiple consecutive location estimates to LCS Client for target UE during a short period of time. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GC-MT-LR procedure for regulatory location service optionally can send more location estimated to GMLC by reusing Nlmf_location_EventNotify service operation. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If more location estimates are required than the more complex periodic deferred location must be used even for regulatory services. Existing periodic deferred location methods are more suitable for cLBS. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1a.4, 6.1 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 4.1a.4	Immediate Location Request

With an immediate location request, an LCS client or AF sends or instigates a location request for a target UE (or group of target UEs) and expects to receive  one or more responses containing location information for the target UE (or group of target UEs) within a short time period which may be specified using QoS. An immediate location request may be used for an NI-LR, MT-LR or MO-LR.

* * * * Next Change * * * *

### 6.1.x	5GC-MT-LR multiple location procedure for the regulatory location service

Figure 6.1.x-1 illustrates an extension procedure of 5GC-MT-LR procedure for the regulatory location service defined in Clause 6.1.1.

This procedure is applicable for providing multiple location estimates of the target UE to LCS client.

Figure 6.1.x-1: 5GC-MT-LR multiple location procedure for the regulatory location service

- Steps 1-7 for 5GC-MT-LR procedure for the regulatory location service clause 6.1.1 are performed with the following differences:

  - At step 1 in clause 6.1.1 the request from external location services client may include additionally a number of requested locations, and a time interval.

  - At step 4 in clause 6.1.1 Namf_Location_ProvidePositioningInfo service operation invoked by GMLC may include additionally the number of requested locations, the time interval, GMLC contact address and LIR reference number. The GMLC contact address and LIR reference number must be provided if the number of requested locations is available and higher than one. The LIR reference number is allocated by GMLC based on predefined rule.

  - At step 7 in clause 6.1.1 Nlmf_Location_DetermineLocation service operation invoked by AMF may include additionally include the number of requested locations and the time interval and GMLC contact address and LIR reference number.

- LMF performs positioning procedures based on Step 8 for 5GC-MT-LR procedure for the regulatory location service clause 6.1.1

- This step is executed if the number of requested locations is available and higher than one. The LMF invokes an Nlmf_Location_EventNotify service operation towards GMLC and provides the current location of the UE and timestamp of the location estimate, and LIR reference number, and periodic event type.

- This step is executed if step 3 was executed. The GMLC sends the current location of the UE and timestamp of the location estimate to external location services client.

- If step 3 and step 4 were executed, then the LMF repeats what is described in clause 6.1.1 step 8 and determines again a geographical location and optionally a location in local coordinates. Steps 3, 4, 5 are repeated N-2 times where N is the number of requested locations. If the time interval was provided, LMF waits the amount of time provided by the time interval before executing UE positioning again. If the time interval is not provided, an implementation value of the time interval is used.

- Based on Step 9 in clause 6.1.1 the LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the last calculated location of the UE.

- Steps 10-11 for 5GC-MT-LR procedure for the regulatory location service clause 6.1.1 are performed with the following differences:

- At step 11 the last calculated location will be sent from GMLC to LCS client.

* * * * End of Changes * * * *