# 3GPP TSG-SA WG2 Meeting #153E	S2-2208258

Elbonia, 10 - 17 October, 2022	(Revision of S2-220xxxx)

Title:	[Draft]LS On long eDRX support for RRC_INACTIVE

Response to:	-

Release:	Rel-18

# Work Item:	     NR_REDCAP_Ph2

Source:	SA2

To:	RAN3

cc:

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 3705, 23.502 CR 3555

1. Overall Description:

SA2 thanks RAN3 for the answers provided in S2-2208098/R3-225119.

Based on the answer provided by RAN3, SA2 has concluded to support the CN based MT communication handling for UE in RRC_INACTIVE with eDRX (>10.24s) as following.

-	It is agreed to support MT data and signalling handling within the CN when the UE is unreachable due to long extended DRX in RRC inactive.

-	The gNB sends an indication to the CN and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state) when the UE has entered RRC_INACTIVE. This allows the CN to apply the HLcom functionality (e.g. data buffering, notifications to other network and application functions etc) based on the unreachability information the gNB provided (as described in solution #6 of TR 23.700-68).

-	If the gNB has indicated the UE has entered RRC_INACTIVE to the CN, the gNB also notifies the CN about the RRC State transition back to RRC_CONNECTED (as described in solution #6 of TR 23.700-68).

NOTE 1:	If the indication of UE transition to RRC_INACTIVE is not sent (or sent after UE has entered RRC_INACTIVE) by the gNB then until CN receives it the CN cannot apply HLcom functionality and other NFs will not be aware of the UE reachability, and certain HLcom related services provided to the AF via NEF would not be available. Downlink data transmitted from the UPF to RAN might be discarded and not delivered to the UE.

SA2 is currently further discussing if the conditions for gNB to send request for CN based MT communication handling can be implementation based (e.g, CN based MT communication handling is always triggered when gNB decides to apply long eDRX for RRC_INACATIVE, or gNB may decides not to trigger such in certain conditions, e.g., if UE is static and has uplink data only).

SA2 has agreed the attached CRs for the support of eDRX for RRC_INACTIVE state with MT communication handling in CN. In the 23.502 CR, there are new flows introduced with impact on NGAP messages (see ENs).

2. Actions:

To RAN3, RAN2

ACTION: 	SA2 kindly asks RAN3 to provide their views on the condition for triggering CN based MT communication handling and the ENs on NGAP messages.

3. Date of Next SA2 Meeting:

SA2#154                   Nov 14 - 18, 2022                                    Toulouse, France

SA2 Adhoc   	Jan 16 – 20, 2023					Elbonia