SA WG2 Meeting #S2-153e(e-meeting)	S2-2208262

10 - 17 October 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson, Huawei? HiSilicon? Qualcomm? Intel? IPLOOK?

Title:	KI#7, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR/Rel-18

Abstract of the contribution: Introduction evaluations and conclusion for KI#7.

# 1. Introduction

This paper introduces the evaluation for solutions related to KI#7 and interims conclusion for KI#7

# 2. Proposal

This paper proposes the following conclusion points to TR 23.700-68 clause 7 and clause 8.

* Start of change *

# 7	Evaluation

## 7.x	Evaluation on Key Issue #7

### 7.x.1	General

There is one solution, solution #20, documented for KI#7 in the current study.

### 7.x.2	Evaluation on solutions

Solution #20: it proposes to use the existing CAG identifier as the base to control the UE accessing via MBSR (i.e. mobile IAB-node). The MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR. The UE CAG configuration per PLMN follows the legacy mechanism as defined in the TS 23.501[2]. RAN and CN supports the UE access control based on the CAG identifier associated with the cell and the allowed CAG identifiers for the UE. The CAG ID(s) used for the MBSR needs to be separated from those used for fixed CAG networks. This is especially important, as the MBSR may move around and cause unexpected collisions. The solution works for UE that supports CAG functionality and also the UE that does not support CAG functionality. CAG is defined to use for NPN access control

It further proposes that the CAG information in subscription data for UE may can be simplified to be a common flag which can be interpreted by all serving AMF to decide if UE accessing via MBSR is allowed or not. This simplification is needed considering the roaming scenario in this study.

The solution proposes also extra/optional control information (e.g. time and location for access) corresponding to the CAG identifier. This is also needed considering the buss/train scenario where ticket purchasing is required. The CAG ID(s) for the MBSR to announce may be controlled based on time duration, geographic areas. The Allowed CAG ID List for the UE may be associated with a time duration, or geographic areas,

* Next change *

# 8	Conclusions

## 8.x	Conclusions on Key Issue #7

The following conclusions are agreed based on the principles from solution#20:

- CAG Identifier is used to control the access of UE via MBSR (i.e. mobile IAB-node) existing CAG mechanism defined in clause 5.30.3 of TS 23.501 [2] can be used for managing UE’s access to MBSR.

- When the MBSR is allowed to be operated as an IAB node for a PLMN, the MBSR is configured with CAG identifier which is unique within the scope of this PLMN. If the MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR.

- The UE CAG configuration per PLMN follows the legacy mechanism as defined in the TS 23.501 [2].

- RAN and CN supports the UE access control based on the CAG identifier associated with the cell and the allowed CAG identifiers for the UE.

- The legacy UE(s) shall also support to access the MBSR cell based on the CAG identifier.

NOTE:	CAG identifiers value assignment for MBSR will be determined during normative phase, e.g. whether dedicated CAG identifiers will be used for MBSR access and if the range (number) of CAG identifiers needs to be increased.

- UE subscription data may be deployed with a common flag to indicate if access via MBSR is allowed.

- Extra information (e.g. time duration and location information) may be deployed together with the CAG Identifier for MBSR. The enhanced Allowed CAG list will be provided to UE and AMF for enforcement, e.g. not accessing the MBSR cell outside of the time duration or geographic area.

* End of changes *