SA WG2 Meeting #S2-153e(e-meeting)	S2-2208262

10 - 17 October 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson, Huawei? HiSilicon? Qualcomm? Intel? IPLOOK?

Title:	KI#7, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR/Rel-18

Abstract of the contribution: Introduction evaluations and conclusion for KI#7.

# 1. Introduction

This paper introduces the evaluation for solutions related to KI#7 and interims conclusion for KI#7

# 2. Proposal

This paper proposes the following conclusion points to TR 23.700-68 clause 7 and clause 8.

* Start of change *

# 7	Evaluation

## 7.x	Evaluation on Key Issue #7

### 7.x.1	General

There is one solution, solution #20, documented for KI#7 in the current study.

### 7.x.2	Evaluation on solutions

Solution #20: it proposes to use the existing CAG identifier as the base to control the UE accessing via MBSR (i.e. mobile IAB-node). The MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR. The UE CAG configuration per PLMN follows the legacy mechanism as defined in the TS 23.501[2]. RAN and CN supports the UE access control based on the CAG identifier associated with the cell and the allowed CAG identifiers for the UE. The CAG ID(s) used for the MBSR needs to be separated from those used for fixed CAG networks. This is especially important, as the MBSR may move around and cause unexpected collisions. The solution works for UE that supports CAG functionality and also the UE that does not support CAG functionality. CAG is defined to use for NPN access control

It further proposes that the CAG information in subscription data for UE may can be simplified to be a common flag which can be interpreted by all serving AMF to decide if UE accessing via MBSR is allowed or not. This simplification is needed considering the roaming scenario in this study.

The solution proposes also extra/optional control information (e.g. time and location for access) corresponding to the CAG identifier. This is also needed considering the buss/train scenario where ticket purchasing is required. The CAG ID(s) for the MBSR to announce may be controlled based on time duration, geographic areas. The Allowed CAG ID List for the UE may be associated with a time duration, or geographic areas,

It is not clear why the time-based control of allowed CAGs in UE is specific to MBSR and requires specific enhancements. the network (HPLMN) at any time can revoke based on its policy the allowed CAGs.

* Next change *

# 8	Conclusions

## 8.x	Conclusions on Key Issue #7

The following conclusions are agreed based on the principles from solution#20:

- CAG Identifier is used to control the access of UE via MBSR (i.e. mobile IAB-node) existing CAG mechanism defined in clause 5.30.3 of TS 23.501 [2] can be used for managing UE’s access to MBSR.

- When the MBSR is allowed to be operated as an IAB node for a PLMN, the MBSR is configured by the HPLMN ? with CAG identifier which is unique within the scope of this PLMN. If the MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR.

- RAN and CN supports the UE access control based on the CAG identifier associated with the cell and the allowed CAG identifiers for the UE.

NOTE 1:	CAG identifiers value assignment for MBSR will be determined during normative phase, e.g. whether dedicated CAG identifiers will be used for MBSR access and if the range (number) of CAG identifiers needs to be increased.

NOTE 2:	If CAG ID associated with MBSR and CAG ID associated with private network, both are broadcasted how to apply access control will be determined during normative phase.

- UE subscription data may be deployed with a common flag to indicate if access via MBSR is allowed.

-

- if the time when a certain CAG is allowed for a UE is up, the CAG is revoked from the network as per 23.501.

Editor's note:  if the CAG a MBSR broadcasts is only valid in a geographic area, the MBSR is configured with time and geographic area where it can broadcast a CAG. (issue: can the MBSR ALWAYS locate itself, can the network verify the behaviour? is the HPLMN of VPLMN configuring this location restriction?

Editor's note if a CAG is allowed only within a geographic area, for a UE, it is FFS whether there is an assumption the UE can always locate itself before attempting to access the MBSR nd the behaviour for when it cnanot access shall be specified before this can be accepted for normative work.

* End of changes *