SA WG2 Meeting #153e	S2-2208298r07

# Oct 10th – 17th, 2022; Elbonia               			(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics

Title: 	23.700-08: Updates to conclusions for KI#5

Document for: 	Approval

Agenda Item: 	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: The paper proposes updates to conclusions for KI#5: Enabling access to localized services via a specific hosting network.

Merging S2-2208318, S2-2208445, S2-2208772

2 Proposal

It is proposed to update 23.700-08 as follows.

FIRST CHANGE

8.5	Key Issue #5: Enabling access to localized services via a specific hosting network

The following conclusions are agreed for normative work:

-	The existing SoR procedure is enhanced as follows, for the scenario where the UE reuses home network credentials/subscription to access hosting network:

-	The SoR contains a prioritized list of hosting networks. The list is determined by the home network and may be associated with validity conditions, including time and location conditions.

-	When the hosting network is an SNPN which provides localized services and the home network is an HPLMN, the architecture specified in clause 5.30.2.7 of TS 23.501 is reused for the UE to access both home network services (using the SNPN as underlay network) and localized services (via the SNPN).

-	If the hosting network is a PNI-NPN, existing mechanisms (e.g. roaming architecture, network slicing, etc.) are reused for the UE to access home network services.

-	The existing URSP rules or LADN feature can be re-used for a UE to access the Localized services after the UE has registered to a Hosting network. If the UE uses credential from the hosting network, the URSP rules are provided by the PCF of the Hosting network. If the UE uses credential from the home network, the URSP rules are either pre-configured in the UE or provisioned by the PCF of the hosting network if it is allowed by the home network.

-	To support the UE to access the localized service by evaluating the URSP rules in the registered hosting network, and as to comply with the localized service restrictions, the UE applies the Route Selection Validation Criteria also for already established PDU Sessions and their traffic.

END OF CHANGES