3GPP TSG-WG SA2 Meeting #153E e-meeting	S2-2208366r02

E-meeting, 2022‑10-10 -- 2022‑10-17	(revision of S2-220xxxx)

Source:	Ericsson

Title:	KI #6, Conclusions

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: This document proposes conclusions for KI#6.

# * * * * 1st change (all new)* * * *

8.X	Conclusions for KI#6: Avoiding UE to switch away from EC PDU Session

Editor’s Note: this is a list of potential conclusions for KI#6 and it’s expected that a subset is agreed based on discussions/SoH at SA2#153E.

Based on the evaluation in Clause 7.X, the following new functionality for KI#6 is proposed for the normative phase:

1	An “edge-treatment” indication is provided by the 5GC to the UE

2a	The indication is sent via a new optional PCO attribute

2b	The indication is sent via a new optional attribute in URSP rules

3a	The “edge-treatment” indication is a Boolean attribute and is passed by the UE to the upper layers

4a	The “edge-treatment” indication applies to the traffic of all the application(s) mapped onto the given PDU or the traffic of all the application(s) that explicitly requests the use of EDC functionality for the given PDU session according to the SMF-provided indication for the use of EDC functionality

4b	There are flow descriptors associated to the “edge-treatment” indication to identify which traffic the indication applies to

Note: for 2b above, the traffic descriptors correspond to those in the RSD and thus the “edge-treatment” indication applies to the whole PDU Session, so 4b does not apply

5 The AF can influence the provisioning of “edge-treatment” indication by the 5GC to UE.

Besides, there are solutions with no normative impact. Solution #45 describes a solution in the UE upper layers. 3GPP networks can additionally use the methods described in solution #41 for setting the Access Preference and finer control using ATSSS rules provide guidance to the UE not to switch traffic from 3GPP access. These proposals could be documented in an informative Annex.

# * * * * End of changes * * * *