3GPP TSG-WG SA2 Meeting #153E e-meeting	S2-2208366r05

E-meeting, 2022‑10-10 -- 2022‑10-17	(revision of S2-220xxxx)

Source:	Ericsson

Title:	KI #6, Conclusions

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: This document proposes conclusions for KI#6.

# * * * * 1st change (all text below 1st EN is new)* * * *

## 8.6	Conclusions for KI#6: Avoiding UE to switch away from EC PDU Session

Based on the evaluation in Clause 7.6, the following new functionality for KI#6 is proposed for the normative phase:

-	An "edge-treatment" indication is provided by the 5GC to the UE

2a 	via a new optional PCO attribute, indicating that the traffic of the PDU Session or identified by a flow descriptor should avoid being switched away from 3GPP access;

-

2b The indication is sent via a new optional attribute in the URSP rule

The "edge-treatment" indication can be passed by the UE to the upper layers;

-

The "edge-treatment" indication can apply to the traffic of all the application(s) mapped onto the given PDU or the traffic of all the application(s) that explicitly requests the use of EDC functionality for the given PDU session according to the SMF-provided indication for the use of EDC functionality;

-	Flow descriptors can be associated to the "edge-treatment" indication to identify which traffic the indication applies to;

-	The handling of the "edge-treatment" indication in the UE is left to implementation.

There are also solutions with no normative impact, which can be documented e.g. in an informative annex:

-	Solution #45 describes a solution in the UE upper layers.

-	3GPP networks can additionally use the methods described in solution #41 for setting the Access Preference and finer control using ATSSS rules provide guidance to the UE not to switch traffic from 3GPP access.

# * * * * End of changes * * * *