3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208411

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion for KI#5

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This contribution proposes the evaluation and conclusion of KI#5 Improved support of RAs including TAs supporting Rejected S-NSSAIs.

# 1. Introduction/Discussion

KI#5 studies whether and how to allow the UE to initiate a registration for an S-NSSAI which was rejected for the RA when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI.

Currently, 9 solutions are proposed for this key issue mainly in the following two categories:

1. Based on allowed S-NSSAI that is not fully supported in RA (hence Partially allowed/conditionally allowed),, where AMF provides for the S-NSSAI a list of  TA(s) where the S-NSSAI is supported to the UE (solution 11, 23, 25, 26, 29). This does not require additional registrations steps when UE enters or exists these TAI list to register with the AMF the Ue is within the TAIs of the partially/conditially allowed S-NSSAI when the UE enters one of the TAIs indicated to the UE where the S-NSSAI is supported.

- Based on rejected S-NSSAI, where AMF provides rejected S-NSSAI with its reject/support TA or new cause to UE (solution 25, 27, 28, 29, 30, 31)

|  | Category 1: | Category 2: |
| --- | --- | --- |
| Pros | - Does not require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA - PDU sessions need not be re-established and disconnected (see solution 29) | - Reuse most of the current procedures. (only Registration procedure is impacted) - Aligned with the current slice management features, where the network determines and updates UE on the Allowed NSSAI, rejected S-NSSAIs, target NSSAI and so on. |
| Cons | - Features related to slice management such as NSAC will be impacted. (e.g. Cannot be used together) - Procedures related to PDU Session management will be impacted and the details are not fully discussed. - Have impact on Homogenous slice support in RA. AMF will allow the UE to access an S-NSSAI which is only supported in part of the TAs. May cause non-backward compatibility. - The AMF in connected mode can detect when the connectivity of a slice is available or not, but in idle mode the MT services cannot assume where in the RA the UE is so the AMF would page anyhow. In MO case the UE can avoid causing MO access attempts for the slices that are not working in current TA. - There can be RAN impact (e.g. Solution #29) to treat the PDU Sessions. | - Require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA. - Requires to release and re-establish sessions at mobility in an out the areas where the slice is supported. - When entering the area of support of S-NSSAI, the RA size may need to be reduced. |

Therefore, it is proposed to take Category 2 solution as the baseline for KI#5.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change (all new) * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.5	Evaluation for KI#5

The related solutions are #11, 23, 25, 26, 27, 28, 29, 30, 31.

Category 1 is based on allowed S-NSSAI that is indicated to be supported not fully within the RA (hence Partially/ Conditionally allowed), where AMF provides for the S-NSSAI a list of TAI(s)  where the S-NSSAI is partially/conditional allowed to UE (solution 11, 23, 25, 26, 29). This does not require additional registrations steps when UE enters or exists these TAIs in the list indicated to the UE where the S-NSSAI is supported.

Category 2 is based on rejected S-NSSAI that however are supported in some TAIs of the RA (hence Partially/conditionally Rejected), where AMF provides the (Partially/conditionally)rejected S-NSSAI with a list of TAs  TA or new cause to UE (solution 25, 27, 28, 29, 30, 31)  with TA(s) where it is supported (or not supported, whichever is applicable) in the RA to the UE (solution 25, 27, 28, 29, 30, 31). This requires the UE to register with the AMF the Partially/conditionally Rejected S-NSSAI if the UE is still interested in the S-NSSAI when it enters one of the TAIs where the partially rejected s-NSSAI is supported.

Table 7.X-1: Evaluation of KI#5 related principles

|  | Category 1: | Category 2: |
| --- | --- | --- |
| Pros | - Does not require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA - PDU sessions need not be re-established and disconnected (see solution 29) - provides the support, in MO case, the UE can avoid causing MO access attempts for the slices that are not supported in current TA or provide the TAs where the access attempt cna be enabled even without support (assuming UE can be redirected in overlapping TAs) | - Reuse most of the current procedures. (only Registration procedure is impacted) - Aligned with the current slice management features, where the network determines and updates UE on the Allowed NSSAI, rejected S-NSSAIs, target NSSAI and so on. |
| Cons | - Features related to slice management such as NSAC will be impacted. (e.g. Cannot be used together if perfect accuracy is required) - Procedures related to PDU Session management will be impacted - Have impact on Homogenous slice support in RA. AMF will allow the UE to access an S-NSSAI which is only supported in part of the TAs unless additional information is provided where there is overlapping TA and the network can redirect upon a MO access attempt. - The AMF in connected mode can detect when the connectivity of a slice is available or not, but in idle mode the MT services cannot assume where in the RA the UE is so the AMF would page anyhow. - There can be RAN impact (e.g. as illustrated by Solution #29) to treat the PDU Sessions. | - Require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA. This may be heavy if the NSSAA applies. - Requires to release and re-establish sessions at mobility in an out the areas where the slice is supported. this may be heavy if secondary DN authentication is needed - When entering the area of support of S-NSSAI, the RA size may need to be reduced to fit the addition of new S-NSSAIs in the Allowed NSSAI. |

# * * * * Second change (all new) * * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X	Conclusions for Key Issue #5

The following principle is proposed to be the conclusion for normative work:

-  AMF provides information related to the supported or not supported TA(s) regarding a rejected S-NSSAI in the RA to UE in the registration procedure and UE configuration procedure if the UE indicates it supports this feature . The UE shall be able to request a rejected S-NSSAI in a supported TA based on the supported/not supported TA information. A UE receiving this information shall trigger registration update when it requests a rejected S-NSSAI in a supported TA. The current concept of allowed NSSAI and uniform support of it in UE’s RA is not impacted by this feature based on indication of where in RA certain rejected S-NSSAIs are supported/not supported. In this approach, the Allowed NSSAI is still uniformly supported in the RA.

-	if NSAC does not apply, or if the operator is happy to use this feature despite NSAC includes UEs or sessions for UEs that are outside the area where a certain S-NSSAI is supported while still in the RA (so maybe NSAC can be inaccurate), it should be possible for the AMF (for supporting UEs) to indicate to the UE that some S-NSSAIs are partially/conditionally allowed in the RA by indicating the TAs where these are supported or not supported. If so, the UE assumes it can use the connectivity for the slices in the TAs where it is indicated to be supported. The connected mode behaviour can be based on sol.29 principle. Whether the partially allowed NSSAI can be used for redirection or Handover of the UE is to be based on RAN feedback. In this approach, the Allowed NSSAI is still uniformly supported in the RA but the Partially Allowed S-NSSAI is not uniformly supported in the RA.

-	whether to apply one of the two options is a per S-NSSAI decision based on AMF policy, both options can be supported in a PLMN and can be applied simultaneously for one UE for different S-NSSAIs.

Editor's note: the partially/conditionally allowed NSSAI approach above is subject to further discussion and analysis also based on RAN feedback.

# * * * * End of changes * * * *