SA WG2 Meeting #153E eMeeting	S2-2208433

Elbonia, Oct. 10 – 14, 2022 	(revision of S2-20xxxxx)

Source:	Samsung

Title:	KI#4: Updated evaluation and conclusion

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes an update to KI#4 evaluation and a conclusion.

# 1. Discussion

This contribution proposes an update to KI#4 evaluation and a conclusion.

# 2. Proposal

It is proposed to document the following changes in TR 23.700-80.

***** Start of Changes *****

## 7.4	Key Issue #4: Enhancing External Parameter Provisioning

Five different solutions are addressing aspects of key issue #4. Solutions #9, #35 and #36 are addressing only Key Issue #4. Solution #29 addresses both Key Issue #4 and Key Issue #2.

Solutions #28 is more general and addresses aspects of all Key Issues and should be evaluated outside KI#4.

Solution #35 has no impact on services, entities, and interfaces. It proposes to reuse procedures for AF session with specific QoS as in clause 4.15.6.6 and clause 4.15.6.6a of TS 23.502 [4], with specific QoS parameters for the AF session, for some type of AI/ML operations. It also proposes to negotiate and apply a policy for background data transfer as per procedures in clause 4.16.7.2 and clause 4.15.6.8 of TS 23.502 [4], for some other types of AI/ML operations.

In solution #9, the AF provides configuration information required for AI/ML operations related area-based to the PCF, via NEF and UDR, using the service specific parameter provisioning procedure as defined in clause 4.15.6.7 of TS 23.502 [4].

In solution #29, the AI/ML AF provisions AI/ML task related parameters (UE ID or UE group ID, AI/ML application identifier, AI/ML task identifier, AI/ML task duration, AI/ML task valid time) to the 5GS (UDR), reusing procedure for external parameter provisioning from clause 4.15.6.2 of TS 23.502 [4].

In solution #36, the AI/ML AF provides AI/ML assisted expected UE behaviour parameters to the 5GS (UDR), possibly associated with a probability assertion (i.e. confidence) and optionally other parameters (e.g. the level of accuracy), reusing procedure for external parameter provisioning from clause 4.15.6.2 of TS 23.502 [4]. AMF and SMF can then further use these provisioned parameters for their own procedures.

In solution #28, the AI/ML AF provisions an AaaML service profile in 5GS (UDR). AaaML service profile contains objective of target AaaML operation, input of provisioned service parameter(s), and expected output.

The following principles are proposed as potential basis for further evaluation of the solutions:

-	AI/ML AF providing specific policy parameters related to the AI/ML operation type to the 5GC, instead of 5GC deriving such policy parameters from AI/ML operation type:

None of the solutions propose to provision the AI/ML operation type to the 5GC. Solution #29 proposes an AI/ML task identifier but 5GC does not use the information, rather it forwards this to the UE.

Solutions #9 and #29 propose that AI/ML AF provisions configuration information.

More discussion is needed on the exact configuration parameters needed for each AI/ML operation type.

-	AI/ML AF provides one or more AI/ML assisted Expected UE Behaviour parameter(s) and can provide at least probability assertions/confidence levels and accuracy levels to each of them:

This is specific to solution #36.

More discussion is needed on the specific parameters (existing and new ones) to be provisioned, and the related scenarios for using these parameters in the 5GC NFs.

***** Next Change *****

## 8.4	Key Issue #4: Enhancing External Parameter Provisioning

It is proposed that the specific mechanisms below proposed by KI#4 solutions are considered for the normative work on enhancing external parameter provisioning to support application AI/ML operations:

-	The following new parameters related to expected UE behaviour can be provisioned by the AF to the 5GC:

-	Confidence and/or accuracy levels for entries of the expected UE behaviour parameters in Table 4.15.6.3-1 of TS 23.502 [4]. A confidence and/or accuracy threshold may also be provided by the NF requesting expected UE behaviour parameters from UDM. indicating that certain confidence and/or level of accuracy must be met for the parameter(s) to be notified by UDM to the NF.

-	Expected Inactivity Time of AIML traffic, provided as a time interval.

***** End of Changes *****