SA WG2 Meeting #S2-153e (e-meeting)	S2-2208437r02

10 - 17 October, 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Samsung

Title:	KI#9 conclusion

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3

Abstract of the contribution: This pCR is proposed to add conclusion for key issue#9.

1. Discussion

This pCR proposes to add conclusion for key issue#9. The conclusion is based on the evaluation of solutions on KI#9 (S2-2208872)

2. Proposal

It is proposed to adopt the following changes in TR 23.700-71.

*** First change (ALL NEW TEXT) ***

8.9	Key Issue #9: Support of Positioning Requirements Related to Satellite Access

Based on the KI#9 evaluation documented in cl. 7.9, the following aspects are concluded as basis for normative work:

Location Verification assisted by TN access can be started firstly on normative work with no RAN dependency.

Location Verification with RAN coordination depends on RAN conclusion. Normative work can be started after RAN conclusion is reached, which including:

-	Verification of UE location provided via satellite access should be performed leveraging the LCS framework at the 5GC.

-	The AMF is the entity in charge of providing the location verification decision, in line with Rel-17 mechanism of UE location verification.

-	The AMF may trigger location service procedures as defined in TS 23.273 [5] to determine the UE location verification decision and TAI. Location information received at AMF is provided by LMF via the NI-LR procedure. The LMF may decide specific positioning methods to be used for verification based on RAN WG decisions.

-	The AMF may receive assistance information from NWDAF and/or NG-RAN to perform the location verification decision. Which assistance information is provided by NG-RAN depends on RAN WG decisions.

*** End of change ***