SA WG2 Meeting #S2-153e (e-meeting)	S2-2208437r05

10 - 17 October, 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Samsung

Title:	KI#9 conclusion

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3

Abstract of the contribution: This pCR is proposed to add conclusion for key issue#9.

1. Discussion

This pCR proposes to add conclusion for key issue#9.

2. Proposal

It is proposed to adopt the following changes in TR 23.700-71.

*** First change (ALL NEW TEXT) ***

8.9	Key Issue #9: Support of Positioning Requirements Related to Satellite Access

Based on the KI#9 evaluation documented in cl. 7.9, the following aspects are concluded as basis for normative work:

-	Verification of UE location provided via satellite access should be performed leveraging the LCS framework at the 5GC.

-	The AMF is the entity in charge of providing the location verification decision, in line with Rel-17 mechanism of UE location verification.

-	The AMF may trigger location service procedures as defined in TS 23.273 [5] to determine the UE location verification decision and TAI. Location information received at AMF is provided by LMF via the NI-LR procedure. The LMF may decide specific positioning methods to be used for verification based on RAN WG decisions.

-	The AMF may receive assistance information from NWDAF (i.e. UE location related analytics) to perform the location verification decision.

*** End of change ***