3GPP SA WG2 Meeting #153E (e-meeting)	S2-2208467

10th  - 17th October, 2022, Elbonia

Source: 	Verizon

Title: 	KI#6, Sol #32: Update to allow the PCF initiate the move of a PDU session from one slice to another

Document for: 	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes some updates to solution#32 for KI#6. Addressing PCF impact (presently missing) paragraph.

1 Introduction

This paper adds impact paragraph for the PCF that triggers the PDU session transfer in solution #32 addressing KI#6 in TR 23.700-41.

This paper also swaps Notes 1 through 3 with a clearer explained sentences.

2 Proposal

It is proposed to include the following change in TR 23.700-41.

*** START OF CHANGES ***

6.32	Solution #32: Solution for Network Control for UE Slice Use

6.32.1	Description

Operators would like to control the use of a network slice so as to optimize the performance of their network. In addition to that, there can be other reasons for a capability to enforce a certain usage of network slices, e.g. an operator may like to pull a network slice out of service for maintenance or other reasons and would like to let the UE use another network slice instead.

Also, the operator may experience congestion in a network slice due to some unforeseen circumstance for example due to Network Slice Admission Control (NSAC) there may a need to off-load a network slice, or simply due to a large amount of other UEs using the network slice (e.g. to access higher prioritized services or the other UEs are considered to be of higher priority) and therefore the network would like to control the use of the slice so as to alleviate e.g. the congestion.

Irrespective of the reasons, in this solution different actions are proposed depending on the level of control required by the operator. The following network initiated capabilities are proposed to be supported:

Action 1: Deregistering network slices for UEs or deregistering the UEs that are registering in network slices without establishing any PDUs.

Action 2: Deregistering network slices for UEs or deregistering the UEs that have PDU sessions that are not active.

Action 3: Enabling PDU sessions to be transferred from one network slice to another network slice, the other network slice can be previously included in the Allowed NSSAI or will be added to the Allowed NSSAI. This action 3 applies to KI#1 as well

The above capabilities can be applied in the context of deregistering the network slice for a limited number of UEs with progressively applying one or more of the above capabilities, as required, or applying all of the above capabilities in case of a network slice being taken out of service permanently or for a limited time (i.e. in such case the capabilities of case 3 is done first and then subsequently applying the deregistration).

For case 1 and case 2, the AMF can deregister a UE (e.g. when the UE is only registered to one network slice) using existing procedure "network initiated deregistration" as specified in clause 4.2.2.3.3 of TS 23.502 [5] or deregistering a network slice for a UE by removing the S-NSSAI from the Allowed NSSAI using the UE Configuration Update procedure as specified in clause 4.2.4.2 of TS 23.502 [5]. The AMF monitors/acquires the needed information regarding the usage of the network slice, e.g. if no PDU Session is established for the slice according to the network policy and/or if not complying with the network policy the AMF deregisters the UE from the network slice. As stated before, the means of detecting non-compliance are out of scope as well as triggers can be initiated by other NFs such as PCF, NSACF.

For case 3, there are several options:

Option 1.The AMF initiates UE Configuration Update procedure towards the UE. The procedure is extended to support a PDU session transfer request from a source (network slice to be removed) network slice to a new target network slice if the AMF can locate another suitable network slice in the Allowed NSSAI or a new network slice is to be requested. If the target network slice is already in the Allowed NSSAI (before the UE Configuration Update procedure or is added to the Allowed NSSAI by the UE Configuration Update procedure, the UE then uses existing procedures to initiate a new PDU session using the target network slice for that purpose and tears down the PDU session on the source slice. If the target network slice is required to be registered by the UE, i.e. needs to be added to the Allowed NSSAI, the UE first requests the network slice by issuing a registration request and including the target network slice in the Requested NSSAI and then when the target NSSAI is in the Allowed NSSAI the UE establish PDU Session(s) using the target network slice.

Option 2. The AMF initiates a request towards the SMF for a PDU session transfer, extending the existing service Nsmf_PDUSession_UpdateSM Context to support transfer of a PDU session from source slice to target slice. The SMF uses existing session management procedures extended with the necessary information to notify the UE to initiate a new PDU session towards the target slice and to tear down the old PDU sessions on the source slice.

Option 3. The PCF initiates SM Policy Control Update Notify to SMF. The procedure is extended to include the target network slice. Then, as option 2, the SMF uses existing session management procedures with extensions to notify the UE to initiate a new PDU session towards the target slice and to tear down the old PDU sessions on the source slice. The PCF selection of UEs whose PDU sessions need to be transferred is use case dependent and relies on the existing mechanisms. In this option, the target network slice selection is per operator’s policy. To ensure that selected target S-NSSAI is part of the subscribed NSSAI, the SM-PCF selects a target NSSAI via UE-PCF based on per NSSAI policy data received from UDR. When the target S-NSSAI is not in the cell the UE is camped on, the transfer will not take place.

NOTE 4：In the LBO case how the SM-PCF is aware the mapping between the S-NSSAI of serving PLMN and the subscribed S-NSSAI need be clarified.

6.32.2	Procedures

6.32.2.1	PDU session Transfer from Source Slice to Target Slice, UE Initiated approach

In this UE Initiated approach, the UE Configuration Procedure is updated to remove in the Allowed Slice the source slice that needs to be removed. There will be a new control information element (IE) to describe the handling of PDU sessions established on a slice that is now removed from the Allowed Slices. The IE instructs the UE to transfer the PDU session from the removed network slice or to be removed network slice in Allowed NSSAI to a new target network slice in the Allowed NSSAI. The to be removed network slice can either be kept in the Allowed NSSAI for the duration of the UCU procedure and later removed, or directly removed by the UE Configuration Update command, but regarded as still allowed until the transfer to the target network slice has been completed.

The call flow in Figure 6.32.2.1-1 illustrates the procedure depicted above. The following is a brief description of the steps in the call flow.

The assumption in the call flow is a UE that has some PDU Sessions already established on a network slice and these PDU sessions have to be transferred to another network slice.

-	In step 1, the AMF initiates a UE Configuration Update Command, marking in (or in relation to) the Allowed NSSAI the network slice to be removed and including a new IE information to instruct the UE regarding the removed slice (or to be removed network slice) and the new target network slice that replaces it. As stated before, if the target network slice is not added by the UCU procedure to the Allowed NSSAI, the UE first requests the network slice to be registered by issuing a registration request and including the target network slice in the Requested NSSAI and then when the target NSSAI is included in the Allowed NSSAI the UE establishes PDU Session(s) using the target network slice. Note that the Figure below does not show the aspect of UE registering to request the target slice to be used. In the UE Configuration Update Command, the AMF may provide back off timer for the source S-NSSAI and/or includes the original slice in the Rejected NSSAI so that the UE cannot request a PDU Session according to the existing URSP rule until the back off timer is expired.

Editor's note:	It is FFS if additional information is to be included in the new IE to perform PDU Session Transfer.

-	In step 2 the UE acknowledges the reception of the Configuration Update Command.

The remaining steps are based on existing procedures per clause 4.3.5.2 of TS 23.502 [5].

Figure 6.32.2.1-1: PDU Session Transfer - UE Initiated Procedure

6.32.2.2	PDU Session Transfer from Source Slice to Target Slice - Network Initiated Approach

6.32.2.2.1	AMF Initiated Approach

In this solution, the Slices in Allowed NSSAI are not changed but one of the slices for example is temporarily taken out of service and should not be used. The AMF, per operator's policy or by learning from other NFs that a slice shouldn't be used instructs the SMF used for the source network slice (i.e. network slice to not be used), to initiate a PDU Session Transfer from the source network slice to the target network slice (new network slice to be used). The Nsmf_PDUSession_UpdateSMContext service is extended to support such a capability e.g. the AMF indicates a Target network slice to the SMF. The SMF in turn sends a PDU Session Modification Request to the UE to initiate a PDU Session transfer. The PDU Modification Session Request can be extended to support this new capability (e.g. adding target slice and optionally source slice that otherwise could be derived from existing PDU Session, either in PCO together with existing "PDU Session Address Lifetime value" or in a new IE. The UE uses existing procedures to establish the PDU session using the new target slice and usually the same DNN and releases the old PDU sessions used with the source network slice.

The call flow in Figure 6.32.2.2-1 illustrates the procedure depicted above. The following is a brief description of the steps in the call flow.

The assumption in the call flow is that a UE has some PDU Sessions already established on a network slice and these PDU sessions have to be transferred to another network slice.

-	In step 1, the AMF initiates a Nsmf_PDUSession_UpdateSMContext Request to initiate a PDU session transfer from a source slice to a target slice. The Request is extended to support this new capability e.g. the Target network slice to the SMF and possible an indication that a transfer is to be initiated.  The Request is now extended with the following optional information so that the SMF can inform the UE to perform a PDU Session transfer from source slice to a target slice:

-	Source network Slice,

-	Target network Slice,

-	Action: Perform Session transfer for all PDU sessions immediately or at a scheduled tie time applicable to the UE location.

-	In step 2, the SMF initiates action to notify the UE about a PDU session transfer from a source slice to a target slice as indicated above is extending the PDU Session Modification Request sent to the UE to include the needed information.

-	In step 3, SMF returns Nsmf_PDUSession_UpdateSMContext response to the AMF.

-	In step 4, the SMF invokes the Namf_Communication_N1N2MessageTransfer to include an SM container towards the UE with a PDU Session Modification Command extended (e.g. with target S-NSSAI) to inform the UE of a PDU Session Transfer.

-	In step 5 the AMF forwards the SM Container content to the UE.

The AMF may include back off timer for the original slice and/or includes the original slice in the Rejected NSSAI to prevent the UE requesting the slice due to URSP rule re-evaluation.

The remaining steps are based on existing procedures per clause 4.3.5.2 of TS 23.502 [5].

The call flow does not show the aspect of UE registering to request the target slice to be used if this step is required, when the target slice is not in the Allowed slices.

Figure 6.32.2.2.1-1: AMF initiated PDU Session Transfer - Session Management Procedure

Editor's note:	the impact of overriding of the SSC mode need be evaluated.

6.32.2.2.2	PCF Initiated Approach

In this solution the PCF initiates the PDU session transfer. The flow is the same as the one described in clause 6.32.2.2.1 with the exception of Step 1. In this solution the PCF sends a Npcf_SMPolicyControl_UpdateNotify to initiate a PDU session transfer from a source slice to a target slice. The update notify is extended to indicate the SMF, the Target network slice and indication that a PDU transfer is to be initiated. The Update Notify message sent from the PCF is extended with the following information to enable the SMF to request the UE to perform a PDU Session transfer from source slice to a target slice:

-	Source network Slice;

-	Target network Slice;

-	Action: Perform Session transfer for all PDU sessions immediately or at a scheduled time applicable to the UE location.

Figure 6.32.2.2.2-1: PCF Initiated PDU Session Transfer - Session Management Procedure

6.32.2.3	SSC Mode and PDU Session Transfer from Source Network Slice to Target Network Slice

It is desirable that SSC mode 3 is employed for performing PDU session transfer from a source network slice to a target network slice. This is critically important for emergency PDU sessions and PDU sessions used for MC applications and which cannot be disconnected before completion or otherwise have to be handled with SSC mode 3. It is thus possible for either of the above solutions that the UE may receive the SSC mode 3 to be used during the PDU session transfer. This implies that the UE overrides the SSC mode currently used for the PDU session to be transferred.

More specifically, in the UE initiated approach, the target NSSAI in the UE Configuration Update Command can indicate that the UE can apply SSC mode 3 logic for the PDU session transfer.

For the network initiated approach, the PDU Modification Session Request can be extended to include the SSC mode 3 logic to be used for PDU session transfer.

6.32.3	Impacts on Existing Nodes and Functionality

AMF:

-	provide back off timer for the source slice so that the UE cannot request a PDU Session according to the existing URSP rule until the back off timer is expired.

PCF:

-	Support S-NSSAI change determination.

-	In case of separated PCFs (i.e. SM PCF and UE PCF), an extension of N43 to obtain UE information (e.g. allowed NSSAI).

-	Support Npcf_SMPolicyControl_UpdateNotify extension.

*** END OF CHANGES ***