3GPP TSG-SA WG2#153E	S2-2208484r01

Elbonia, 10 – 17 October 2022	(was S2-22xxxxx)

Source:	CATT, Xiaomi

Title:	KI#3: Overall Evaluation and Conclusion

Document for:	Discussion/Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract of the contribution: This contribution proposes overall evaluation and conclusion on KI#3.

- Introduction

This paper proposes overall evaluation and conclusion on KI#3.

- Proposal

It is proposed to capture the following texts into TR 23.700-86.

>>>>First Change<<<<

7.3	Key Issue #3: Ranging/Sidelink Positioning device discovery

For Key Issue #3 "Ranging/Sidelink Positioning device discovery", the Solution#4, #5, #14, #18, #26, #27 and #28 can be summarized and evaluated as the following:

-	Sol#4 proposes the Model A and Model B Direct Discovery as defined in TS 23.304 [4] are reused as basis for Ranging/Sidelink Positioning devices discovery. For Model A discovery, it is proposed Target UE sends an Announcement message containing the Ranging/Sidelink Positioning related parameters and Reference UEs monitor the Announcement message. For Model B discovery, it is proposed Reference UE sends a Solicitation message containing Ranging/Sidelink Positioning related parameters and Target UE responds to the Reference UE with a Response message containing Ranging/Sidelink Positioning related parameters. Discovery mechanism for V2X usage is not mentioned.

-	Sol#5 proposes the Reference UE or Target UE broadcasts a Ranging/Sidelink Positioning Request message that contains the parameters provided by application layer, and the Reference UE/Target UE responds if the Application Layer ID is matched.

-	Sol#14 introduces a new reference point SR5 that is used to control the Sidelink Positioning and Ranging operation over the PC5 reference point between the UEs. Device and Service Discovery Function (DSDF) over the V2X/5G ProSe layer is introduced to discover and identify the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service sessions. The discovery is carried out over SR5 over PC5 reference point between the UEs using PC5 communication service provided by ProSe or V2X layer.

-	Sol#18 focuses on the network triggered ranging device discovery. It proposes one UE sends the ranging device discovery request to the network, then the network notifies the peer UE to turn on or activate the ranging device discovery. After receiving the notification from the network, the peer UE initiates the ranging device discovery with the UE. This solution includes two alternatives, i.e., 5GC based network triggered ranging device discovery and Application layer-based Network triggered ranging device discovery. The former is performed over the Control plane assuming same LMF is selected for the 2 UEs and the 2 UEs are from same PLMN, and the latter is performed over the application layer which is out of SA2 scope. This solution allows the Ranging/SL Positioning UEs to switch off the discovery functionality, e.g. for the purpose of power saving, and switch on based on the network instruction, applying only to the In Coverage case to trigger and receive network instruction.

-	Sol#26 focuses on the functional split and positioning protocol to enable the support of Sidelink (SL) positioning (and Ranging), implying that Ranging/Sidelink Positioning device discovery is part of RSPP protocol.

-	Sol#27 focuses on LMF selection for Ranging/Sidelink positioning by the AMF. It is not clear how Ranging/Sidelink Positioning device discovery is performed in this solution.

-	Sol#28 proposes a solution that uses LMF for Ranging/Sidelink positioning. It is not clear how Ranging/Sidelink Positioning device discovery is performed in this solution.

>>>>Second Change<<<<

8.3	Key Issue #3: Ranging/Sidelink Positioning device discovery

For Key Issue #3 "Ranging/Sidelink Positioning device discovery", the followings are taken as conclusions:

-	Both Model A and Model B Direct Discovery as defined in TS 23.304 [4] are reused for the commercial and public safety use cases and procedures for V2X communication over PC5 reference point as defined in TS 23.287 [3] are reused for the V2X use cases for Ranging/Sidelink Positioning device discovery with the following enhancements:

-	Service Code indicates “Ranging/Sidelink Positioning”.

-	The expected roles of the UEs (i.e. Target UE, SL Reference UE, Located UE, Assistant UE, SL Positioning Server UE) in discovery need to be matched for a specific service.

NOTE 1:	The parameters used in Ranging/Sidelink Positioning device discovery procedures can be decided in normative phase.

NOTE 2:  The Ranging/Sidelink Positioning device discovery procedures and the parameters used in the procedures should be aligned with RAN WGs.

>>>>End Changes<<<<