SA WG2 Meeting #153e	S2-2208533r06

10 – 17 October 2022, e-Meeting	(revision of S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	KI#1: Conclusions

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: Proposes conclusions for KI #1.

# 1. Introduction

Conclusions for KI #1 in TR 23.700-71 for KI #1 are provided here.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-71.

*** First Change (all new) ***

# 8.1	Key Issue #1: Architectural Enhancement to support User Plane positioning

# -	A user plane connection may be used between a UE and LMF with the following properties.

- The UE uses the URSP which includes user plane positioning related PDU session parameters (e.g., DNN and S-NSSAI) to establish the PDU session for user plane positioning.

- It is LMF to decide whether to use user plane or control plane positioning mode when receiving positioning requests from AMF.

- If LMF decides to utilize user plane positioning, the LMF sends its UP positioning address and security related information to UE to trigger the UP connection if it is not available.

- LMF and UE may maintain the established user plane connection between UE and LMF for subsequent LCS session.

-	The user plane connections can be supported using other transport decided by stage 3 and common security procedures and protocol to be determined by SA3.

-	As per both TS 38.305 from RAN and TS 23.271 for GERAN/UTRAN/E-UTRAN, the continuous User Plane work aligned with both RAN and 2G/3G/4G may use solution#2.

-	The UP connection between UE and LMF supports LPP message transfer and Supplementary service.

-

*** End of Changes ***