3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208566r01

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion for Exposure

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: A Conclusion for exposure is added.

# 1. Introduction

This contribution proposes a conclusion for KI#2 considering the conclusion of KI#3 in FS_XRM.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48.

# * * * * First change * * * *

## 8.2	Conclusion for KI#2: Fast and efficient network exposure improvements

The following bullet points summarize the principles for the way forward of this key issue:

-	5G System may support API based exposure of congestion level information towards AF as following:

-	QNC for GBR QoS Flow: data rate cannot be guaranteed; If NG-RAN has received a list of Alternative QoS Profiles as described in clause 5.7.2.4 of TS 23.501[2], it also notifies the Alternative QoS Profile(s) according to the same logic described in clause 5.7.3.4.1b of TS 23.501[2] but via User Plane to local UPF.

-	Congestion level: a value denoting the degree of current congestion. A lower value means less congestion is experienced.

-  AF uses Nnef_AFSessionWithQoS to subscribe the above exposure to NEF/PCF, same as local exposure mechanism defined in TS 23.548[3].

-  Exposure path of Network Exposure defined in clause 6.4 of TS 23.548[3] is reused with extensions of GTP-U header and UPF/L-NEF services to expose the above information.

# * * * * End of changes * * * *