3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208568r05

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, China Mobile, KDDI, Lenovo

Title:	KI#4&5: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Evaluation and conclusion for KI#4&5 is proposed.

# 1. Introduction

This paper proposes the evaluation and conclusion for KI#4&5 together.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change (all new texts) * * * *

## 7.X Evaluation for KI#4&5

### 7.X.1 General

32 solutions proposed to address the KI#4&5, i.e., PDU Set identification and PDU Set based handling. The main idea of these solutions can be summarized as following.

1.	AF provisions necessary assistance information to 5GS.

2.	Based on the assistance information, UPF identifies the PDU Set and also the PDU Set info (e.g. importance/dependency) accordingly.

3.	The identified PDU Set info is further sent to RAN via the GTP-U header of the DL packets.

4.	Based on the PDU Set info, RAN performs the PDU Set based packet handling.

Accordingly, the solutions can be further elaborated as the following table.

| Solutions | Parameters provisioned by AF | Identification of PDU Set | PS level QoS Parameters | Parameters forwarded to RAN via GTP-U | PDU Set importance to RAN |
| --- | --- | --- | --- | --- | --- |
| Sol#7 | - Media packet filtering - Media packet classification | - Meta-data extension headers in QUIC - RTP/NALU header - IP header - TCP/UDP option |  | - PDU Set SN (start/end) - PDU Set priority as PDU Priority Mark (PPM) | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#8 | - All PDU Set Required indication. - PDU Set presence indication - PDU Set level QoS - N6 tunnel info. | - N6 extension header via MASQUE - 3GPP specific RTP extension header | - PSDB - PSER | - PDU Set SN - Size of PDU Set (bytes) - Late PDU Set delivery indication - Burst size. |  |
| Sol#9 |  | - Server co-located with UPF. | - PSDB | - PDU Set SN - Number of PDUs in a PDU Set |  |
| Sol#10 | - Importance level - Flow description |  |  |  | - Multiple QoS Flows are used for different PDU Set importance levels. |
| Sol#11 | - Importance/dependency among flows. - Flow description |  |  | - PDU Set SN - PDU Set importance - PDU Set boundary - PDU SN within a PDU Set - PDU Set dependency | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#12 | - PDU Set level QoS requirements - Flow description | - RTP/SRTP header or header extension; - NALU header - Traffic characteristics | - PSDB - PSER - Max PDU Set Loss Rate/Number | - PDU Set boundary - PDU Set SN |  |
| Sol#13 | - Importance for flows - Flow descriptions allowed to be dropped and/or requiring prioritized transmission. |  |  |  |  |
| Sol#14 | - Flow description - Media characteristics - PDU Set level handling requirements |  | - PDU Set Validity time |  | - Multiple QoS Flows are used for different PDU Set importance levels. - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#15 | - PDU Set handling indication | - RTP/SRTP header (extension) - NALU header |  | - PDU Set SN - PDU SN within a PDU Set - importance | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#16 | - Flow description with enhanced SDF adding support of RTP/SRTP header/payload and RTCP/SRTCP header | - RTP/SRTP header |  | - PDU Set boundary |  |
| Sol#17 | - XRM stream info including RTP/SRTP header/payload description. | - RTP/SRTP header/payload - Traffic characteristics |  |  | - Sub-QoS flows are used for different PDU Set importance levels. |
| Sol#18 | - XRM stream info | - RTP/SRTP header/payload - Traffic characteristics - IP packet filter |  | - Start/end of PDU Set - Importance (XQFI) - dependency | - Sub-QoS flows are used for different PDU Set importance levels |
| Sol#19 |  | - RTP/SRTP header |  | - PDU Set SN - Importance - dependency | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#20 |  | - Metadata added by AS as a proprietary way. |  | - Last PDU indication - PDU Set SN - PDU SN within a PDU Set - PDU Set size - PDU Set Content Criteria and PDU Set Content Ratio |  |
| Sol#21 | - New SDF info |  |  | - PDU Set SN - Last PDU indication |  |
| Sol#22 |  | - N6 tunnel between UPF and AS, like GTP-U. | - PSER | - XR stream ID - XRMD ID - PDU handling type - PDU Set SN - PDU SN within a PDU Set - Last PDU flag - PDU Set priority - Reference PDU Set SN | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#23 | - Partial PDU Set handling request - Packet loss threshold |  |  | - PDU Set SN - PDU SN within a PDU Set - Dependency |  |
| Sol#24 | - Flow description - Dependency info - Traffic pattern - Media protocol info | - RTP/SRTP header/payload - Traffic characteristics - Fixed GOP structure |  |  | - Multiple QoS Flows are used for different PDU Set importance levels. |
| Sol#25 | - PDU Set level QoS - PDU Set identification |  | - PDU Set 5QI - PSDB - Nominal PSDB - PSER - PDU Set maximum size - PSDT - PDU Set Content Criteria and PDU Set Content Ratio |  | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#26 | - Flow description - Associated XR traffic configuration info (PDU Set level QoS requirements, frame type/importance, PDU Set handling indication, burst arrival time & periodicity) | - IP packet filters - RTP/SRTP payload/header | - PSER - PSDB |  | - Multiple QoS flows are used for different PDU Set importance levels. |
| Sol#49 | - Indication of PDU Set integrated handling - Flow description - Integrity checking timer |  |  |  |  |
| Sol#50 |  |  | - Relative QoS | - PDU Set Size - PDU Set SN |  |
| Sol#51 | - Indication of all PDUs needed. |  |  |  |  |
| Sol#52(consolidated solution) | - PDU Set handling indication - Whether all PDUs needed. - PSDB - PSER - Burst periodicity | - RTP/SRTP header/payload - 3GPP specific RTP header extension - N6 tunnel header - UPF implementation | - PSDB - PSER - Indication of all PDUs needed - Late PDU Set delivery indication - PDU Set priority | - PDU Set SN - Start/end of the PS - PDU SN within a PDU Set - Number of PDUs within a PDU Set or PDU Set size - PDU Set importance - PDU Set dependency | - A single QoS Flow is used for different PDU Set importance level. - Multiple QoS Flows are used for different PDU Set importance levels. - Sub-QoS flows are used for different PDU Set importance levels. |
| Sol#53 |  |  |  | - PDU Set SN |  |
| Sol#54 | - Guidance info about the priority/importance of the packet | - RTP header/payload |  | - Priority/importance |  |
| Sol#55 |  |  |  | - PDU Set SN - Start/end PDU - PDU SN within a PDU Set - Number of PDUs within a PDU Set - PDU Set importance - PDU Set dependency |  |
| Sol#56 | - Flow description - RTP/SRTP header type - Assistance info (number of temporal/spatial layers, PDU Set periodicity, PDU Set dependency) |  | - PDU Set Validity Time | - PDU Set type - PDU Set SN - PDU Set dependency - Start/end of a PDU Set | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#57 | - Indication of event for PDU set and/or burst marking settings |  |  |  |  |
| Sol#68 | - PDU Set related handling info (PDU Set identification, PDU Set handling requirements for different PDU Set types) |  |  | - PDU Set type - PDU Set SN - PDU SN within a PDU Set | - A single QoS Flow is used for different PDU Set importance levels. |
| Sol#69 |  |  |  |  |  |
| Sol#70 |  |  | - FEC PDU Set | - PDU Set payload size |  |

### 7.X.1 AF Information Provisioning

Before PDU Set granularity handling, the AF may provision assistance information to 5GS, including:

-	PDU Set handling indication (solution 8, 15,19, 25, 26, 49) indicates if PDU Set based handling should be activated for a service data flow.

NOTE:	This indication may be implicitly indicated via other PDU Set related information provided by the AF.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25, 49, 51). There may be different cases, i.e., all PDUs within a PDU Set are needed, only PDUs up to the first PDU in error are useful to the decoder, and a PDU Set with partial PDU losses is still useful.

-	Burst periodicity (solution 8, 12, 14, 24, 26, 56) indicates periodicity of the service data flow, which may be used for identification of the PDU Sets or power saving.

-	The PDU Set granularity QoS requirements:

-	PDU Set Delay Budget (solution 8, 12, 25, 26).

-	PDU Set Error Rate (solution 8, 12, 22, 25, 26).

### 7.X.2 Identification of PDU Set and PDU Set information

#### 7.X.2.1 PDU Set information

The following PDU Set information are proposed in different solutions:

-	PDU Set Sequence number (SN) (solution 7, 8, 9, 11, 12, 14, 19, 20, 21, 22, 50, 53, 55, 56, 68) allows the 5GS to distinguish the different PDUs for corresponding PDU Sets.

-	Start/End PDU of the PDU Set (solution 11, 12, 15, 18, 21, 22, 55, 56) allows 5GS to identify the boundary of a PDU Set.

-	PDU SN within a PDU Set (solution 11, 20, 22, 55, 56, 68) allows 5GS to distinguish the PDUs within a PDU Set. Besides, when only up to the first bit in error of a PDU Set is useful to the receiver, the PDU SN is used to identify the useful PDUs within a PDU Set.

-	Number of PDUs within a PDU Set (solution 9, 20, 50) and/or PDU Set size in bytes allows 5GS to identify size of PDU Set for PDU Set integrated scheduling.

-	PDU Set importance (solution 7, 11, 14, 15, 18, 19, 22, 24, 54) allows 5GS to identify the importance of a PDU Set compared to other PDU Sets.

-	PDU Set dependency (solution 11, 14, 19, 22, 23, 24, 56) allows 5GS to identify the dependency among different PDU Sets.

#### 7.X.2.2 PDU Set Information identification on UPF and supported N6 protocol

The following methods are proposed to support PDU Set identification on UPF based on assumptions that different N6 protocols are used:

1.	Matching RTP/SRTP header and payload (RFC 3550/6184/7798/draft-ietf-avtcore-rtp-vvc/ draft-ietf-avtext-framemarking) as proposed in Sol# 7, 12, 14, 15, 16, 17, 18, 19, 24, 26 and 54.

2.	Defining/extending the current N6 protocols to carry related info from AS:

2.1	defining new 3GPP specific RTP extension header as proposed in Sol#8 and #58.

2.2	extending GTP-U protocol between AS and UPF as proposed in Sol#22.

2.3	extending HTTP header based on MASQUE tunneling between AS and UPF as proposed in Sol#8 and #58.

2.4 extend transport header (UDP, QUIC) as proposed in in Sol#7.

3.	By non-standardized UPF implementation mechanism, e.g., detection based on traffic characteristics, as proposed by Sol#7, 12, 17, 18, 20, 24 and 25. IP header fields (DSCP/TOS, IP port, IPv6 flow label) in proposed solutions 7, 17, 18, 55) may identify PDU sets however further details are non-standard.

Opiton#1 has less requirements on the application server by reusing existing IETF RFCs/drafts. As clarified in LS S4-221174, for real-time applications that require e2e latencies lower than 200ms, RTP-based protocols are recommended.

Option#2.1 depends on the further confirmation from SA4 WG and whether this can be completed within Rel-18 timeline depends on SA4 progress.

Option#2.2 requests the AS to support and establish the GTP-U connection via the pre-configured TEID info while the GTP-U protocol is defined and mainly used within 3GPP.

Option#2.3 requests UPF to support HTTP MASQUE proxy and dynamically establish the tunnelling connection between UPF and AS. HTTP header needs to be extended to further support carrying the PDU Set information by 3GPP.

Option#3 is up to the UPF implementation without need of standardization.

Further, Solution #55 allows PDU Set Descriptors to be exchanged between a PDU Set Handling Service in the UPF and a PDU Set Handling Service in the UE. PDU Set Descriptor is sent in advance of sending the PDUs. Solution #55 also proposed to support peer-to-peer communication in a way that the PDU Set Handling Service in the UPF can be used to receive UL packets and PDU Set Descriptors from a UE and send downlink packets and PDU Set Descriptors to a peer UE.

### 7.X.3 RAN awareness of PDU Set information

The PDU Set information listed in clause 7.X.2.1 is transferred to RAN via GTP-U header in most solutions except for PDU Set importance. UPF provides PDU Set importance via the following different ways in different solutions:

Option 1: UPF classifies the DL traffics into different QoS Flows based on PDU Set importance (solution 10, 14, 24, 26).

Option 2: UPF classifies the DL traffics into different sub-QoS Flows based on PDU Set importance (solution 17, 18).

Option 3: UPF adds PDU Set importance into GTP-U header (solution 7, 11, 14, 15, 19, 22, 25, 54, 56, 68).

For option 1, the PDU Sets with identified importance levels are further mapped into different QoS Flows with corresponding QoS parameters (e.g. PER/PSER) by UPF. Based on the different QoS parameters, NG-RAN can handle the PDU Set importance-based scheduling via existing capabilities, which can significantly decrease the complexity of NG-RAN. This also follows the current 5GS QoS framework for prioritized packet handling.

For option 2 and 3, the QoS Flow is further refined into sub QoS-Flows or packets/PDU Sets with different importance labels in GTP-U header. These impact current RAN scheduling by requesting RAN to support different QoS handling within a single QoS Flow.

It shall be noted though that no benefits from using importance information have been shown.

### 7.X.4 PDU Set Importance/Dependency Handled by UPF

In Solution 13, 22 and 53, UPF can also optimize the PDU Set based packet handling based on the identified PDU Set Importance/Dependency information as follows:

-	In Sol#13, the RAN reports the RAN load level to UPF and UPF blocks or buffers data based on the network load associated FAR.

-	In Sol#19, in case of a PDU Set transmission failure, the UPF may drop another later PDU Set(s) which is correlated to the failed PDU Set and its importance level is the same or less.

-	In Sol#22, in presence of network congestion, the UPF can rely on the PPPS to determine which PDU Sets should be dropped preferably.

-	In Sol#53, in case of PDU Set transmission failure, RAN reports the status to UPF and UPF determines whether to retransmit the PDU Set based on the importance/dependency information, or drop other PDU Sets which depends on the dropped one would also be dropped together.

Additionally, in Solution#23 and #49, UPF can also drop the packets for PDU set integrated handling as follows:

- In Sol#23, UPF may drop the packets of the PDU set if the lost packets within the PDU set are above the threshold.

- In Sol#49, if not all PDUs of one PDU set are received, UPF may stop delivering the PDU Set and inform the downstream node (either UPF or RAN node) to drop the PDU set to save transmission resources.

### 7.X.5 PDU Set QoS Parameters

New QoS parameters for PDU Set based QoS handling in 5GS are proposed as following:

-	PDU Set Delay Budget (PSDB) (solution 8, 9, 12, 14, 25, 26). The PDU Set Delay Budget defines an upper bound of the time that a PDU Set may be delayed between UE and the N6 termination point at the UPF.

-	PDU Set Error Rate (PSER) (solution 8, 12, 22, 25, 26). The PDU Set Error Rate applies to PDU set which needs PDU set content integrated handling.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25, 49, 51,70). There may be different cases, i.e., all PDUs within a PDU Set are needed, only PDUs up to the first PDU in error are useful to the decoder, and a PDU Set with partial PDU losses is still useful. Specifically, solution #70 allows RAN to discard FEC data when RAN has acknowledgement that the PDUs in the PDU Set, excluding the FEC data, is successfully delivered to the UE. Hence, in case of high load, RAN can avoid transmitting the overhead induced by FEC while in case of low load that overhead will be transmitted (potentially increasing the chance of a congestion).

-	Whether to drop a PDU Set in case PSDB is exceeded (solution 8, 22, 24, 25). Due to the reference relationship among different PDU Sets, a PDU Set may still be useful for the decoding of subsequent PDU Sets, even its PSDB already expires.

-	PDU Set Discard Timer (PSDT) (solution 25). The PSDT is used to indicate the accurate upper bound of time to keep transmitting the PDU Set even its PSDB already expires, i.e., how long time a PDU Set will be considered as useful by the applications.

# * * * * Second change * * * *

## 8.X Conclusions for KI#4 and KI#5

The following aspects are concluded as principles for the normative work to support the following two key issues:

-	Key Issue #4: PDU Set integrated packet handling

-	Key Issue #5: Differentiated PDU Set Handling

NOTE:	Whether and how to support the PDU Set handling for Uplink will be studied and led by RAN WG. SA2 will align with RAN’s progress and decision for Uplink, if any.

### 8.X.1	Control plane enhancements for supporting PDU Set in downlink

#### 8.x.1.1	PDU Set QoS Parameters

PDU Set QoS treatment is determined using dynamic or non-dynamic PCC.

The following PDU Set QoS parameters are defined to support PDU Set handling:

-  PDU Set Error Rate: The PSER defines an upper bound for the ratio between the number of PDU Sets not successfully received and the total number of PDU Sets sent towards a recipient measured over a measurement window. A PDU Set is considered as error in case all or partial PDUs of the PDU Set are not successfully delivered, based on different content requirement, e.g. all PDUs are needed for the usage of PDU Set, the first packet is needed for the usage of PDU Set.

NOTE: Whether the following Default Content Criterion (DCC) and Default Content Ratio (DCR) are considered for PSER is pending to SA4 reply. The DCC represents the criterion for the QoS Flow based on which a recipient can determine whether the PDU Sets of a QoS flow can be considered successfully delivered or not. DCC includes one of the following:; PDU Set successfully received if X% of its bits have been successfully received (in that case the Default Content Ratio (DCR) will be set to X%); PDU Set partially received up to the first error bit.

Editor’s Note: Whether a “PDU Set Valid Time” is needed is FFS.

-  PDU Set Delay Budget: The PSDB defines an upper bound for the time that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. time between reception of the first PDU and the successful delivery of the last PDU of a PDU Set. PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL.

NOTE :	The PSDB definition assumes a limited time difference between the arrival at the UPF of first and of the last PDU of the PDU set. Such assumptions need to be captured in the normative phase.

-  Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication).

-  Whether a PDU Set is still valid in case PSDB is exceeded (PDU Set Valid Indication)

Editor’s Note: Whether a “PDU Set Valid Time” is needed is FFS.

If PDU Set based QoS handling is used, PCF determines the above PDU Set QoS Parameters based on information provided by AF (described in 8.X.2) and/or local configuration. The PDU Set QoS parameters are sent to SMF as part of PCC rule, then SMF sends them to RAN.

Editor’s Note: Whether a “PDU Set Valid Time” is needed is FFS.

#### 8.X.1.2	AF Information Provisioning

PDU Set related assistance information provisioning by AF is supported for dynamic PCC. AF may provision one or more of the following PDU Set related assistance information to NEF/PCF during AF QoS request procedure:

-

-  PDU Set QoS parameters listed in clause 8.x.1.1.

-  Traffic periodicity

Editor’s Note: AF may don’t understand the concept of PDU Set. Whether application understandable terms should be used instead is FFS. Proposal in S2-2208822 is to use “application layer information unit” to replace “PDU Set”

### 8.X.2	User plane enhancements for supporting PDU Set in downlink

#### 8.X.2.1 PDU Set Information

The following PDU Set related information may be identified by UPF to support PDU Set based handling:

-  PDU Set SN: Conveys the start and end of a PDU set by the change in the sequence number and inherently handles out-of-order packets by marking the right sequence number.

-  Optional, Start/End PDU of the PDU Set

-  PDU SN within a PDU Set

-  Optional, Number of PDUs within a PDU Set

-  PDU Set Size in Bytes

Editor’s Note: Support of PDU Set dependency (i.e. dependency information between frames/slices/layers) are FFS. (Potential SoH)

#### 8.X.2.2 PDU Set Information identification on UPF and supported N6 protocols

PSA UPF may identify the PDU Set based on instruction from SMF and packet header of N6 protocols:

-  by matching RTP/SRTP header and payload (RFC 3550/3711/6184/7798/draft-ietf-avtcore-rtp-vvc/draft-ietf-avtext-framemarking are supported).

-	by matching new RTP header to be defined in SA4 5G_RTP WI.

NOTE: The support of new RTP header defined in SA4 is TBD during normative phase based on progress of SA4.

NOTE: In above cases, it is assumed that the RTP/SRTP header and/or payload necessary for the identification of PDU Set Information is not encrypted.

-  by UPF implementation, e.g., PDU Set detection based on traffic characteristics. IP header parameters DSCP/TOS, IP port, IPv6 flowlabel may be used to detect PDU set, however detailed mechanisms in UPF for PDU Set information identification will not be standardized.

- By using MASQUE approach with HTTP header extension

Editor’s Note: Other N6 protocols, e.g., GTP-U, IP/TCP/UDP/QUIC options, carrying PDU Set information are FFS. (Potential SoH)

#### 8.X.2.3 Delivering PDU Set Information to RAN

PDU Set Information (listed in 8.X.2.1), except for “PDU Set Importance”, are informed by UPF to RAN via GTP-U header of user plane packet.

UPF (1) classifies the DL traffics into different QoS Flows based on SMF instruction.

Editor’s Note: Whether PDU Set importance is used for mapping different QoS Flows, sub-QoS Flows, or included in GTP-U header is FFS. (Potential SoH)

### 8.X.3 PDU Set based QoS handling

RAN performs PDU Set based QoS handling based on received PDU Set QoS Parameters via control plane, and PDU Set Information via user plane. The details of RAN behaviours are defined in RAN WG.

# * * * * End of changes * * * *