3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208579r01

Elbonia, October 10 – 14, 2022

Source:	China Mobile

Title:	KI#5: conclusion for support of edge relocation based on GSMA OPG impacts and improvements

Document for:	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2

Abstract: update conclusion for KI #5.

# 1. Discussion

# 2. Text Proposal

It is proposed to capture the following aspects in TR 23.700-48.

# * * * * First change * * * *

## 8.5	Interim conclusion for KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

# Edge Relocation

For edge relocation in GSMA OPG scenario, the EAS reselection methods should be the same as the initial selection.

Edge relocation within the same hosting PLMN’s EHE is supported, with the below principles:

- 1) SMF initiates AF/EAS relocation and selection of the target EHE e.g. due to UE mobility.

- 2) SMF determines AF in the target EHE domain corresponding to the UE target location based on service area or EHE domain supported by AF

- 3) Notification of User plane management event procedure is enhanced to support AF/EAS change (Sol #23)

EAS relocation between different PLMN’s EHE is depend on GSMA’s feedback. Depending on the feedback, for different PLMNs case:

1)	The AF initiates EAS relocation (e.g. due to AF internal triggers) and sends PLMN ID corresponding to target EAS to the network for indicating the EAS should be relocated to the target PLMN (identified by PLMN ID).

2)	The PCF creates the PCC rule including PLMN ID (if PCF authorizes the traffic routing in the target PLMN is allowed).

3)	The SMF determines the target DNAI for accessing target PLMN.

# * * * * End of changes * * * *