3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208612

Elbonia, Oct 10th – 17th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Conclusion for FS_5TRS_URLLC KI#6

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: Providing Conclusion for KI#6.

1. Introduction/Discussion

There are 2 ENs in the conclusion for FS_5TRS_URLLC KI#6.

For the first EN, we need to wait for responses from RAN WGs.

For the second EN, we may use the periodicity for better QoS.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-25.

# * * * * First change * * * *

8.4	Key issue #6: Adapting downstream scheduling based on RAN feedback for low latency communication

The following bullet points summarize the principles for the way forward:

-	Proactive feedback requires that 5GS and the AF receive time information from the same master clock. Since this assumption cannot hold in all deployment, both pro-active and reactive feedback mode shall be supported.

-	The AF provides adaptation capability information of the application to 5GS.

Principles for Proactive feedback:

-	The AF may indicate a capability for BAT adaptation in the AF request, along with the BAT as specified in Rel-17.

-	If the AF indicates its capability for BAT adaptation ora BAT window and the PCF receives a policy authorization request from the AF/NEF/TSCTSF that indicates that capability or a BAT window:

-	the PCF sets a trigger to be notified for the "BAT offset" event for the corresponding PCC Rule via the SM policy control service to the SMF.

-	If the SMF receives an indication for a BAT adaptation capability or a BAT window, in a TSCAC, the SMF includes that indication or a BAT window into TSCAI along with the QoS Flow establishment request. This indicates to the NG-RAN that the NG-RAN may provide a BAT offset in an N2 SM information as a response to the SMF.

-	As a response to the QoS Flow establishment request, the NG-RAN may provide a "BAT offset" that is within the BAT window, if available value. The BAT offset is provided from NG-RAN to SMF, eventually forwarded via PCF/TSCTSF/NEF to AF.

-	If the AF does not receive the BAT offset (e.g. NG-RAN did not provide it), the AF assumes that the BAT value is used as a Burst Arrival Time in 5GS.

-	The SMF configures the UPF for clock drifting reports as specified in TS 23.502 [3]. In a case the SMF receives a clock drifting report from UPF, the SMF adjusts the BAT offset based on the existing procedures in TS 23.502 [3] and provides the updated BAT offset to the AF via PCF/TSCTSF/NEF.

-	The AF may also indicate its capability for periodicity adaptation or a Periodicity Range in the AF Request together with the parameter for BAT adaption mentioned above.. .

-	The RAN may provide a periodicity feedback together with a BAT offset mentioned above. The periodicity feedback shall be within the Periodicity Range (if available). If RAN provides feedback with only one periodicity, the assumed BAT is accepted based on proposed periodicity. If RAN provides feedback only for BAT, the assumed periodicity is accepted and the signaling continues as described already.

- 	If AF can accept the changed periodicity and changed BAT, it applies those to the traffic. If AF can only accept the changed periodicity but cannot support proposed BAT, then it provides the BAT (or BAT window) and the signaling continues as described already.

-  If the AF accept the periodicity and BAT, it needs to send a confirmation to the 5GS in order to notify SMF/CUC the periodicity or BAT which shall be used in integration with TSN transport network as described in clause 8.3.

Editor's note:	The above industrial applications behaviours regarding periodicity need to be confirmed by SA1.

Principles for Reactive feedback:

-	The AF may request the 5GS to report the BAT offset along with a certain threshold for reporting BAT offset; that is a time offset to the observed timing of the packet reception in the user plane in the NG-RAN. In this case the AF subscribes for the QoS notifications as described in the QoS notification control procedure in TS 23.501 and includes an indication of "burst arrival time adaptation" in the QoS-request to the 5GC.

-	If the PCF receives indication for " burst arrival time adaptation" along a subscription for QoS notifications in policy authorization request from AF/NEF/TSCTSF, the PCF sets the QoS notification control parameter as described in TS 23.501 and in addition sets a trigger to be notified for the "BAT offset" event for the corresponding PCC Rule via the SM policy control service to the SMF. The SMF provides the notification control parameter to the NG-RAN as described in TS 23.501, and in addition includes the indication of " burst arrival time adaptation" to the QoS profile.

-	If the Notification control is enabled and indication of " burst arrival time adaptation" is set in the QoS profile, and the NG-RAN determines that the GFBR can no longer be guaranteed for a QoS Flow, the NG-RAN notifies the SMF as described in TS 23.501 and in addition may include a BAT offset to the N2 SM information that is sent to SMF, eventually forwarded via PCF/TSCTSF/NEF to AF.

-	If the NG-RAN receives the indication for "burst arrival time adaptation", the NG-RAN indicates the parameter to the UE via RRC signalling. The NG-RAN also indicates a threshold for the BAT offset reports to the UE.

-	If the UE receives the indication for "burst arrival time adaptation" from NG-RAN, the UE determines a relative BAT offset value in reference to the current Burst Arrival Time experienced by UE (i.e. in reference to when UE currently receives bursts) and the scheduling UL time slot at UE (e.g. in Configured Grants, as defined in TS 38.321 [11]). The UE sends the BAT offset to RAN when the time offset value reaches the configured threshold, and NG-RAN sends the BAT offset value to SMF.

Editor's note:	The need for UL BAT adaptation and the associated RRC signalling as described above is to be confirmed by RAN WG2.

# * * * * End of changes * * * *