3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208648r06

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Google Inc., OPPO, Apple

Title:	Solution#11 Update: 5GC awareness of URSP recognition

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This paper proposes to update solution#11.

# Discussion

This PCR is proposed to update Solution#11 based on SA2#152e revision S2-2206758r05 and clarify the following aspects:

- To assist 5GC awareness of URSP enforcement, it is beneficial for both of the UE and the 5GC to ensure that the URSP rules to be provisioned to the UE are enforceable. With enforceable URSP rules, the mechanisms that allow 5GC to determine whether or when the UE enforces the enforceable URSP rules can be followed efficiently.

In Solution#11, the UCU procedure is enhanced to allow the UE indicating if and which URSP rule is not recognized/supported by UE. The approach is beneficial to assist in URSP enforcement at the UE and allow the network for recognizing the applicable URSP configurations for the UE enforcing the URSP rules without leading to infringement of user privacy.

However, solution#11 would still result in unnecessary power consumption and latency caused by the evaluation of URSP rules in UE policy.

- Obervation1: the UE needs to check all URSP rules whenever it is provisioned with UE policies in "Delivery of UE policies" message in UCU procedure.

- Observation2: the response message of “Result for delivery of UE policies” may be delayed due to the evaluation procedure at the UE, which would potentially cause procedure error at the network expecting the response message for the result.

- Observation 3: The operator may reconfigure URSP based on UE feedback, but may still end up configuring unenforceable URSP rules.

Therefore, this PCR proposes to update solution for resolving abovementioned issues as follows:

- The network indicates to the UE to report its unsupported URSP rules and configure URSP policy by taking the information into consideration.

- To avoid the UE/network being stalled when the UE performs URSP evaluation procedure, the UE provides the unsupported URSP rules in a UL NAS transport message.

# Proposal

It is proposed to update this solution in TR 23.700-85.

### ****** Start of changes ******

## 6.11	Solution #11: 5GC awareness of URSP recognition by UE

### 6.11.1	Description

This solution is proposed to address KI#2 to assist 5GC awareness of URSP enforcement. It is beneficial for both the UE and the 5GC to ensure that the URSP rules provisioned to the UE are enforceable. With enforceable URSP rules, the mechanisms that allow 5GC to determine when the UE enforces the URSP rules can be followed efficiently.

Currently the UCU procedure is used to provision UE policy (including URSP policy) to UE. However, the result in the UCU Response message only indicates whether the UE supports the type of UE policy per Policy Section.

Meanwhile, if UE supports URSP but does not recognize a URSP rule (e.g. due to not recognizinge the TD parameter in a URSP rule), the UE will ignore(skip) the URSP rule without any notification to the network side. This implicit ignoring will result in the inefficiency for the UE to perform URSP rules evaluation and enforcement and for the 5GC to configure enforceable URSP rules at the UE for routing specific application traffic to a specific route, e.g. expected slice as described in an RSD of a URSP rule.

Therefore, it is proposed to enhance the UCU procedure so that the UE can indicate in the UL NAS Transport message if and which TD type(s) of the URSP rule are not recognized by UE.

### 6.11.2	Procedures

Figure 6.11.2-1: Result of the delivery of UE policies indicate if and which URSP rule is supported by UE

The procedure above is extended from the UCU procedure in clause 4.2.4.3 of TS 23.502 [3]:

- 	In step 3, the PCF indicates to the UE via AMF to obtain information from the UE for URSP policy in the UE policy container.

-	When receiving the indication, the UE will evaluate each URSP rule in the UE policy container received in step 3 and identify which component(s) of the URSP rule (i.e. TD  types) are not recognized by UE.

-	The UE indicates corresponding status information of the UE for URSP policy in step 6a.

Based on the feedback from UE, the PCF may update the parameters (e.g. if some parameters in TD are wrongly configured) and provision the updated URSP to UE again. Whether and how to determine the URSP Rules to be sent to the UE based on the feedback received is implementation specific.

NOTE: The PCF may decide to send unrecognized values to the UE after a specific time period, based on local policies.

### 6.11.3	Impacts on services, entities and interfaces

UE:

-	Receive the indication and check the URSP rules when it is provisioned in "Delivery of UE policies" message for reporting unrecognized URSP rules; and

-	Provide feedback about which TD type(s) that cannot be recognized in the URSP rule in NAS Transport message.

PCF:

- Store TD types that are not recognized by the UE, use them for URSP Rule generation.

- Be configured with local policies to enable checking if the UE recognizes TD types.

### ****** End of changes ******