SA WG2 Meeting #SA2-153E 	S2-2208662r02

Oct 10 – 17, 2022, Electronic meeting

Source:	China Mobile, Huawei, HiSilicon, China Telecom?, Interdigital?, Nokia?, OPPO?,Samsung?, vivo?,

Title:	Evaluation and conclusion for KI1

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes Evaluation and conclusion for KI1

# Discussion

# Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

## 7.X	Evaluation for KI#1

Sol#1,#36,#38,#39,#40,#62,#63,#65,#66 are related with KI#1: Policy control enhancements to support multi-modality flows coordinated transmission for single UE.

In Sol#1, AF provides individual deply requirement for each flow and provides delay different threshold for a couple of flows. PCF uses QoS monitoring to calculate the real-time delay difference. PCF may adjust PCC rules for the couple of flows to guarantee the delay difference less than the delay difference threshold. In this solution, the delay and delay different for a couple of multi-modality flows can be calculated, and based on PCC rule adjustment, the delay difference can be kept within the delay difference threshold.

In Sol#36, AF provides a group of flow description, QoS requirement and Handling Together Indication to PCF. PCF generates a group of PCC rules and Handling Together Indication, sends to SMF. PCF/SMF handle the group of PCC rules as a whole e.g. if the QoS requirement of any flow is rejected , PCF rejects all of QoS requirements , or if any PCC rule fails, the SMF notifies the PCF all the PCC rules in the group of PCC rules fail. In this solution, the policies for the group of flows are enforced or removed together.

In Sol#38, AF provides UE ID, Flow description and QoS requirements to 5GS. In UDM, a Coordination Identifier for the tactile and multi-modal communication service is allocated and provided to the AF. AF provides the Coordination Identifier to UE in application layer (out of scope).UE provides Coordination Identifier in PDU session establishment, and SMF and PCF may use the Coordination Identifier to obtain the QoS Policies. In this solution, the intention is to associate the applications in the same UE to be guaranteed by the coordinated/same policy, with providing the same coordination identifier. There is some impact for UE.

In Sol#39, this solution is to reuse URSP methods to configure/update the URSP rule with the Traffic Descriptor including XRM application information (e.g. Domain descriptors, Application descriptors) to associate the application within the same PDU Session. There is no specific impact for PCF, but configuration requirement.

In Sol#40, this solution is mainly for IMS or audio/video services. AF provides the mid-attribute/Flow Identification parameters to the PCF via SDP as per existing signalling, or AF provides the mid-attribute/Flow Identification parameters by using the procedure for AF session setup with required QoS. PCF can assign the same Packet Delay Budget to the QoS Flows transporting the audio and the video streams of the multi-modality traffic. There are limitation for this solution is that only the audio/video services coordination transmission can be supported. The other type of traffic e.g. tactile, control signalling, the coordination transmission is not supported.

In Sol#62, it is suggested to reuse application coordination with re-using existing 5GS mechanisms and some extension for PCF/NEF API or parameters. Option 1 is to extend the existing Nnef_AFsessionWithQoS service to allow the AF to provide service information for multi-modal (XRM) applications. Option 2 is to define a new NEF service/API for multi-modal (XRM) applications.

In Sol#63, it is suggested to re-use the IMS mechanism for the audio and video traffic synchronization. While for other types of multi-modal flows, this solution describes to use the enhanced interaction between AF and 5GS to realize flow coordination, the idea is almost the same as Sol#36.

In Sol#65, AF provides service flow coordination group ID together with group level treatment requirements to PCF via NEF. PCF can provide PCC rules with group level treatment policy for service flows in the group, where the group level treatment policy/requirement may include the following information: 1) Synchronized delivery and joint QoS fulfilment information, including an ordered list of PDB values per service flow in the coordination group. 2) Joint admission control and resource allocation information, indicating associated resource allocation in 5GS among service flows.

SMF derives the QoS profile and AQP(s) of each QoS flow based on ordered list of PDB values. SMF is responsible for providing the information to individual RAN Nodes with the PDU Session modification procedure. RAN performs admission control for each received PDU session modification request following the current procedures assisted by the flow group treatment policy information.

The RAN/SMF/PCF can identify the QoS flows in the same QoS flow coordination group and apply the group level treatment policy, e.g., AQP level alignment to ensure synchronized delivery, joint QoS fulfilment, joint admission control/resource allocation among the QoS flows.

In Sol#66, AF/AS assign an AF specific service flow group ID for XR application clients. For the paired service flows (or AF specific service flow group ID) multi-modality QoS policy information is stored or provisioned in PCF (or UDR) per UE. QoS policy information contains a required threshold of delay difference between the paired service flows. NG-RAN and UPF schedule the packet delivery not to exceed the delay difference between the paired QFIs, which is up to implementation. UE requests PDU session establishment, it includes XR/media dedicated DNN and S-NSSAI to guarantee the single PDU session for the XR service.

Based on the above analysis, there are mainly two types of solutions:

1) Sol#40,#62, #63 propose for the audio/video service(the same type as IMS), some extension for PCF/NEF API or parameters(e.g. mid-attribute/Flow Identification) to support the coordination transmission, since the application layer can guarantee the packet synchronization.

2) Sol#1, 36, 38,63,65,66 propose for the service beyond audio/video, AF provide a group of QoS requirement to PCF as below:

-	Service flow coordination group ID

-	Handling Together Indication

-	Individual 5GS delay requirement

-	A couple of flows delay difference threshold

-	Group level treatment requirements (joint fulfilment requirement)

PCF enforce the coordinated policy for the group of multi-modality flows.

PCF can generate coordinate policy may include the below and send to SMF:

-	Handling Together Indication

-	Synchronized delivery and joint QoS fulfilment information, including an ordered list of PDB values per service flow in the coordination group.

-	Joint admission control and resource allocation information, indicating associated resource allocation in 5GS among service flows.

SMF/NG-RAN/UPF enforces the coordinated transmission and policy control:

-	SMF handle the group of PCC rules as a whole if any PCC rule fails, the SMF notifies the PCF all the PCC rules in the group of PCC rules fail.

-	NG-RAN performs admission control for each received PDU session modification request following the current procedures assisted by the flow group treatment policy information.

-	NG-RAN/UPF may support the delay difference not larger than the delay difference threshold by implementation.

# * * * *second of change * * * *

## 8.X	Conclusions for Key issue#1: Policy control enhancements to support multi-modality flows coordinated transmission for single UE

The following aspects are concluded as principles for the normative work:

-	AF provides group treatment requirements for a group of multi-modality flows to 5GS. The AF provides QoS requirement for multi-modality flows to NEF/PCF before or after the PDU session establishment. The UE identifier could be UE IP address or GPSI. The group treatment requirement may include:

-	The group ID of the multi-modality flows

-	Service/QoS requirement for each flow in the group, optionally includingalternative service/QoS requirements,  and an indicator if the flow is considered critical within the group

-

-	Group level treatment requirements, e.g., an indication if the application layer handles the flows in a synchronized manner, or a joint QoS fulfilment and joint resource allocation requirement for the flows.,

- NOTE: The AF can derive the application layer synchronized handling indicator, e.g., from the existing SDP mid-attributes exchanged between the application endpoints

-	Related to the application layer synchronized handling of the flows:

- The maximum 5GS delay difference threshold for the group of flows

- A trigger to monitor the delay difference for the group of flows

-	The PCF can send the updated URSP including XRM application information to UE through the UCU procedure to ensure that an XRM service uses a single PDU Session per UE.

-	PCF can notify AF/NEF if the monitored delay difference for the group of flows is larger than threshold, to negotiate group treatment requirement with AF.

-	Based on AF indication, PCF can enforce joint policy fulfilment and remove the group policy rules if one or part of the joint policy rules of the group flows is/are rejected.

-	PCF determines a group level QoS treatment policy, maybe with alternative QoS parameters, and send to SMF. SMF may use it for PDU session management decisions or adjustment of the QoS parameters of the flows in the group.

-	NG-RAN may consider the group treatment requirements for admission control, radio resource allocation and QoS fulfilment (e.g. Alternative QoS profile) decisions,

e.g. when NG-RAN performs admission control or QoS fulfilment for one QoS flow, the NG-RAN will also take information of other QoS flows into consideration according the group flow treatment policy.

- In order to ensure that the UE selects the correct DNN/S-NSSAI combination for the XRM traffic, a traffic descriptor for the XRM session will be used during URSP evaluation. In order to allow the DNN/S-NSSAI combination to be used by other UEs for non-XRM traffic and other XRM sessions, the network may provide a Coordination Identifier (e.g. an FQDN) to the AF and the AF may provide the Coordination Identifier to the UE. The Coordination Identifier can then be used as the Traffic Descriptor.

NOTE: 	How the AF provides the Coordination Identifier to UE hosted applications that want to participate in the service is out of SA2's work scope.

# * * * *end of change * * * *