SA WG2 Meeting #SA2-153E 	S2-2208662r09

Oct 10 – 17, 2022, Electronic meeting

Source:	China Mobile, Huawei, HiSilicon, China Telecom?, InterDigital, Nokia?, OPPO?,Samsung?, vivo?,

Title:	Evaluation and conclusion for KI1

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes Evaluation and conclusion for KI1

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

# * * * *second of change * * * *

8.X	Conclusions for Key issue#1 and Key issue#2

The following aspects are concluded as principles for the normative work:

For Key issue#1, single UE case,

-		AF provides group treatment  requirements, group ID information for multi-modality application traffic flows to the 5GS. AF may provide QoS requirements and alternative QoS (service) requirements individually for each service data flow. The AF can provide the information  to NEF/PCF before or after the PDU session establishment. The UE identifier can be UE IP address or GPSI.

Editor’s Note: whether the AF can provide maximum 5GS delay difference threshold to 5GS to guarantee the flows delay difference is FFS.

-	When PCF gets the group treatment requirements, the policies can be generated to support:

-	PCF enforces the group level flow admission and QoS fulfilment policy

-	PCF enforces the group level policy for the use of alternative QoS profiles.

Editor’s Note: 	The details on how the PCF enforces the flow admission, QoS fulfilment and alternative QoS profiles is determined in the normative phase.

Editor’s Note: 	Whether the PCF sends the group policy information to SMF/NG-RAN, and Whether the NG-RAN should support the group treatment policies and how NG-RAN uses them is FFS.

- In order to ensure that the UE selects the correct DNN/S-NSSAI combination for the XRM traffic, the existing URSP Rule evaluation framework can be reused. A traffic descriptor (e.g. an FQDN) for the XRM session will be used during URSP evaluation.

For Key issue#2, multiple UE case,

-	The AF procedure is extended with QoS requirements for individual service flows and group treatment requirements according to the conclusions for KI#1.In case multiple UEs are involved, AF provides the information related to all UEs to NEF either using a single procedure or using multiple UE specific procedures.

-	In either case, the AF may indicate that specific service data flows belong to the same group by associating them with a common flow group ID.

-	PCF uses the individual service data flow QoS requirements and group treatment requirements and policies in the same way as concluded for Key Issue #1.

Editor’s Note:		Whether multiple PCFs may be involved in the procedures for multiple UEs and how the coordination across PCFs works in that case is FFS.

- -	Multiple UEs select the correct DNN/S-NSSAI combination for the XRM traffic is the same as for Key Issue#1.

# * * * *end of change * * * *