SA WG2 Meeting #SA2-153E 	S2-2208662r17

Oct 10 – 17, 2022, Electronic meeting

Source:	China Mobile?, Huawei?, HiSilicon?, China Telecom?, InterDigital?, Nokia?, OPPO?,Samsung?, vivo?, Ericsson

Title:	Evaluation and conclusion for KI1

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes Evaluation and conclusion for KI1

# Discussion

# Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

# * * * *second of change * * * *

## 8.X	Conclusions for Key issue#1 and Key issue#2

The following aspects are concluded as principles for the normative work:

For Key issue#1, single UE case,

The following aspects are concluded as principles for the normative work:

- Media synchronization of flows associated to multi-modality traffic is achieved by reusing application layer mechanisms (e.g. as specified for IMS, comprising SIP/SDP together with RTP/RTCP). The requirement is to have a common wall-clock time for all data flows and UEs that need to be in sync.

- Those data streams that are closely related and require strong application coordination are transmitted in a single PDU session by a single UE. However, those data streams that contribute to the immersive experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs

- The application server (AF) shall retain control over all UEs/PDU sessions and data flows that comprise a multi-modal application. Hence, the application may require a different set of QoS parameters for each data stream, enable/disable each device independently, and resolve error conditions according to application layer logic.

- XRM applications provide service requirements (including flow description, QoS info, etc.) when the XRM session is initiated and server IP address/port is allocated. This allows the XRM application to request the necessary resources on demand, according to the characteristics of the XRM session requested from the UE.

- The procedure for AF session setup with required QoS, is reused for XRM applications (untrusted AFs) interacting with NEF. However, current Nnef_AFsessionWithQoS service shall be extended to allow the AF to provide information for multiple medias.

  - Normative impact to AF and NEF: extend the existing Nnef_AFsessionWithQoS service to allow the AF to provide, at the same time, service requirements for multiple IP data flows associated to a multi-modal (XRM) application

Additional impacts are FFS.

# * * * *end of change * * * *