SA WG2 Meeting #153E (e-meeting)	S2-2208673r010

October 10 – 17, 2022, Elbonia	(revision of S2-xxxxxxx)

Source:	Tencent, Tencent Cloud, Xiaomi, China Mobile, Huawei, Samsung?, ZTE?

Title:	KI#3, conclusion update

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM /Rel-18

Abstract of the contribution:

For KI#3 "5GS information exposure for XR/media Enhancements", the conclusion part is updated to clarify which entity may be impacted for exposing the information. Two more parameters provided by AF are also added.

Regards to the Delay difference and round trip delay, as in sol#42 and #47, they are measured by UPF and RAN. The RAN part of UL/DL packet delay measurement is as defined in clause 5.33.3 of TS 23.501.There is no additional RAN impact compared with the requirements defined in clause 5.33.3 of TS 23.501. The information can be exposed as the exposure path defined in clause 6.4 of TS 23.548 [61] or via SMF/PCF/NEF.

Regards to data rate, it may be measured and reported by the UPF as in sol#5. It could also be measured and reported by RAN via SMF/PCF/NEF as in sol#48.

Furthermore, as in sol#6, the AF may provide the Alternative QoS parameter set requirements to help the network to decide the related QoS parameters; as in sol#44, the AF may also provide the Averaging window in order to help the network to decide the AW for quick QoS notification to the AF. These two parameters are considered helpful for the XR service.

Based on rapporteur’s suggestion, S2-2208673r01 merges the revisions from S2-2208567, S2-2208658, S2-2208663, S2-2208823 and S2-2208999.

1. Proposal

For KI#3 "5GS information exposure for XR/media Enhancements", this contribution proposes to update the conclusion to clarify which entity may be impacted for exposing the information. Two more parameters provided by AF are also added.

*********** First Change***********

7.x	Evaluation of Solutions for Key Issue #3

Key Issue #3 is for 5GS information exposure for XR/media Enhancements. Currently, there are 11 solutions to solve this KI. All the solutions use the same subscribing method. The summary of the solutions are listed in Table 7.x-1.

Table 7.x-1 Candidate Solutions

| Solution | Exposed information | Exposure path |
| --- | --- | --- |
| #5: Exposure of the UE data rate, normal data transmission interruption event and congestion information | Normal Data Transmission interruption event | via RAN notification: RAN->SMF->PCF->AF; or via GTP-U from RAN to UPF: RAN->UPF(API)->AF |
|  | radio congestion information |  |
|  | data rate | measured and exposure by UPF: UPF(API)->AF |
| #6: Mean bit rate change report | mean bitrate for non-GBR QoS flow | via RAN notification: RAN->SMF->PCF->AF |
|  | requested Alternative QoS Parameter Sets | from AF to NEF. |
| #41: Use of ECN bits for L4S to enable codec/rate adaptation to meet requirements for services | current load level information | ECN bits marking in RAN for L4S |
| #42: Exposure of round-trip delay to support XR/media enhancements | round-trip delay | QoS monitoring via UPF. |
| #43: Information Exposure to AF for XR/media Enhancements | radio congestion information | Uplink: RAN(GTP-U)->UPF(API)->AF Downlink: RAN(GTP-U)->UPF(ECN)->UE |
| #44: XRM AF driven Quick QoS Notification Control | requested AW | AF->NEF |
| #45: Information Exposure to AF for XR/media Enhancements | estimated QoS | OAM->NWDAF->NEF |
| #46: Use of ECN marking for L4S for scalable congestion control and meet requirements for services | radio congestion information | Uplink: UE or RAN or UPF supports ECN marking; Downlink: RAN or UE supports ECN marking. ECN marking in different layers in RAN may be supported. |
| #47: Delay Difference and Delay Notifications and Reports | delay difference | QoS monitoring via UPF |
|  | two way delay |  |
| #48: Enhanced Notification of Rata Adaptation Request | rate adaptation suggestion | via RAN notification |
| #67: Supporting ECN mark for L4S in 5GS for XR/media Enhancements | radio congestion information | ECN marking in RAN or UPF |

In Table 7.x-1, the information exposed to AF could help the XR/media application server to enable application codec/rate adaptation.

- Non-ECN related solutions (Solutions #5, #6, #42, #44, #45, #47,and #48)

Solution#5 proposes to expose Normal Data Transmission interruption event, congestion information and data rate to the application. Regards to Normal Data Transmission interruption event and congestion information, these two information are exposed by RAN through RAN notification or GTP-U. Regards to data rate, it is measured and exposed by PSA UPF.

Solution#6 proposes to expose mean bitrate for non-GBR QoS flow through QoS notification. This solution also proposes that the AF could request Alternative QoS Parameter Sets to enable the network to provide alternative QoS parameters as references for the currently supported QoS level for the XR traffic.

Solution #42 proposes that the round trip delay can be measured by the UPF and RAN. The RAN part of UL/DL packet delay measurement is as defined in clause 5.33.3 of TS 23.501. There is no additional requirements on RAN compared with the requirements defined in clause 5.33.3 of TS 23.501. Two cases are considered. One is that the XR traffic is transmitted within a single QoS flow; The other one is that the XR traffic is transmitted within multiple QoS flows, e.g. the main UL and DL of XR traffic are separated into two flows. The UPF exposes the round-trip delay to the AF directly or via NEF.

Solution #44 proposes that the AF could request the average window (AW) to the NEF. The AW can determine the measurement periodicity (i.e. the rate) of the GFBR/MFBR.

Solution#45 proposes to support estimated QoS exposure to the AF. It reuses the procedure in Clause 6.2.3.2 of TS 23.288 for the NWDAF to get the information from OAM. Then the NWDAF predicts the average estimated QoS based on the information.

Solution #47 proposes that delay difference and round trip delay is measured by UPF and RAN. RAN part of UL/DL packet delay measurement is as defined in clause 5.33.3 of TS 23.501. The information can be reported and exposed via SMF/PCF/NEF.

Solution #48 proposes that the RAN may provide rate adaptation suggestion to the AF. The RAN could expose the information through RAN notification.

- ECN related solutions (Solutions #41, #43, #46 and #67)

Solution #41 and #46 exposes the radio congestion information to the application server through ECN bits marking for the purpose of L4S in RAN; while Solution #43 exposes the radio congestion information to the UE through ECN bits marking for L4S in UPF. Solution #67 could be considered as the complementary solution for ECN marking for L4S. The L4S marking is according to [37] and [62].

Solution #41 requires the RAN to support ECN bits marking for L4S of payload packets for the uplink and downlink. A QoS flow used for L4S enabled traffic is established. The criteria for the RAN to perform marking is up to RAN implementation.

Solution #46 provides different solution options for supporting ECN marking of payload packets for both uplink and downlink. For the uplink, either the RAN or the UE needs to support the ECN marking. The ECN marking could be set in different layers. If the ECN marking is set in PDU IP header, there will be no impact to the UPF. Other options require the UPF to transfer the ECN marks to the PDU IP header. For the downlink, the RAN needs to support ECN marking. If the ECN marking is set in PDU IP header, there will be no impact to the UE. Other options require the UE to support the ECN marking. Regards to this solution, whether RAN can support the ECN marking and which layer the marking should be set need to be coordinated with RAN WG.

Solution #43 requires the RAN to send the congestion information to the UPF, and then the UPF could send the congestion information to the AF for the uplink, and send the congestion information to the UE via ECN marking in the layer 4 protocol for the downlink.

Solution #67 proposes that the AF could provide ECN marking for L4S instruction information. Based on the AF information, the PCF generates the corresponding policy, and sends it to the SMF, RAN or UPF to enable the ECN marking.

*********** Second Change***********

8.1	Conclusions for Key issue#3: 5GS information exposure for XR/media Enhancements

The following bullet points summarize the principles for the way forward to support current congestion level information exposure:

-	5G System may use ECN marking for the purpose of Low Latency, Low Loss and Scalable Throughput services L4S according to [37] and [62] for uplink or and downlink QoS Flows via one of the following two methods.

-	To support L4S, NG-RAN performs ECN marking according to [37] and [62] for uplink and downlink in IP layer of the received packets.

NOTE 1:	The criteria for RAN to determine (e.g. its congestion level) when to perform the marking is up to RAN implementation.

-	PSA UPF performs ECN marking according to [37] and [62] for uplink and downlink QoS Flows based on current congestion level information reported from NG-RAN via GTP-U header

NOTE 2:	If the network operator want to apply the ECN marking for L4S, it shall guarantee that any sender (UE or Server) requesting classic ECN congestion control will not tag its packets with the ECT(1) in order to avoid conflicted usage of ECT(1) in L4S. Otherwise, L4S is not supported in network.

NOTE 3: Supports for L4S and for exposure of congestion level (e.g. per QoS flow congestion level) is pending RAN WG's feedback on the feasibility of RAN judgment and/or exposure of the corresponding info.

-	5G System also may support API based exposure of congestion level information towards AF as following:

-	The following information may be exposed by RAN:

-	QNC for GBR QoS Flow: data rate cannot be guaranteed;

-	AF uses Nnef_AFSessionWithQoS to subscribe the above exposure to NEF/PCF, same as local exposure mechanism defined in TS 23.548 [61].

-	Exposure path of Network Exposure defined in clause 6.4 of TS 23.548 [61] is reused with extensions of GTP-U header and UPF/L-NEF services to exposure the above information.

-	Exposure path of RAN/UPF reporting congestion level information via SMF/PCF/NEF is also supported.

-

The following bullet points summarize the principles for the way forward to support exposure for other network information:

-	Data rate, delay difference and round trip delay of QoS Flow may be exposed to AF.

-	Data rate may be measured and exposed by UPF. Exposure path defined in clause 6.4 of TS 23.548 [61] is reused to expose the above information. Exposure path of UPF reporting via SMF/PCF/NEF is also supported.

-  Delay difference based on QoS monitoring can be exposed from PCF to AF.

-  Round trip delay for multiple QoS flows of the XR service (e.g. the UL and DL are separated into two flows) can be obtained and exposed by the UPF via the exposure path defined in clause 6.4 of TS 23.548 [61] or via SMF/PCF/NEF.

Editor's note:	It is FFS whether to expose the Normal data transmission interruption event to AF.

-	Estimated bandwidth for 5QI may be exposed by NWDAF (according to information described in clause 6.9.2 in TS 23.288[61]) to AF.

* * * * End of Changes * * * *