3GPP SA WG2 Meeting #153E  			   	   S2-2208676

Elbonia, October 10-17, 2022

Source:	Samsung

Title:	KI#7: Proposal for Conclusion for KI#7

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys

Abstract of the contribution: This contribution provides a conclusion proposal to KI#7.

# 1.	Discussion

This contribution provides a conclusion proposal to KI#7.

# 2 Proposal

It is proposed to include the below changes to TR 23.700-80.

FIRST CHANGE

## 8.7	Key Issue #7: 5GS Assistance to Federated Learning Operation

Based on the evaluation in cl. 7.7, the following architecture principles and specific mechanisms below proposed by KI#7 solutions are considered for the normative work on 5GS assistance to FL operation.

-

-  Specific mechanisms proposed for 5GC assistance to application FL operation are as follows.

-  New QoS mechanisms for groups of UEs are required to assist FL operation aligned with conclusions for GMEC Key Issue #3: NEF exposure framework for provisioning of traffic characteristics and monitoring of performance characteristics applies, as this is resolved there.

END OF CHANGES