3GPP SA WG2 Meeting #153E  			   	 S2-2208676r05

Elbonia, October 10-17, 2022

Source:	Samsung, Huawei, HiSilicon

Title:	KI#7: Proposal for Conclusion for KI#7

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys

Abstract of the contribution: This contribution provides a conclusion proposal to KI#7.

# 1.	Discussion

This contribution provides a conclusion proposal to KI#7.

# 2 Proposal

It is proposed to include the below changes to TR 23.700-80.

FIRST CHANGE

## 8.7	Key Issue #7: 5GS Assistance to Federated Learning Operation

### 8.7.X	5GS assisted FL performance improvement

For the 5GS assisted FL performance improvement, the following principles are proposed:

-  To improve the efficiency of FL model training or the accuracy or generalization ability of trained FL model, the FL group can be updated by AF based on the 5GC information. For example, according to the results of QoS monitoring, the AF can remove UEs with large transmission delay from FL group, and select UEs with small transmission delay to join FL group.

-  Support the AF to determine the central server for one or multiple rounds of FL to reduce the latency divergence by enhancing the DN Performance Analytics to provide analytics of latency divergence between UEs and application servers.

-  Support the AF to adjust the transmission delay of UEs in the FL group to reduce the latency divergence by enhancing the DN Performance Predictions of NWDAF to provide the predicted UE transmission delay.

### 8.7.Y	Miscellaneous conclusions of Key Issue #7

Based on the evaluation in cl. 7.7, the following architecture principles and specific mechanisms below proposed by KI#7 solutions are considered for the normative work on 5GS assistance to FL operation.

-  Key principles proposed for 5GC assistance to application FL operation are as follows.

-  Specific mechanisms proposed for 5GC assistance to application FL operation are as follows.

-  QoS mechanisms for groups of UEs are required to assist FL operation as described in clause 8.6.

END OF CHANGES