SA WG2 Meeting #153e	S2-2208703r03

# October 10th – 17th, 2022; Elbonia

# Source:	China Mobile, Tencent, Tencent Cloud, Huawei, HiSilicon

# Title:	New WID: Architecture Enhancements for XR and media services

# Document for:	Approval

# Agenda Item:	10.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Architecture Enhancements for XR (Extended Reality) and media service

Acronym: XRM

Unique identifier:

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940068 | Study on XR (Extended Reality) and media services | Study item phase of this work item proposal, comprehensive study the key issues and make conclusions |
| 900027 | Study on supporting tactile and multi-modality communication services (Release 18) | Study of requirement about tactile and multi-modality communication services |
| 890009 | Operation Points for 8K VR 360 Video over 5G | SA4 study to specify operation points as well as new media decoding capabilities to enable support for up to 8K video. |
| 870013 | Feasibility Study on Typical Traffic Characteristics for XR Services and other Media | SA4 study to collect traffic, codecs and protocol characteristics to identify and document additional relevant XR and other media services and related requirements. |
| 840019 | 5G System Enhancement for Advanced Interactive Services, | SA2 work item to enhance QoS mechanism e.g. define new standardized 5QI for cloud gaming and XR services |
| 800014 | Study on Audio-Visual Service Production (Release 17) | Multiple devices collecting data for the same task with strict KPI requirements. |
| 930020 | Supporting tactile and multi-modality communication services | SA1 requirement about tactile and multi-modality communication services |

3	Justification

In 5G era, mobile media services, cloud AR/VR, cloud gaming, video-based tele-control for machines or drones, are expected to contribute more and more traffics to 5G network. All media traffics, in spite of which codec was used, have some common characteristics. These characteristics can be very useful for better transmission control and efficiency. However, currently 5GS uses common QoS mechanisms to handle media services together with other data services without taking full advantage of these information. For examples:

-	Packets within a frame have dependency with each other since the application needs all of these packets for decoding the frame. Hence one packet loss will make other correlative packets useless even they are successfully transmitted. For example, XR applications impose requirements in terms of Media Units (Application Data Units), rather than in terms of single packets/PDUs.

-	Packets of same video stream but different frame types (I/P frame) or even different positions in the GoP (Group of Picture) are of different contributions to user experience, so a layered QoS handling within the video stream can potentially relax the requirement thus lead to higher efficiency.

Additionally, the XR/media traffics have the characteristics of high throughput, low latency, and high reliability requirement, and the UE battery level may impact the user’s experience since the high throughput require the high power consumption in terminal side. So considering the limited radio resource and end-to-end QoS policy control from system perspective, the 5GS should be enhanced to support trade-off among throughput, latency and reliability and device battery life.

Furthermore, considering the XR/media traffics have natural interval between periodic video/audio frames, it would be possible to enhance power saving mechanisms (e.g. CDRX) considering the XR/media traffic pattern.

Some advanced XR or media services may include more modalities besides video and audio stream, such as information from different sensors and tactile or emotion data for more immersing experience e.g. haptic data or sensor data. To support such tactile and multi-modality communication services (identified by SA WG1 TACMM), the 5G system may need to address service requirement of different types of traffic steams with coordinated QoS selection and packet processing, guaranteed latency and reliability, time synchronization of these parallel information, in order to ensure best service experience.

This work item is to specify the enhancements that have been concluded in TR 23.700-60 as a result of the FS_XRM  study.

4	Objective

This work item will implement the conclusions of the FS_XRM Study Item in clause 8 of TR 23.700-60:

- KI#1: Policy control enhancements to support multi-modality flows coordinated transmission for single UEs;

NOTE: Aspects about group level policy handling, SMF/RAN impact, delay difference, are not included in the above objective for KI#1, and those aspects will be further considered.

- KI#2: Application Synchronization and QoS Policy Coordination for Multi-modal Traffic among Multiple UEs;

NOTE: Aspects about multiple PCFs are not included in the above objective for KI#2, and this aspect will be further considered.

- KI#3: 5GS information exposure for XR/media enhancements;

NOTE: Aspects about supports for L4S and for exposure of congestion level (e.g. per QoS flow congestion level) is pending RAN WG's feedback for KI#3.

- KI#4: PDU Set integrated packet handling;

- KI#5: Differentiated PDU Set Handling;

NOTE: Aspects about the N6 protocol, PDU set importance, etc, will be further considered for KI#4&KI#5.

- KI#6: Uplink-downlink transmission coordination is performed to meet Round-Trip latency requirements;

- KI #7: Policy enhancements for jitter minimization;

- KI #8: Enhancements to power savings for XR services;

NOTE: Aspects about periodicity will be further considered for KI#8.

KI

- KI#9: Trade-off of QoE and Power Saving Requirements ;

NOTE: The conclusion for KI#9 is under discussion, will be further updated.

NOTE 1: Exact conclusion contents of normative work for each Key Issue shall be based on conclusion in clause 8 of TR 23.700-60.

NOTE 2 : Whether to define a network slice type supporting media services can be determined during normative phase.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Updates to 5G System architecture to support XRM | SA#99 March 2023 |  |
| 23.502 | Updates to 5G System procedures to support XRM | SA#99 March 2023 |  |
| 23.503 | Updates to 5G System Policy Control to support XRM | SA#99 March 2023 |  |

6	Work item Rapporteur(s)

Dan WANG, China Mobile	wangdanyjy@chinamobile.com,   responsible for objective #1, #2, #4, #5, #9

Yixue LEI, Tencent		yixuelei@tencent.com , responsible for objective#3, #6, #7, #8

7	Work item leadership

SA2

8	Aspects that involve other WGs

SA3 for security aspects

SA4 for media types of emerging and XR-based services and traffic characteristics aspects.

RAN1/2/3 for RAN part enhancements.

SA5 for charging aspects

9	Supporting Individual Members

Supporting IM name

China Mobile

Tencent, Tencent Cloud

CATT

China Telecom

China Unicom

CAICT

Huawei

HiSilicon

Spreadtrum Communications

vivo

ZTE

Qualcomm

Xiaomi

OPPO

Intel

Oracle

Broadcom

CBN

ABS

Toyota

Interdigital

Alibaba

Futurewei

Nokia

Nokia Shanghai Bell

Apple

Telecom Italia

NTT DOCOMO

KDDI

SK Telecom

Verizon UK Ltd

Lenovo

Motorola Mobility

KPN

Bosch

Ericsson

TCL

AT&T

Samsun

Deutsche Telekom

MediaTek Inc.

LG Electronics

Allot

Philips International B.V.

Vodafone

DISH Network

Charter Communications

Google

MATRIXX Software

Telefonica

BT

Microsoft

Rakuten

Meta USA

Sony

Futurewei

CableLabs