3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208726r02

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5: Update to Evaluation on MBS session

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: The evaluation and conclusion are provided for KI#5 in this document.

# 1. Discussion

KI#5 is for providing power saving mechanism for capability-limited devices.

In a nutshell, all of the 3 solutions following the similar logic – i.e., providing UE the scheduled time. And those solutions are in fact not mutually exclusive.

-	Solution #14, #15 are aiming at providing the time information via service announcement message, and Sol#14 further specifies the prioritization of MBS data reception and current power saving mechanisms.

-	On the other hand, Sol#25 is for the case when the "scheduled time" is unknown to AF when providing the service announcement. This option is for multicast only.

Current evaluation description for Sol#25 can be updated to improve the readability, since it now uses NAS (option 2) or service announcement (option 1) instead of using "periodic or one time transmission" or "Deferred activation".

For solution #25:

-	The idea of "periodic or one time transmission" in clause 6.25.3.1 is to make the wake-up time as a part of MBS session context, and provide the wake-up time to the UE via NAS message at UE join and later 5GC triggers the session activation based on the wake-up time.

Note that one simple (but may not be effective) method is to send the service announcement again to all of the involved UEs. That is truly not signalling efficient. Another way around is to defer the service announcement, i.e., after the AF determining the wake-up time. That is also a very strong assumption towards the AF and may not fly for some use cases (e.g., for the cases like UE needs to know TMGI very early).

In fact Rel-17 already support this feature to some extent. Quote from TS 23.247 clause 7.2.1.3:

4. The SMF determines whether the user is authorized to join the Multicast MBS session taking into account the MBS subscription data received from the UDM and the Any UE indication if received from the MB-SMF. The SMF considers the UE as authorized to the Multicast MBS session if the UE is authorized to use multicast MBS services, and if the MBS Session ID(s) in the PDU Session Modification Request is included in the MBS subscription data or Any UE indication is received. If authorization check fails, the SMF rejects the join request with a cause value. If a UE joins prior to the start time of the multicast MBS session, the SMF may accept the join request and indicate to the UE the start time, or it may reject the join request with an appropriate error cause and optionally a back-off timer. If a UE joins while the multicast MBS session is inactive, the SMF accepts the join request.

It can be seen from the bold italic sentence that the start time is very like the wake-up time here. In addition, MB-SMF may need to support one extra trigger for Session Activation (besides MB-UPF notification and AF request).

Compare Sol#25 "periodic or one time transmission” with Sol#14 that the scheduled MBS delivery time is included in service announcement, Sol#25 has impact on AF, MB-SMF, SMF and UE, and the benefit is not clear.

-	The idea of "Deferred activation" in clause 6.25.3.2 (see also below) is another alternative to provide the time information, i.e., AF provides the wake-up time to 5GC whenever needed, and such information will be provided to UE.

6.25.3.2	Deferred activation for aperiodic transmission of MBS data to capability-limited devices

Figure 6.25.3.2-1: Deferred activation for aperiodic transmission of MBS data to capability-limited devices

If an MBS session is used to transmit data at irregular intervals, the devices need to be informed about the data transmission while the MBS session is ongoing. The AF sends a delayed activation request indicating the desired activation time for an MBS session to the MB-SMF, and the MB-SMF request the SMFs to perform a delayed activation. The SMFs in turn request the AMFs to perform deferred paging. If UEs negotiated an eDRX cycle and joined the multicast session, the AMF pages the UE in the Paging Hyperframes (PH) / time intervals calculated according to eDRX procedures before the transmission start and indicates the transmission start time.

#1 The use case of aperiodic transmission is not clear. In our view, for capability-limited devices, MBS could be used for software downloading, which can be scheduled that is covered in Option-1 of Sol#25 or Sol#14.

#2 From system impact perspective, AMF is not supposed to be involved in the UE’s handling of MBS Session whenever possible (due to the SMF-centric multicast solution chosen in Rel-17), the new procedure of NAS Transfer in step ④ is not aligned with that principle.

#3 In text box ⑤, it is not clear how IDLE UE can start reception of MBS data.

For Option-2 of Sol#25, below are some

[Proposal-1]: it is proposed to update the description for solution #25 in clause 7.5.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

## 7.5	Key Issue #5: Coexistence with existing power saving mechanisms for capability-limited devices

For Key Issue #5, there are following solutions:

-	Sol#14 is intended for both multicast and broadcast MBS Session.

-	Sol#15 covers only broadcast MBS Session.

-	Sol#25 is intended for multicast MBS Session.

For interaction between broadcast MBS and power saving mechanism, Sol#14 and Sol#15 are aligned.

The table below is to provide an overview of the solutions of KI#5.

Table 7.5-1: Comparison of multicast solutions for KI#5

|  | Solution #14 | Solution #15 | Solution #25 (periodic or one time transmission) | Solution #25 (Deferred activation) |
| --- | --- | --- | --- | --- |
| Inform UE the scheduled time | service announcement | Service Announcement. | Service Announcement or NAS message (i.e. PDU session modification command). | NAS message (built by AMF) |
| Method to control the "awake" timing | DRX level - counted in milliseconds. | SDP level (NTP time values) - counted in seconds. | SDP level (NTP time values) - counted in seconds. | SDP level (NTP time values) - counted in seconds. |
| 5GC enhancement | N/A. | N/A | MB-SMF: activate the MBS session at wake-up time SMF: provides NAS message including wake-up time. | MB-SMF: provides wake-up time to SMF(s) in delayed activation request. SMF: provides wake-up time in delayed activation request with UE list. AMF: finds the UE in power saving mode and paging them individually, and provide the wake-up time in NAS message. |
| UE behaviour | DRX for MBS takes precedence over the existing Power saving mechanism. E.g. MICO, PSM, or eDRX. | Provided time takes precedence over the existing Power saving mechanism. | Provided time takes precedence over the existing Power saving mechanism. | Provided time takes precedence over the existing Power saving mechanism. |
| Use cases | MBS service Suitable data transmission at times known in advance, e.g. for periodically repeated data transmissions | MBS service Suitable data transmission at times known in advance, e.g. for periodically repeated data transmissions | Multicast MBS service. Suitable for data transmission at not previously known times | Multicast MBS service. Suitable for data transmission at not previously known times |

NOTE:	Note that the above-mentioned table may not exhaustively list the proposal of the solutions. For example, Solution#14 also suggest to keep UE connected for a while in the middle of an MBS data transfer.

In fact the main idea of DRX level control and SDP level control are not against each other, they are, on the other hand, the complementary solutions and therefore can be adopted at the same time. In addition, s Solution #25, also propose to make use of service announcement to convey the knowledge of session start/end time of the MBS session. Solution #25 also propose to provide such info via NAS message therefore are fit for multicast MBS service, and it

enables data transmission at times which are not known when the service announcement is performed.

For solution #25:

-	The idea of "periodic or one time transmission" is to make the wake-up time as a part of MBS session context, and provide the wake-up time to the UE via PDU Session Modification and 5GC triggers the session activation based on the wake-up time.

This solution has impact on AF, MB-SMF, SMF and UE, and the benefit is not clear compared to the service announcement approach or Sol#14.

-	The idea of "Deferred activation" is another alternative to provide the time information, i.e., AF provides the wake-up time to 5GC whenever needed, and such information will be provided to UE via individual paging. The proposal uses session activation and provides the MBS information to UE by the AMF. It is not clear how the IDLE UEs can start receiving MBS data in Figure 6.25.3.2-1.

The use case of aperiodic transmission is not clear. For capability-limited devices, MBS could be used for software downloading, which can be scheduled which is covered in Sol#14.

From system impact perspective, the AMF is not supposed to be involved in the UE’s handling of MBS Session whenever possible (due to the SMF-centric multicast solution chosen in Rel-17), however the new procedure of NAS Transfer in Figure 6.25.3.2-1 is not aligned with that principle.

# * * * * End of changes * * * *